| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0322_r1 |
| --- | --- |

| Title: | CE4: Triangle prediction merge list construction (CE4-4.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma | Email: | xianglinwang@kwai.com yiwenchen @kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document are the results of CE4-4.3 on triangle prediction merge list construction. In this method, regular merge candidate list is re-used for triangle partition merge prediction with no extra motion vector pruning. For a given picture, if backward prediction is used, L0 and L1 motion vectors of each merge candidate in the regular candidate list are selected in a L0-L1 interleaving manner for triangle merge prediction. If backward prediction is not used, L0 motion vectors from the regular candidate list are first selected for triangle merge prediction, followed by zero motion vectors or L1 motion vectors from the regular merge list. Simulation results reported show that the proposed solutions incur no coding performance loss for random access configuration, and in average 0.16~0.17% coding loss for low delay B configuration relative to VTM-4.0.

# Introduction

In VTM-4.0[1], triangle prediction can be used to enable triangular shape prediction units for inter prediction. It is currently supported under merge mode, with its own merge candidate list construction process that is different from the regular merge candidate list construction.

Figure 1. Motion vectors used for triangle merge list construction

Figure 1 shows the block locations where motion vectors are derived for triangle merge list construction. Block 1 to 5 are spatial neighboring block and block 6 and 7 are collocated temporal block. These up-to seven motion vectors are first put into a neighboring motion vector list. To form a uni-prediction merge list for triangle prediction, this neighboring motion vector list is scanned, and uni-prediction motion vectors are first selected into the merge list. If the merge list is not full, L0 motion vectors of the bi-prediction motion vectors in the neighboring motion vector list are added into the merge list. Then L1 motion vectors of those bi-prediction motion vectors are added. After that, the average motion vector of L0 and L1 motion vectors are added. At last, uni-prediction zero motion vectors may be added to make the merge list full.

Overall, the merge list construction process is quite different from the regular merge list construction.

# Proposed method for CE tests

According to this proposal, to maximize the common operations shared by different merge modes, the regular merge list is constructed and its motion vectors are used for triangle prediction.

Once the regular merge list is constructed, for a given picture where backward prediction is used, L0 and L1 motion vectors of each merge candidate in the regular candidate list are selected in a L0-L1 interleaving manner for triangle merge prediction. If backward prediction is not used, L0 motion vectors from the regular candidate list are first selected for triangle merge prediction, followed by zero motion vectors (which is corresponding the test “a” in this CE test) or L1 motion vectors from the regular merge list (which is corresponding to test “b” in this CE test).

# Results

The proposed modification is integrated into VTM-4.0 and the results of the proposed implementation under CTC [2] are shown below against VTM4.0.

Table 1. Results of CE4-4.3.a test under Random Access

Table 2. Results of CE4-4.3.a under Low delay B

Table 3. Results of CE4-4.3.b under Random Access

Table 4. Results of CE4-4.3.b under Low delay B

It can be seen that the test results show no coding performance loss for random access configuration. For low delay B configuration, test results do show 0.16~0.17% average coding performance loss. This is due to the fact that in the proposed solutions, there is no motion vector pruning involved in selecting the uni-prediction merge candidate for triangle prediction, while in the VTM-4.0 anchor implementation a full motion vector pruning process is performed.

# Complexity analysis

Complexity analysis is performed according to the description of Core Experiment 4[4]. Corresponding results for tests 4.4.3.a and 4.4.3.b are presented in Table 5 and Table 6 respectively.

Table 5. Summary of complexity analysis for Test 4.4.3.a

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 0 | 0 | 1 | 0 | 5 | – |

Table 6. Summary of complexity analysis for Test 4.4.3.b

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 0 | 0 | 1 | 0 | 5 | – |

# Conclusion

In this proposal, a different merge list construction process was proposed for triangle prediction. The proposed schemes reuse the regular merge list with no extra motion vector pruning, followed by a simple process to derive the uni-prediction merge candidate for triangle prediction. The reported results show no performance impact with the proposed simplifications for random access configuration, and in average 0.16~0.17% coding performance loss for low delay B configuration. It is recommended to consider adoption of one of the proposed schemes into VVC for triangle prediction.

# Reference

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

- “ Non-CE10: Triangle prediction merge list construction”, X. Wang, Y.-W. Chen, doc. JVET-M0233, January 2019.

- H. Yang, G. Li, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-M1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).