| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0323 |
| --- | --- |

| Title: | CE2: Reducing worst-case memory bandwidth of affine mode (CE2-4.2.a, CE2-4.2.b, CE2-4.2.c and CE2-4.2.d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang Huanbang Chen Timofey Solovyev Haitao Yang Jianle Chen | Email: | yiwenchen@kwai.com chenhuanbang@huawei.com |
| Source: | Kwai Inc. Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

Reported in this document is the results of CE2-4.2 on reducing the worst-case memory bandwidth of affine motion mode. In the proposed scheme, affine sub-block size is 4x4 for a uni-directional affine CU and 8x4/4x8 or 8x8 for a bi-directional affine CU. Four subtests are conducted for CE2-4.2. For uni-prediction CU, 4x4 sub-block size is used for all subtests. For bi-prediction CU, four different configurations are tested:

- 8x4 (CE2-4.2.a)

- 4x8 (CE2-4.2.b)

- 8x4/4x8 adaptively switched depending on aspect ratio of coded block (CE2-4.2.c)

- 8x8 (CE2-4.2.d).

It is asserted that the test CE2-4.2.a – CE2-4.2.c reduce the worst-case memory access bandwidth by about 32% while the test CE2-4.2.d reduces the worst-case memory access bandwidth by 50% for the current VVC. The simulation results reportedly show that CE2-4.2.a introduces luma losses of 0.10% and 0.11% respectively under RA and LDB cases. CE2-4.2.b introduces luma losses of 0.11% and 0.12% respectively under RA and LDB cases. CE2-4.2.c introduces luma losses of 0.10% and 0.11% respectively under RA and LDB cases. CE2-4.2.d introduces luma losses of 0.21% and 0.18% respectively under RA and LDB cases.

# Introduction

Motion compensation (MC) is usually the largest consumer of memory bandwidth in decoder implementation. Therefore, for a video coding standard, providing a reasonable upper limit on MC bandwidth requirement is extremely important in facilitating its cost-effective implementations and ensure its success in industry.

Memory access bandwidth requirement for MC is usually determined by the operating block size and the type of prediction (e.g. uni-directional or bi-directional). In VVC, the worst case of MC memory bandwidth occurs with the bi-directional MC of a 4x4 block, which is utilized by the CUs coded as bi-directional affine mode (where MC sub-block size is always 4x4).

# Proposed Implementation

According to proposals [1][2], 4x4 blocks are restricted from using bi-directional motion compensation for affine motion mode, which results in 32% reduction of the worst-case bandwidth requirement in the current VVC. Four different schemes as described below are tested in CE2-4.2.

CE2-4.2.a) 4x4 sub-block is used for a uni-directional affine coded CU; 8x4 sub-block is used for a bi-directional affine coded CU.

CE2-4.2.b) 4x4 sub-block is used for a uni-directional affine coded CU; 4x8 sub-block is used for a bi-directional affine coded CU.

CE2-4.2.c) 4x4 sub-block is used for a uni-directional affine coded CU; 8x4/4x8 sub-block is used for a bi-directional affine coded CU according to its width and height conditions. 8x4 is used if its width is not shorter than its height. Otherwise 4x8 is used.

CE2-4.2.d) 4x4 sub-block is used for a uni-directional affine coded CU; 8x8 sub-block is used for a bi-directional affine coded CU.

# Results

The results of each subtest of CE2-4.2 are shown below against VTM4.0 [3] under CTC [4].

Table 1. Performance of CE2-4.2.a

Table 2. Performance of CE2-4.2.b

Table 3. Performance of CE2-4.2.c

Table 4. Performance of CE2-4.2.d

# Conclusion

In this proposal, results are provided for test CE2-4.2, according to which the bi-directional MC block size is restricted to be at least 4x8/8x4 or larger to reduce the worst-case memory access bandwidth for motion compensation in VVC. The proposal results in 32% reduction of the worst-case bandwidth requirement for the current VVC, with about 0.1% BD-rate increase. It is recommended to consider adoption of the proposed restriction into VVC.

# Reference

- “CE2: Reducing worst-case memory bandwidth of affine mode (test 2.4.1)”, Y.-W. Chen, X. Wang, doc. JVET-M0226, January 2019.

- “CE2: Affine sub-block size restrictions (Test 2.4.4)”, H. Chen, T. Solovyev, H. Yang, J. Chen, doc. JVET-M0472, January 2019.

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).