| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0325_r2 |
| --- | --- |

| Title: | CE9-related: Improvements on bi-directional optical flow (BDOF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang} @kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Bi-directional optical flow (BDOF) is included in the VVC to enhance the granularity and accuracy of the motion vectors (MVs) used for motion compensation. Additionally, in the current BDOF design, the number of bitwise right shifts, which are applied to calculate certain intermediate parameters, is determined based on the internal bit-depth ranging from 8-bit to 12-bit.

The contribution proposes to unify all the bit-shift operations in the BDOF by extending the right shifts that are used for internal 8-bit to be used for internal 10-bit and 12-bit. The performance of the proposed BDOF modifications are summarized as follows:

For 10-bit coding (CTC): {Y, U, V} BD-rate of {-0.01%, -0.01%, -0.03%}, Enc: 102%, Dec: 101%;

For 8-bit: coding:       {Y, U, V} BD-rate of { 0.00%, 0.00%, 0.00%}, Enc: 100%, Dec: 100%;

For 12-bit coding:       {Y, U, V} BD-rate of {-0.17%, -0.10%, -0.09%}, Enc: 100%, Dec: 100%.

# Introduction

In the VVC working draft 4 [1], bi-directional optical flow (BDOF) is applied, which provides the motion refinements based on 4×4 subblocks for bi-predicted coding units (CUs) that has both forward and backward predictions. In addition, at the Marrakech meeting, JVET-M0063 [2] was adopt which applies different numbers of bitwise right shifts to calculate the sample gradients, the difference between L0 and L1 predictions and the sum of the gradients of L0 and L1 predictions. Specifically, the BDOF derivation method in the VVC working draft 4 [1] can be summarized as the following main steps:

- Gradient derivation: the horizonal and vertical gradients are calculated as the difference between two neighboring prediction samples, i.e.,

| ∂I0/1∂xi, j=I0/1i+1, j-I0/1i-1, j≫shift1 ∂I0/1∂yi, j=I0/1i, j+1-I0/1i, j-1≫shift1 | (1) |
| --- | --- |

- Correlation parameter calculation: the BDOF parameters ψxi,j, ψyi,j and θi,j are firstly derived as

| θi,j=I1i, j≫shift2-I0i, j≫shift2 ψxi,j=∂I1∂xi, j+∂I0∂xi, j≫shift3 ψyi,j=∂I1∂yi, j+∂I0∂yi, j≫shift3 | (2) |
| --- | --- |

Then, the following BDOF correlation parameters can be calculated as

| S1=(i,j)∈Ωψxi,j∙ψxi,j, S3=(i,j)∈Ωθi,j∙ψxi,j S2=(i,j)∈Ωψxi,j∙ψyi,j S5=i,j∈Ωψyi,j∙ψyi,j S6=(i,j)∈Ωθi,j∙ψyi,j | (3) |
| --- | --- |

- Motion refinement derivation: the refined MV vx,vy  is derived as

| vx=S1>0?clip3-thBIO',thBIO',-S3∙2shift2-shift3≫log2S1:0 vy=S5>0?clip3-thBIO',thBIO',-S6∙2shift2-shift3-vxS2,m≪12+vxS2,s2≫log2S5:0 | (4) |
| --- | --- |

where S2,m=S2≫12 , S2,s=S2&212-1, and thBIO'=2shift6.

In equations (1), (2) and (4), the variables shift1, shift2, shift3 and shift6 are dependent on the internal bit-depth BD, which are set equal to Max(2, 14 - BD), Min(8, BD - 4), Min(5, BD - 7) and Max(1, 13-BD), respectively.

# Proposal

This contribution proposes to unify all the bitwise shift operations used in the BDOF by extending the right shifts that are used for internal 8-bit to be used for internal 10-bit and 12-bit. Specifically, based on the proposed unification, the variables shift1, shift2, shift3 and shift6 in equations (1), (2) and (4) are fixed to be the values when the variable BD is set equal to 8, i.e., they are always set equal to 6, 4, 1and 5, respectively, independent on the internal bit-depth that is used for the coding.

# Simulation results

The proposed methods were implemented based on the VTM-4.0 reference software [3]. The following three tests are conducted in the contribution to verify the performance of the proposed methods.

- Test One: both the anchor and the test are generated by following the common test conditions (CTC) as specified in [4].

- Test Two: both the anchor and the test are generated by following the CTC except that the value of InternalBitDepth is set to 8. Additionally, due to the reduced internal bit-depth, only the 8-bit test sequences in Class C, D and F are tested.

- Test Three: both the anchor and the test are generated by following the CTC except that the value of InternalBitDepth is set to 12. Additionally, the bug-fix of the in-loop reshape in merge request #330 [5] is applied in both anchor and the test in order to avoid high bit-depth overflow of reshape look-up table.

## BD-rate performance of Test One

## BD-rate performance of Test Two

## BD-rate performance of Test Three

# Proposed spec changes

#### 8.5.7.4 Bidirectional optical flow prediction process

……

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 2, 14 − bitDepth ) Max(6, bitDepth-6).

- The variable shift2 is set to equal to Max( 8, bitDepth − 4 ) Max(4, bitDepth-8).

- The variable shift3 is set to equal to Max( 5, bitDepth − 7 ) Max(1, bitDepth-11).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to Max( 2, 1  <<  ( 13 − bitDepth ) ) 1<<Max(5, bitDepth-7).

……

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij ( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-862)

sGy2 = ij(tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-863)

sGxGy = ij(tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-864)

sGxdI = ij( − tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-865)

sGydI = ij( − tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-866)

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

T. Chujoh, T. Ikai, “Non-CE9: An improvement of BDOF”, JVET-M0063, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/330.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).