| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0326 |
| --- | --- |

| Title: | Non-CE7: Simplification of cbf coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, the intra sub-partition flag, cbf of previous sub-partition and transform depth are utilized to select one out of four contexts for coding the cbf of a luma block. The transform depth is utilized to select one out of two contexts for coding the cbf of a Cb block while the cbf of the corresponding Cb block is utilized to select one out of the two contexts for coding the cbf of the Cr block. In this proposal, it is proposed to remove the context selection based on intra sub-partition flag, cbf of previous sub-partition and transform depth. In the proposed simplification, only one context is used by the luma cbf coding and only one context is used by the Cb cbf coding. Two contexts are still used by Cr cbf coding which is the same as current VVC. In summary, the context selection for cbf coding of luma and Cb blocks are totally removed and the CABAC contexts for cbf coding are reduced by four. The simulation results reportedly show that the proposed simplifications shows {0.04%, 0.01%, 0.00%} and {-0.01%, 0.63%, -0.10%} BD-rates changes on Y, Cb and Cr components for AI and RA over VTM-4.0.

# Introduction

In VVC, a coefficient block flag (cbf) is signaled for each transform block (e.g. could be a luma transform block or chroma transform block) to indicate whether there are coefficients in this transform block and the corresponding context selection procedures are illustrated as below:

##### Derivation process of ctxInc for the syntax element tu_cbf_luma

There are four CABAC contexts could be utilized to coding the tu_cbf_luma and the procedure to determine the context index (ctxInc) is illustrated below. The variable trDepth specifies the current transform block is smaller than the current CU.

- If IntraSubpartitionSplitType is equal to ISP_NO_SPLIT or current block is not luma block, the following applies:

ctxInc = trDepth = = 0  ?  1  :  0

- Otherwise (IntraSubpartitionSplitType is not equal to ISP_NO_SPLIT and current transform block is luma block), the following applies:

  - The variable prevTuCbfY is derived as follows:

  - If the current transform unit is the first one to be parsed in a coding unit, prevTuCbfY is set equal to 0.

  - Otherwise, prevTuCbfY is set equal to the value of tu_cbf_luma of the previous luma transform unit in the current coding unit.

  - The variable ctxInc is derived as follows:

ctxInc = 2 + prevTuCbfY

##### Derivation process of ctxInc for the syntax element tu_cbf_cb

There are two CABAC contexts could be utilized to coding the tu_cbf_cb and the procedure to determine the context index (ctxInc) is illustrated below. The variable trDepth specifies the current transform block is smaller than the current CU.

ctxInc = trDepth = = 0  ?  0  : 1

##### Derivation process of ctxInc for the syntax element tu_cbf_cr

There are two CABAC contexts could be utilized to coding the tu_cbf_cr and the procedure to determine the context index (ctxInc) is illustrated below. The variable trDepth specifies the current transform block is smaller than the current CU.

ctxInc = the value of tu_cbf_cb in the current coding unit

# Proposed Methods

First, it is proposed to remove the ISP-dependent context selection for the cbf coding of a luma block. Secondly, it is proposed to remove the transform depth dependent context selection for the cbf coding of the luma and cb blocks. Thirdly, we combine the first two modifications. In the combined scheme, only one context is used by coding cbf for luma block and one context for Cb blocks and thus the context selection process is totally removed for the cbf coding of luma and Cb blocks. The coding of the cbf for Cr block is kept the same without any change in the proposed scheme.

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results of the proposed implementation under CTC [2] are shown below against VTM-4.0.

Table 1. Performance of removing the ISP-dependent context selection

Table 2. Performance of removing the transform depth dependent context selection

Table 3. Performance of removing the combined simplifications

# Conclusion

In this proposal, a simplification on cbf coding is proposed to reduce the condition checking for context selection as well as the number of CABAC context models. The modification provides {0.04%, 0.01%, 0.00} and {-0.01%, 0.63%, -0.10%} BD-rate increase for Y, Cb and Cr components under AI and RA, respectively. It is recommended to consider adoption of the proposed simplification into VVC for cbf coding.

# Reference

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).