##### 9.5.4.2.5 Derivation process of ctxInc for the syntax element tu_cbf_luma

Inputs to this process are the variable binIdx, the colour component index cIdx, the binary logarithm of the transform block width log2TbWidth and the transform block height log2TbHeight and the current transform tree depth trDepth.

Output of this process is the variable ctxInc.

The variable ctxInc is derived as follows:

- If IntraSubpartitionSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

ctxInc = 1trDepth = = 0  ?  1  :  0		(9-32)

- Otherwise ( IntraSubpartitionSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

- The variable prevTuCbfY is derived as follows:

- If the current transform unit is the first one to be parsed in a coding unit, prevTuCbfY is set equal to 0.

- Otherwise, prevTuCbfY is set equal to the value of tu_cbf_luma of the previous luma transform unit in the current coding unit.

- The variable ctxInc is derived as follows:

ctxInc = 2 + prevTuCbfY

Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins

| tu_cbf_luma[ ][ ][ ] | 00,1,2,3 (clause 9.5.4.2.5) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| tu_cbf_cb[ ][ ][ ] | 0trDepth = = 0 ? 0 : 1 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | tu_cbf_cb[ ][ ][ ] | na | na | na | na | na |