| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0327 |
| --- | --- |

| Title: | CE10-related: Simplification on combined inter and intra prediction (CIIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, two simplification methods are proposed to reduce the computational complexity of combined inter and intra prediction (CIIP) in the VVC working draft. In the first method, it is proposed to exclude decoder-side motion vector derivation (DMVR) and bi-directional optical flow (BDOF) when generating the inter prediction samples in the CIIP mode. In the second method, it is proposed to restrict the inter prediction of the CIIP mode to always be uni-prediction when generating the inter prediction samples. Compared to the VTM-4.0 anchors, the first method reportedly leads to {Y, U, V} BD-rate variations of {-0.05%, -0.11%, -0.15%} for RA configuration. The second method reportedly results in {Y. U, V} BD-rate variations of {-0.01%, -0.14%, -0.15%} and {-0.05%, 0.00%, -0.19%} for RA and LD configurations, respectively.

# Introduction

In the VVC working draft 4 [1], combined inter and intra prediction (CIIP) is applied, which combines the intra prediction and the inter prediction of one inter merge coding unit (CU). Due to the reconstructed neighboring samples used by the intra prediction, the CIIP needs to be carried out in the reconstruction loop where the decoding is done sequentially. Therefore, the number of computational operations (e.g., multiplications, additions and bitwise shifts) used in the CIIP cannot be too high to ensure enough throughput for real-time decoding.

# Proposal

For better decoding throughput, this contribution proposes two methods to reduce the computational complexity of the CIIP mode, as described as follows:

- For the CIIP design in the VVC working draft, decoder-side motion vector derivation (DMVR) and bidirectional optical flow (BDOF) are always enabled to generate the inter prediction samples when the current CIIP CU is bi-predicted. Given the additional computations resulting from the DMVR and BDOF, it is proposed in the first method to always disable those two tools for the inter prediction of the CIIP mode.

- In the second method, it is proposed to restrict the inter prediction that is used to generate the combined prediction for the CIIP to always be uni-directional. To convert bi-prediction to uni-prediction, the proposed method always selects the uni-prediction from list L0 when the current CU is bi-predicted.

# Simulation results

The proposed methods were implemented based on the VTM-4.0 reference software [2] and tested by following the common test conditions (CTC) as specified in [3].

## BD-rate performance of Method One

## BD-rate performance of Method Two

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).