| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0328 |
| --- | --- |

| Title: | Non-CE9: Mutually exclusive DMVR/BDOF at CU level |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, DMVR and BDOF could be both enabled in one CU where final MC has a dependency on DMVR, and BDOF has a dependency on final MC samples which introduces latency issue. In this proposal, a scheme of mutually exclusive DMVR and BDOF at CU-level is proposed. In the proposed scheme, an MVD-based criterion is used to select DMVR or BDOF. When the motion vector difference (MVD) between the selected candidate and any previous ones is within a pre-defined threshold, BDOF is exclusively applied; otherwise DMVR is exclusively applied. The results show 0.33% BD-rate increase against VTM-4.0, with 2% and 6% encoding and decoding time reduction, respectively.

# Introduction

In current VVC, a CU coded as regular merge mode may apply both DMVR and BDOF, where DMVR is first performed on sub-PUs of a CU. BDOF is then applied based on the refinement results of DMVR for the sub-PU. The co-existence of DMVR and BDOF on one CU may cause latency issue for hardware implementation. In M0223[1], it was proposed to mutually apply DMVR/BDOF at a sub-PU level. In the proposed second scheme in M0223, both DMVR and BDOF are applied to one CU based the original MV and then either DMVR refined results or BDOF refined results are selected based on the motion vector refined by DMVR. Although M0223 solve the latency issue, it requires all the computation needed for both DMVR and BDOF which is still a burden at decoder.

# Proposed Methods

To solve the latency and the computational complexity caused by applying both DMVR and BDOF in one CU, it is proposed to mutual exclusively apply DMVR or BDOF in a CU. In the proposed scheme, for a CU which could applies both DMVR and BDOF, if the MV difference between the selected candidate and any of the previous candidates in the same merge list is not less than a pre-defined threshold (that is, ¼-, ½- and 1-pixel-wide intervals for blocks with less than 64, less than 256 and more than 256 pixels, respectively), BDOF is applied; otherwise, DMVR is applied.

# Results

The proposed modification is integrated into VTM-4.0 [2] and the results of the proposed implementation under CTC [3] are shown in Table 1 against VTM-4.0. The second scheme proposed in M0223 is also implemented in VTM-4.0 and the results are illustrated in Table 2. It is observed that the proposed method provides better coding performance and lower computation burden in comparison to M0223.

Table 1. Performance of the proposed simplification

Table 2. Performance of the second scheme in M0223

To compensate the drop of coding gain, additional tests is carried out by applying BDOF to Sub-block TMVP (SbTMVP) on top of the proposed scheme. The results are illustrated in Table 3 and the results show that BDOF on SbTMVP provides 0.08% gain.

Table 3. Performance of the proposed modification with enabling of BDOF on SbTMVP

# Conclusion

In this proposal, a scheme of mutually exclusive DMVR/BDOF at CU level is proposed to solve the latency issue and reduce the computational complexity of enabling both DMVR and BDOF in one CU. It is recommended to consider adoption of the proposed simplification into VVC.

# Reference

- “ Non-CE9: Co-existence analysis for DMVR with BDOF”, S. Sethuraman, doc. JVET-M0223, January 2019.

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).