| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0329 |
| --- | --- |

| Title: | CE8-related: Encoder improvements on IBC search |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 { xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, several encoder improvements are proposed to enhance the intra block copy (IBC) search in the VTM-4.0, including 1) enhanced IBC local search by using block vector (BV) candidates from previously IBC coding units (CUs); 2) enhanced predictor-based IBC search by considering the BV candidates derived by history-based BV prediction (HBVP) and extended BV derivation; 3) adaptive SAD and SATD for encoder rate-distortion (R-D) calculation; 4) simplified IBC hash table generation in original sample domain. Compared to CE8 anchors, for Class SCC sequences, the proposed encoder improvements reportedly provide 1.26%, 0.84% and 0.34% BD-rate reductions for AI, RA and LD configurations, respectively, without increasing encoding and decoding time.

# Introduction

At the 12th JVET meeting, intra block copy (IBC) was adopted into the VVC working draft, which predicts the samples of one current CU from the already reconstructed samples in the current CTU and partial left CTU [1]. Additionally, one fast IBC search is applied at encoder to estimate the block vectors (BVs) for non-merge IBC CUs. Figure 1 illustrates the IBC search process in the VTM-4.0 [2], which mainly contains the following three steps:

- The first step is hash-based BV search where the hash value of each 4×4 block in the current picture is calculated. One CU is regarded as perfectly matching with its reference block only if the hash values of all the 4×4 subblocks with in the CU match that of its collocated subblocks in the reference block.

- Due to quad/binary/ternary-tree, one same CU can be obtained by multiple partition combinations. when the hash-based IBC search cannot provide valid BV candidates, one cached BV search is applied. The method stores the best BV when one CU is encoded at the first time. Then, when the same CU is encoded again through another partition path, only the stored BV is tested.

- When none of the hash-based IBC search and the cached BV search succeeds, one local BV search is applied based on block matching. The local BV search examines two kinds of BV candidates, i.e., predictor-based BV candidates and spatial BV candidates. The predictor-based BV candidates include the BVs of the 5 spatial neighboring blocks used for the IBC merge mode and their corresponding derived BVs if the reference block associated with one neighboring BV is also IBC-coded. The spatial BV candidates include the candidate positions within one pre-defined search range left to and on top of the current CU.

Figure 1 Flowchart of the encoder IBC search process in the VTM-4.0

# Proposal

To further improve the IBC coding efficiency, the following non-normative methods are proposed to improve some modules of the existing IBC search in Figure 1.

- Local BV search improvement: one BV candidate table is maintained at encoder which contains at most 64 BV candidates of previously coded CUs. Specifically, the table is emptied at the beginning of one picture and updated after the IBC search of each CU is done by adding the 8 best BV candidates of the CU to replace 8 old BV candidates in the table based on first-in-first-out rule. The BV candidates in the table will be used as the BV candidates for the local BV search of one current CU.

- Predictor-based BV search improvement: in addition to 5 spatial neighboring BVs, it is proposed to include the BV candidates (up to 5) derived from the HBVP for the predictor-based BV search. Moreover, one extended BV derivation method is applied to generate additional BV candidates. As shown in Figure 2, similar to the existing BV derivation in the VTM-4.0, when the reference block of one IBC CU is also IBC-coded, one additional BV can be derived by the proposed method through adding the BVs of the two blocks. However, unlike the existing method that derives BV only once, the proposed method continues the BV derivation until the reference block pointed by the newly derived BV is no longer one IBC block.

Figure 2 Extended BV derivation

- Adaptive SAD/SATD for encoder RD: because transform skip mode is frequently selected for screen content video, the SATD is inaccurate to estimate the overhead of residual coding. Therefore, it is proposed to adaptively replace the SATD by the SAD for calculating the R-D cost at encoder. To select between the SAD and SATD, similar encoder method used for adaptive merge with motion vector difference (MMVD) in the VTM-4.0 is used by computing the percentage of hash-matching 4×4 blocks in one picture. If the percentage is larger than one threshold (i.e., 60%), the SAD is selected; otherwise, the SATD is selected.

- Simplified hash table generation: in the VTM-4.0, when the luma mapping with chroma scaling (LMCS) tool is enabled, the IBC hash table is generated using mapped luma samples. This increases the complexity of hash table generation due to the additional sample-wise mapping operations. To reduce the complexity, it is proposed to generate the IBC hash table using the original luma samples.

- A bug fix for locating the collocated 4×4 subblocks for hash-based BV search.

# Simulation results

The proposed methods were implemented based on the VTM-4.0 reference software. The test conditions and evaluation criterion that are used in CE8 [3] are applied. Table 1 shows the overall BD-rate performance of the proposed encoder improvements for AI, RA and LD configurations, respectively.

Table 1 BD-rate performance of the proposed IBC search improvements compared to CE8 anchors

Table 2 summarizes the AI results of “tool-on” test for each of the proposed IBC search improvement methods. In Table 2, the anchor is the CE8 anchor and each tested case is the anchor plus enabling one IBC search improvement.

Table 2 Summary of the individual BD-rate savings of different IBC search improvement methods

|  | Class SCC |  |  | Class F |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Local BV search improvement | -0.24% | -0.22% | -0.22% | -0.14% | -0.06% | -0.17% |
| Predictor BV search improvement | -0.33% | -0.60% | -0.56% | -0.08% | -0.24% | -0.31% |
| Local BV search improvement + Predictor BV search improvement | -0.89% | -1.03% | -1.01% | -0.33% | -0.43% | -0.47% |
| Adaptive SAD/SATD selection | -0.31% | -0.15% | -0.14% | -0.04% | 0.00% | 0.00% |
| Simplified IBC hash table generation | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Bug-fix of hash-based BV search | -0.21% | -0.33% | -0.33% | 0.01% | -0.07% | -0.10% |

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-M1028, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).