| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 25–29 Mar. 2019 | Document: JVET-N0333 |
| --- | --- |

| Title: | CE3-related: MMLM only cross-component prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yi-Wen Chen Xianglin Wang | Email: | jhuhong-jheng@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, a multiple-model linear mode (MMLM) only cross-component prediction is presented. In current VVC Draft 4 [1], there are three cross-component linear model (CCLM) modes, including INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM. In this proposal, MMLM is always used in the three LM modes to derive the CCLM predictors. With the proposed modification, the coding gain of MMLM is obtained with no additional modes added comparing to current VVC. Simulation results reportedly show {-0.31%, -0.84%, -1.30%} BD-rates on Y, Cb and Cr components for AI configuration over VTM-4.0, with 100% encoding time and 99% decoding time.

# Introduction

To reduce the cross-component redundancy, a cross-component linear model (CCLM) prediction mode is used in VVC, for which the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

predCi, j=α·recL'i, j+ β

The reconstructed luma and chroma samples from the top and left neighbouring blocks are used to derive the parameters (α and β) of the model at the encoder and the decoder. The min-max method is used for the derivation of the CCLM parameters. The parameters are then used to derive the chroma sample prediction of the current CU from the reconstructed luma samples of the current CU. In current VVC Draft 4 [1], there are three cross-component linear model (CCLM) modes, including INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM. In INTRA_LT_CCLM mode, both left and top neighbouring reference samples are used for parameter derivation, whereas in INTRA_L_CCLM and INTRA_T_CCLM only the left neighbouring samples and the top neighbouring samples, respectively, are used for the derivation of the parameters of the model.

# Proposed Implementation

MMLM and normal CCLM co-exist in current MMLM related CE tests [2] which requires more additional R-D checks or additional checks to adaptively select between the MMLM or normal CCLM. To remove the additional checks, it is proposed to always use MMLM for the parameter derivation in the conventional three CCLM modes directly. The number of full RD checks are kept aligned with the VTM4 anchor. It is noted that in the proposed scheme, the MMLM derivation is identical to the one used in current MMLM related CE tests.

# Results

The proposed method is integrated based on the software of CE3-1.8.1[2] and the results of the proposed implementation are shown in Table 1 against the VTM4.0 [3] under CTC [4].

Table 1. Performance of the proposed modification

To reduce the worst-case computational complexity of CCLM, the method is also integrated based on the software of CE3-1.8.2 which incorporates block size restriction.

Table 2. Performance of the proposed modification with block size restriction

# Conclusion

In this proposal, a MMLM only cross component prediction is proposed. The results show that the test provides {-0.29%, -0.62%, -1.04%} BD-rate reduction for Y, Cb and Cr, respectively under AI with negligible run time increase. It is recommended to adopt the proposed modifications into VVC.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

- G. Van der Auwera, L. Li, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).