| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0334 |
| --- | --- |

| Title: | CE2/4/9-related: Overflow prevention for motion field storage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 { xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

It is reported that in the VVC working draft 4, the motion vectors (MVs) derived by merge mode with motion vector difference (MMVD) and decoder side motion vector refinement (DMVR) and the control-point motion vectors (CPMVs) derived by constructed affine merge candidates may exceed the range of the valid MV/CPMV values, i.e., 18-bit. In this contribution, it is proposed to clip those problematic MVs/CPMVs to prevent the 18-bit overflow for the MV storage in the VVC.

Additionally, in the current working draft, MV clipping is applied to regular inter non-merge mode and affine non-merge mode to ensure the calculated MVs/CPMVs after the addition of the MV/CPMV predictors and the MV/CPMV differences are within the 18-bit dynamic range. However, such clipping operations are missing in the VTM-4.0 reference software. The contribution proposes to change the software to be aligned with the working draft.

Simulation results show the proposed changes have no impact on the BD-rate performance under common test condition (CTC).

# Technical details

## Preventing MV/CPMV overflow for motion field storage

In VVC working draft 4 [1], one mixed motion field is applied which contains the motion vectors (MVs) of 4×4 subblocks and the control-point motion vectors (CPMVs) of affine coding units (CUs). In addition, the dynamic ranges of both subblocks’ MVs and affine CPMVs are 18-bit, i.e., [-218, 218-1]. At the Marrakech meeting, JVET-M0145 was adopted to avoid the MV overflow by clipping the subblock MVs and CPMVs to 18-bit in affine mode [2]. In JVET-M0145, to avoid the overflow of the CPMVs of affine merge mode, the clipping is only applied to the CPMVs derived by the inherited affine merge candidates.

The contribution studied the dynamic range of the MVs and affine CPMVs for the existing inter coding tools in the VVC and identified the following three cases where the derived MVs/CPMVs can potentially go beyond the range of valid values represented by the allowed bit-depth (i.e., 18-bit) of motion field storage.

- Merge mode with motion vector difference (MMVD): the derived MVs of MMVD mode is generated by applying one signaled MV offset to one selected merge candidate as indicated by the MMVD base index. Even if the selected merge candidate is within the range of valid MVs, due to the applied MV offset, the final MV could be potential outside the 18-bit dynamic range, resulting in the overflow of the derived MVs.

- Decoder side motion vector derivation (DMVR): the DMVR enhances the MVs of one bi-predicted merge candidate by symmetrically applying one motion refinement to its original L0 and L1 MVs. Similar to the MMVD, because of the applied motion refinement, the derived MVs could potential exceed the 18-bit dynamic range.

- Constructed affine merge mode: constructed affine merge candidates of one affine CU are derived from the translational MVs of its spatial neighboring subblocks based on either 4-parameter or 6-parameter affine mode. Even when the used subblock MVs are within the range of valid MVs, the calculated CPMVs may be outside the 18-bit dynamic range.

To prevent the above motion overflow problems, this contribution proposes to clip the derived MVs and CPMVs to the bit-depth of the motion field storage after the motion derivations of the MMVD, the DMVR and the constructed affine merge mode are finished.

## Mismatch between the working draft and the VTM-4.0 on clipping the MVs/CPMVs of non-merge inter modes

In VVC working draft 4 [1], MV clipping is applied to regular inter non-merge mode and affine non-merge mode to ensure the MVs/CPMVs obtained by adding the corresponding MV/CPMV predictors and MV/CPMV differences are within the 18-bit dynamic range.

8.5.2.2 Derivation process for luma motion vectors for merge mode

……

4.  When predFlagLX[ 0 ][ 0 ] is equal to 1, the luma motion vector mvLX[ 0 ][ 0 ] is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 218 ) % 218                 	(8-272)

mvLX[ 0 ][ 0 ][ 0 ] = ( uLX[ 0 ]  >=  217 ) ? ( uLX[ 0 ] − 218 ) : uLX[ 0 ]	(8-273)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 218 ) % 218	                      (8-274)

mvLX[ 0 ][ 0 ][ 1 ] = ( uLX[ 1 ]  >=  217 ) ? ( uLX[ 1 ] − 218 ) : uLX[ 1 ]	(8-275)

8.5.5 Derivation process for subblock motion vector components and reference indices

……

5.  When predFlagLX[ 0 ][ 0 ] is equal to 1, the luma motion vectors cpMvLX[ cpIdx ] with cpIdx ranging from 0 to NumCpMv − 1, are derived as follows:

uLX[ cpIdx ][ 0 ] = ( mvpCpLX[ cpIdx ][ 0 ] + mvdCpLX[ cpIdx ][ 0 ] + 218 ) % 218	(8-574)

cpMvLX[ cpIdx ][ 0 ] = (uLX[ cpIdx ][ 0 ]  >=  217 ) ? (uLX[ cpIdx ][ 0 ] − 218 ) : 
	uLX[ cpIdx ][ 0 ]	(8-575)

uLX[ cpIdx ][ 1 ] = ( mvpCpLX[ cpIdx ][ 1 ] + mvdCpLX[ cpIdx ][ 1 ] + 218 ) % 218	(8-576)

cpMvLX[ cpIdx ][ 1 ] = (uLX[ cpIdx ][ 1 ]  >=  217 ) ? (uLX[ cpIdx ][ 1 ] − 218 ) : 
	uLX[ cpIdx ][ 1 ]	(8-577)

However, the above clipping operations are missing in the VTM-4.0 reference software [3]. To fix such mismatch, the contribution proposes to change the software to be aligned with the working draft.

# Simulation results

The proposed method was implemented based on the VTM-4.0 reference software [3] and tested based on the common test conditions as specified in [4].

## BD-rate performance of the proposed MV/CPMV overflow solution

## BD-rate performance of the proposed mismatch bug-fix on MV/CPMV clipping of non-merge inter modes

# Proposed spec changes

#### 8.5.1 General decoding process for coding units coded in inter prediction mode

……

- If dmvrFlag is equal to 1, the derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with refMvLX[ xSbIdx ][ ySbIdx ] and refIdxLX as inputs, and refMvCLX[ xSbIdx ][ ySbIdx ] as output and the input refMvLX[ xSbIdx ][ ySbIdx ] is derived as follows;

refMvLX[ xSbIdx ][ ySbIdx ] = mvLX[ xSbIdx ][ ySbIdx ] + dMvLX[ xSbIdx ][ ySbIdx ]	(8-263)

refMvLX[ xSbIdx ][ ySbIdx ][ 0 ] = Clip3( −217, 217 − 1, refMvLX[ xSbIdx ][ ySbIdx ][ 0 ] )  (8-xxx)

refMvLX[ xSbIdx ][ ySbIdx ][ 1 ] = Clip3( −217, 217 − 1, refMvLX[ xSbIdx ][ ySbIdx ][ 1 ] )  (8-xxx)

……

#### 8.5.2.2 Derivation process for luma motion vectors for merge mode

……

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]                          (8-292)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]                          (8-293)

mvLX[ 0 ][ 0 ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 0 ] )      (8-xxx)

mvLX[ 0 ][ 0 ] [ 1 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 1 ] )      (8-xxx)

……

#### 8.5.5.6 Derivation process for constructed affine control point motion vector merging candidates

……

      - When availableFlagLX is equal to TRUE, the following assignments are made:

predFlagLXConst2 = 1	                                            	(8-694)

refIdxLXConst2 = refIdxLXCorner[ 0 ]	                                    (8-695)

cpMvLXConst2[ 0 ] = cpMvLXCorner[ 0 ]                                	(8-696)

cpMvLXConst2[ 1 ] = cpMvLXCorner[ 1 ]	                                    (8-697)

cpMvLXConst2[ 2 ] = cpMvLXCorner[ 3 ] + cpMvLXCorner[ 0 ] − cpMvLXCorner[ 1 ]	(8-698)

cpMvLXConst2[ 2 ][ 0 ] = Clip3( −217, 217 − 1, cpMvLXConst2[ 2 ][ 0 ] )       	(8-xxx)

cpMvLXConst2[ 2 ][ 1 ] = Clip3( −217, 217 − 1, cpMvLXConst2[ 2 ][ 1 ] )       	(8-xxx)

……

      - When availableFlagLX is equal to TRUE, the following assignments are made:

predFlagLXConst3 = 1	                                            	(8-699)

refIdxLXConst3 = refIdxLXCorner[ 0 ]	                                    (8-700)

cpMvLXConst3[ 0 ] = cpMvLXCorner[ 0 ]	                                    (8-701)

cpMvLXConst3[ 1 ] = cpMvLXCorner[ 3 ] + cpMvLXCorner[ 0 ] − cpMvLXCorner[ 2 ]	(8-702)

cpMvLXConst3[ 1 ][ 0 ] = Clip3( −217, 217 − 1, cpMvLXConst3[ 1 ][ 0 ] )       	(8-xxx)

cpMvLXConst3[ 1 ][ 1 ] = Clip3( −217, 217 − 1, cpMvLXConst3[ 1 ][ 1 ] )       	(8-xxx)

cpMvLXConst3[ 2 ] = cpMvLXCorner[ 2 ]                               	(8-703)

……

      - When availableFlagLX is equal to TRUE, the following assignments are made:

predFlagLXConst4 = 1	                                            	(8-704)

refIdxLXConst4 = refIdxLXCorner[ 1 ]	                                    (8-705)

cpMvLXConst4[ 0 ] = cpMvLXCorner[ 1 ] + cpMvLXCorner[ 2 ] − cpMvLXCorner[ 3 ]	(8-706)

cpMvLXConst4[ 0 ][ 0 ] = Clip3( −217, 217 − 1, cpMvLXConst4[ 0 ][ 0 ] )       	(8-xxx)

cpMvLXConst4[ 0 ][ 1 ] = Clip3( −217, 217 − 1, cpMvLXConst4[ 0 ][ 1 ] )       	(8-xxx)

cpMvLXConst4[ 1 ] = cpMvLXCorner[ 1 ]                               	(8-707)

cpMvLXConst4[ 2 ] = cpMvLXCorner[ 2 ]                                	(8-708)

……

      - The following assignments are made:

predFlagLXConst6 = 1		                                           (8-715)

refIdxLXConst6 = refIdxLXCorner[ 0 ]	                                     (8-716)

cpMvLXConst6[ 0 ] = cpMvLXCorner[ 0 ]	                                     (8-717)

cpMvLXConst6[ 1 ] = cpMvLXCorner[ 1 ]	                                     (8-718)

cpMvLXConst6[ 0 ][ 0 ] = Clip3( −217, 217 − 1, cpMvLXConst6[ 0 ][ 0 ] )       	(8-xxx)

cpMvLXConst6[ 0 ][ 1 ] = Clip3( −217, 217 − 1, cpMvLXConst6[ 0 ][ 1 ] )       	(8-xxx)

cpMvLXConst6[ 1 ][ 0 ] = Clip3( −217, 217 − 1, cpMvLXConst6[ 1 ][ 0 ] )       	(8-xxx)

cpMvLXConst6[ 1 ][ 1 ] = Clip3( −217, 217 − 1, cpMvLXConst6[ 1 ][ 1 ] )       	(8-xxx)

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

P. Hanhart, Y. He, “Non-CE2: Motion vector clipping in affine sub-block motion derivation”, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).