| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0335 |
| --- | --- |

| Title: | CE2/4-related: unification of MV rounding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 { xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the VVC working draft 4, different motion vector (MV) rounding methods are used by advanced motion vector prediction (AMVP), subblock-based temporal motion vector prediction (SbTMVP), merge mode with motion vector difference (MMVD), pairwise merge, affine mode and triangle prediction. This contribution proposes to unify all the MV rounding operations in the VVC to be the same as that used by the MV scaling of the AMVP, SbTMVP, MMVD and triangle prediction. Simulation results reportedly show that the proposed unification provides average {Y, U, V} BD-rate savings of {0.02%, 0.03%, 0.02%} and {0.05%, -0.01%, 0.34%} for RA and LD configurations, respectively.

# Proposal

In the VVC working draft 4 [1], MV rounding is performed at various stages of inter prediction, including 1) the MV scaling when deriving the candidate MVs of advanced motion vector prediction (AMVP), subblock-based temporal motion vector prediction (SbTMVP), merge mode with motion vector difference (MMVD) and triangle prediction; 2) the MV derivation of affine mode; 3) the averaging of two MV candidates for pairwise merge and triangle prediction. Additionally, MV rounding operations are also used by adaptive motion vector resolution (AMVR) at the VTM-4.0 to round the MV predictors to the same precision of the motion vector difference (MVD). However, in the existing design, two difference MV rounding methods are applied for the above inter prediction processes. Specifically, assuming A is the MV value before rounding and Shift is the bitwise right shift, the two MV rounding methods are described as

- Method one: the MV rounding used for the MV scaling operations involved in the AMVP, SbTMVP, MMVD and triangle prediction,

| L = (A + Offset + (A < 0? 1: 0))>>Shift | (1) |
| --- | --- |

- Method two: the MV rounding used for the affine MV derivation, the MV averaging of pairwise merge and triangle prediction, and the MV precision adjustment of the AMVR,

| L = (A >= 0)? (A + Offset)>>Shift : -((-A + Offset)>>Shift) | (2) |
| --- | --- |

where Offset is the rounding offset that is set equal to (Shift > 0)? 1<<(Shift - 1) : 0. Figure 1 compares the mapping functions of the two MV rounding functions.

Figure 1 Comparison of the mapping function of the MV rounding functions in the VVC: (left) Method one; (right) Method two.

As illustrated in Figure 1, both the mapping functions of two MV rounding methods are symmetric about the original of the coordinates. Additionally, Method one is simpler than Method two due to its fewer arithmetic operations.

To achieve one unified design, this contribution proposes to unify all the MV rounding operations in the VTM-4.0 based on Method one.

# Simulation results

The proposed unification method was implemented based on the VTM-4.0 reference software [2] and tested based on the common test conditions as specified in [3]. Table 1 shows the overall BD-rate performance of the proposed method for RA and LD configurations, compared to the VTM-4.0 anchors.

Table 1 BD-rate performance of the proposed method, compared to the VTM-4.0 anchors

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).