| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0336 |
| --- | --- |

| Title: | CE2/4-related: On motion field storage reduction for spatial motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to reduce the size of motion field storage for spatial motion prediction. In the first method, except the motion vector (MV) candidates of history-based motion vector prediction (HMVP), all the MVs of 4×4 subblocks and the control-point motion vectors (CPMVs) of affine coding units (CUs) are converted into one 8-bit mantissa and 4-bit exponent representation prior to the storage. In the second method, only the MVs that stored in line-buffer, i.e., the MVs from neighboring 4×4 subblocks from top CTUs, are represented based on 7-bit mantissa and 4-bit exponent format while all the other MVs and CPMVs in local buffer are still stored in the original bit-depth. It is asserted that for 4K video, both the proposed methods can reduce the size of motion field storage by > 30%. Simulation results show the negligible impact of the proposed methods on BD-rate performance.

# Introduction

In VVC working draft 4 [1], one mixed motion field is applied which contains the motion vectors (MVs) of 4×4 subblocks and the control-point motion vectors (CPMVs) of affine coding units (CUs). In addition, the dynamic ranges of both subblocks’ MVs and affine CPMVs are 18-bit, i.e., [-218, 218-1]. At the Marrakech meeting, JVET-M0512 was adopted to reduce the size of temporal motion buffer [2]. With the adoption of JVET-M0512, two different representation methods are used to store spatial and temporal motion field: the subblocks MVs and affine CPMVs used for spatial motion prediction are stored in original 18-bit while the temporal MVs are stored based on 6-bit mantissa and 4-bit exponent representation, i.e., the temporal motion field is stored more efficiently than spatial motion field.

However, in practice, the implementation cost of motion field storage for spatial MV and CPMV prediction are very expensive, because they need to be stored using on-chip memory. For example, based on the memory analysis methodology in JVET-K0052 [3], Table 1 summarizes the buffer size used to store spatial MVs and CPMVs for 4K video. In Table 1, for easy calculation, only the memory used for storing MVs and CPMVs is counted without considering the memory used to store other related motion information, e.g., reference indices, affine mode flag and CU width and height. As can be seen, approximately 10.8KB on-chip memory is needed to store the MVs and CPMVs for spatial motion prediction.

Table 1 The buffer size used to store spatial MVs and CPMVs in the VVC

| Storage | Local MV storage | Local CPMV storage | MV line-buffer | HMVP buffer | Total |
| --- | --- | --- | --- | --- | --- |
| Size | 864 bytes | 1296 bytes | 8640 bytes | 45 bytes | 10845 bytes |

# Proposal

To reduce memory footprint of the VVC, this contribution proposes the following two methods to reduce the motion field storage size for spatial motion prediction.

Method One: in the first method, it is proposed to store the spatial motion information using a 8-bit mantissa and 4-bit exponent representation to reduce the storage requirement. The converted spatial motion includes the 4×4 subblock MVs and the affine CPMVs stored in the local buffer and the 4×4 subblock MVs stored in the line-buffer, but excludes the HMVP MV candidates. Table 2 shows the buffer usage when the first method is applied. Compared to Table 1, Method One reduces the size of motion storage by around 33%.

Table 2 The buffer size used to store spatial MVs and CPMVs by Method One

| Storage | Local MV storage* | Local CPMV storage* | MV line-buffer* | HMVP buffer | Total |
| --- | --- | --- | --- | --- | --- |
| Size | 576 bytes | 864 bytes | 5760 bytes | 45 bytes | 7245 bytes |

* indicates the motion information that is stored in mantissa-exponent format

Method Two: to avoid frequent MV conversions between different representations, it is proposed in the second method to only convert the 4×4 subblock MVs that are stored in the line-buffer into one 7-bit mantissa and 4-bit exponent prior to the storage. All the other MVs and CPMVs stored in the local buffer and the HMVP MVs are still stored in the original bit-depth, i.e., 18-bit. Table 2 shows the corresponding storage usage when the first method is applied. As can be seen, Method Two reduces the total motion storage size by around 31%.

Table 3 The buffer size used to store spatial MVs and CPMVs by Method Two

| Storage | Local MV storage | Local CPMV storage | MV line-buffer* | HMVP buffer | Total |
| --- | --- | --- | --- | --- | --- |
| Size | 864 bytes | 1296 bytes | 5280 bytes | 45 bytes | 7485 bytes |

* indicates the motion information that is stored in mantissa-exponent format

# Simulation results

The proposed method was implemented based on the VTM-4.0 reference software [4] and tested based on the common test conditions as specified in [5].

## BD-rate performance of Method One

## BD-rate performance of Method Two

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

F. Bossen, K, Misra, A. Segall, “Non-CE4: On Temporal Motion Buffer Compression”, JVET-M0512, Jan. 2019, Marrakech, Morocco.

M. Zhou, B. Heng, “Non-CE4: A study on affine merge mode”, JVET-K0052, Jul. 2018, Ljubljana, Slovenia.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).