| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0337 |
| --- | --- |

| Title: | CE8-related: Adaptive disabling of intra sample interpolation for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to adaptively disable the intra interpolation at fractional sample positions by introducing control flags in SPS and tile group header. Experimental results reportedly show that when the IBC is enabled, the proposed method provides 0.42%, 0.42% and 0.45% BD-rate savings for Class SCC sequences for AI, RA and LD configurations, respectively. When the IBC is disabled, the corresponding BD-rate savings are 3.26%, 1.70% and 0.32% for AI, RA and LD configurations, respectively.

# Introduction

For the intra prediction in VVC working draft 4 [1], when a target sample of one intra angular mode is predicted from its reference sample at fractional sample position, interpolation filtering is applied to generate the intra prediction sample at 1/32-pel accuracy. Specifically, depending on the selected intra mode, either 4-tap DCT-IF filters (the same filter used for chroma motion compensation) or 4-tap gaussian filters are applied for luma component while bilinear filters are applied for chroma components.

Compared to natural content, screen content shows very different characteristics. As shown in Figure 1, because of the nature of camera lens, camera-captured videos usually show smooth boundaries across different objects while screen content presents shape edges. Therefore, for screen content coding (SCC), the intra interpolation may blur the meaningful high-frequency information of the reconstructed neighboring samples, therefore reducing the intra prediction efficiency.

Figure 1 Comparison between (left) natural content and (right) screen content

# Proposal

It is proposed to introduce additional flags in SPS and tile group header to control the enabling and disabling of the intra sample interpolation of intra angular modes for the SCC. When the intra sample interpolation is disabled, the intra prediction sample is directly copied from the reference sample at the integer position that is left to the target fractional position (for angular directions that refer to the above neighboring samples) or below the target fractional position (for angular directions that refer to the left neighboring samples).

Additionally, to determine whether to enable or disable the intra sample interpolation, similar encoder method used for adaptive merge with motion vector difference (MMVD) in the VTM-4.0 [2] is used in the proposed method by computing the percentage of hash-matching 4×4 blocks in one picture. If the percentage is larger than one threshold (i.e., 60%), the intra sample interpolation is disabled; otherwise, the intra sample interpolation is enabled. However, unlike the MMVD encoder method which uses all 4×4 blocks, the 4×4 blocks that are associated with most-used hash values in one picture are excluded when deciding if the intra interpolation should be enabled/disabled. This is because those blocks usually belong to large background areas in one picture which contains less textures and the angular intra modes are usually not selected for those blocks.

# Simulation results

The proposed methods were implemented based on the VTM-4.0 reference software [2] and tested based on the common test conditions as specified in [3]. Table 1 shows the overall BD-rate performance of the proposed method, compared to the VTM-4.0 anchors. Because the proposed method is proposed for the SCC, the SCC test sequences are included for evaluation. Table 2 shows the corresponding BD-rate performance when the IBC is disabled. In Table 2, the anchors are the results of the VTM-4.0 with disabling the IBC mode. To save space, only the AI results are shown in Table 1 and Table 2. The complete simulation results of RA and LD configurations can be found the spreadsheets in the package.

Table 1 BD-rate performance of the proposed method, when IBC is enabled

|  | AI, Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 106% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class F | -0.04% | -0.04% | -0.01% | 98% | 97% |
| Class SCC 1080p | -0.42% | -0.18% | -0.18% | 97% | 100% |

Table 2 BD-rate performance of the proposed method, when IBC is disabled

|  | AI Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | -0.25% | -0.24% | -0.19% | 99% | 101% |
| Class SCC 1080p | -3.26% | -2.80% | -2.82% | 93% | 103% |

Additionally, to demonstrate the benefit of picture-level adaptation, Table 3 and Table 4 show the corresponding BD-rate performance of the proposed method with the IBC being on and off, when the determination of enabling/disabling the intra sample interpolation is made at sequence level. Specifically, in Table 3 and Table 4, the samples of the first picture in each test sequence is used to decide whether to enable or disable the intra sample interpolation and the decision is used for all the pictures in the same video sequence.

Table 3 BD-rate performance of the proposed method (sequence-level adaptation), when IBC is enabled

|  | AI Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class SCC 1080p | -0.17% | -0.07% | -0.08% | 96% | 100% |

Table 4 BD-rate performance of the proposed method (sequence-level adaptation), when IBC is disabled

|  | AI Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class SCC 1080p | -2.17% | -1.93% | -1.93% | 97% | 101% |

# Proposed spec changes

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_gbi_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_intra_sample_interp_enabled_flag | u(1) |
| sps_intra_sample_interp_override_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| …… |  |
| } |  |

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_intra_sample_interp_override_enabled_flag ) |  |
| tile_intra _sample_interp_enabled_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| …… |  |
| } |  |

#### 7.4.3.1 Sequence parameter set RBSP semantics

sps_intra_sample_interp_enabled_flag specifies whether intra sample interpolation is applied for intra prediction. sps_intra_sample_interp_enabled_flag equal to 0 specifies that intra sample interpolation is not used in the CVS. Otherwise (sps_intra_sample_interp_enabled_flag is equal to 1), intra sample interpolation is used in the CVS.

sps_intra_sample_interp_override_enabled_flag equal to 1 specifies the presence of tile_intra _sample_interp_enabled_flag in the tile group header of the pictures referring to the SPS. sps_intra _sample_interp_override_enabled_flag equal to 0 specifies the absence of tile_intra _sample_interp_enabled_flag in the tile group header of the pictures referring to the SPS.

#### 7.4.5.1 General tile group header semantics

tile_intra_sample_interp_enabled_flag specifies whether intra sample interpolation is applied for intra prediction. tile_intra_sample_interp_enabled_flag equal to 0 specifies that intra sample interpolation is not used for all the coding blocks in the current tile group. Otherwise (tile_intra_sample_interp_enabled_flag is equal to 1), intra fractional sample interpolation is used for all the coding blocks in the current tile group. When not present, its value is inferred to be the value of sps_intra_sample_interp_enabled_flag.

#### 8.4.4.2.7 Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

……

- If cIdx is equal to 0, the following applies:

- If tile_intra_sample_interp_enabled_flag is equal to 1, the following is applied:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-139)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(8-140)

- Otherwise, The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]                                                                              (8-xxx)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If tile_intra_sample_interp_enabled_flag is equal to 1, the following is applied:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-141)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-142)

- Otherwise, The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]	                                                                              (8-xxx)

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).