| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 25–29 Mar. 2019 | Document: JVET-N0338 |
| --- | --- |

| Title: | Non-CE6: Unified Transform Selection for implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, the transform selection for ISP and normal intra mode (non-ISP) are unified if MTS is implicit. When MTS is implicit, the proposed configuration shows the results -0.24%, -0.04%, and -0.03% BD-rate difference for AI, RA, and LDB settings, respectively.

# Introduction

In VTM-4.0 [1], the transform selection in implicit MTS is based on the shape of coding block for non-ISP mode and the other transform selection is predefined for ISP mode. Specifically, two separate transform selections are designed for intra coding units in VTM-4.0; one is for non-ISP mode and the other is for ISP mode. This proposal aims to unify the two separate designs into one.

# Proposed Implementation

As described in previous section, we propose a transform selection for both ISP and non-ISP mode when MTS is implicit. The proposed selection scheme is shown in Table 1 and following rules:

- If w=1 or h=1, then there is no horizontal transform for w=1 or vertical transform for h=1, respectively.

- If w=2 or w>32, transformH = DCT-II

- If h =2 or h >32, transformV = DCT-II

- Otherwise:

  - The transform selection is based on Table 1 , which is the same transform table proposed in [2]

Table 1. The proposed transform Selection

| Intra mode | transformH | transformV |
| --- | --- | --- |
| Planar Ang. 31,32,34,36,37 | DST-VII | DST-VII |
| DC Ang. 33, 35 | DCT-II | DCT-II |
| Ang. 2, 4, 6…28,30 Ang. 39,41,43…63,65 | DST-VII | DCT-II |
| Ang. 3,5,7…27,29 Ang. 38,40,42…64,66 | DCT-II | DST-VII |

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results of the proposed implementation are shown below against the VTM4.0 under CTC [3].

Table 4. Performance of the proposed modification under All Intra

Table 5. Performance of the proposed modification under Random Access

Table 6. Performance of the proposed modification under Low Delay B

# Conclusion

In this proposal, we propose an unified transform selection for the intra coding units when MTS is implicit. The result reportedly shows only -0.24%, -0.04%, and -0.03% difference in BD-rate under All intra, RA, and LDB, respectively. It is recommended to adopt the proposed modifications into VVC.

# References

| [1] | "VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | Y. Lin, M. Mao, S. Song, J. Zheng, J. An and C. Zhu, "Prediction dependent transform for intra and inter frame coding," Document JVET-J0064, San Diego, US, 2018. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).