| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0339 |
| --- | --- |

| Title: | Non-CE3: Unification on WAIP for normal and ISP intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, for the ISP mode, the selection of wide-angle mode solely relies on the shape of current CU rather than sub-CU. It is asserted that when the wide-angle mode is enabled for a square sub-CU for the ISP mode, it needs to access more neighboring pixels than the normal intra CU in the same size. In this contribution, it is proposed to unify the wide-angle derivation of non-ISP mode and ISP mode by selecting the wide-angle based on the size of sub-CUs in the ISP mode. Based on the proposed unification, non-ISP and ISP modes can share one same module for the generation of intra predictors. Under the proposed unification, the result reports that the BD-rate difference of AI, RA, and LDB is 0.01%, 0.00%, and -0.01%, respectively.

# Introduction

In VTM-4.0 [1], the ISP vertically or horizontally splits current CU into 2 or 4 sub-CU depending on the shape of current CU, as explained in [2]. Meanwhile, since the quad/binary/ternary partition is used in the VVC and introduces rectangle block with different aspect ratio, the wide-angle mode is applied for better coding efficiency [1]. The wide-angle mode is derived based on the aspect ratio of current CU. As shown Figure 1, the number of reconstructed sampled used for one non-ISP CU are 2w+1 and 2h+1 from above and left neighbors of the CU, respectively. However, the ISP in current VVC derives the wide-angle mode based on the shape of current CU instead of sub-CU. As the example in Figure 2, if the wide-angle mode of ISP depends on the shape of 8x4 CU, the number of reconstructed samples required for the 4x4 sub-CU are 3w+1 and 3h/2+1 (w:4, h:4) from above and left blocks, respectively, while the 4x4 block in non-ISP mode requires 2w+1 and 2h+1 (w:4, h:4) from above and left blocks. As a result, two dedicated hardware designs are needed for non-ISP and ISP in order to select adequate reconstructed samples from neighboring blocks. In other words, the derivation of WAIP in non-ISP and ISP mode is not a unified design.

Figure 1. For the normal intra prediction, the number of reconstructed samples used for different aspect ratios is fixed

Figure 2: Example of 4x4 block for non-ISP and ISP mode. (a) the number of reconstructed samples in non-ISP are 2w+1 and 2h+1 from above and left block, respectively. (c) the 4x4 sub-block in 8x4 ISP requires 3w+1 and 3h/2+1 reconstructed samples from above and left block, resptively.

# Proposed Implementation

In this proposal, it is proposed to derive the wide-angle mode based on the size of sub-CUs in ISP mode. With the proposed modification, the generation of intra predictors for non-ISP CUs and the sub-CUs in ISP mode could be unified so that non-ISP and ISP modes share the same logic rather than two dedicated design.

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results of the proposed implementation are shown below against the VTM4.0 under CTC [3].

Table 3. Performance of the proposed modification under All Intra

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | OverVTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 98% | 98% |
| Class A2 | -0.01% | 0.10% | -0.02% | 101% | 102% |
| Class B | 0.01% | 0.01% | 0.04% | 100% | 95% |
| Class C | 0.00% | -0.03% | -0.07% | 99% | 98% |
| Class E | 0.03% | 0.02% | 0.01% | 101% | 101% |
| Overall | 0.01% | 0.01% | -0.01% | 100% | 98% |
| Class D | 0.01% | -0.02% | -0.06% | 102% | 104% |
| Class F | 0.03% | -0.02% | -0.07% | 99% | 97% |

Table 4. Performance of the proposed modification under Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.03% | 98% | 97% |
| Class A2 | -0.01% | 0.00% | 0.01% | 99% | 99% |
| Class B | 0.02% | -0.12% | -0.01% | 100% | 98% |
| Class C | -0.01% | -0.11% | -0.19% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.04% | 99% | 99% |
| Class D | 0.02% | -0.14% | -0.34% | 98% | 98% |
| Class F | 0.01% | -0.17% | -0.04% | 100% | 100% |

Table 5. Performance of the proposed modification under Low Delay B

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.23% | -0.33% | 99% | 102% |
| Class C | -0.02% | 0.35% | -0.24% | 99% | 92% |
| Class E | 0.01% | 0.95% | 0.14% | 94% | 91% |
| Overall | -0.01% | 0.26% | -0.18% | 98% | 96% |
| Class D | -0.05% | 0.49% | -0.22% | 98% | 99% |
| Class F | -0.13% | 0.41% | 0.15% | 100% | 96% |

# Conclusion

In this proposal, we propose to unify the derivation of wide-angle intra mode between non-ISP and ISP block in order to achieve better area and power result for hardware design without compromising BD-rate performance. The result reportedly shows only 0.01%, 0.00%, and -0.01% difference in BD-rate under AI, RA, and LDB, respectively. It is recommended to adopt the proposed modifications into the VVC working draft.

# References

| [1] | "VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Line-based intra coding mode (Tests 1.1.1 and 1.1.2)," Document JVET-M0102, Marrakesh , MA, 2019. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).