| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0340 |
| --- | --- |

| Title: | CE4-related: An improved method for triangle merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma | Email: | xianglinwang@kwai.com yiwenchen @kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, a new method is proposed to derive uni-prediction merge list for triangle prediction. In this method, regular merge candidate list is re-used for triangle partition merge prediction with no extra motion vector pruning. For each merge candidate in the regular merge candidate list, one and only one of its L0 or L1 motion vector is used for triangle prediction. In addition, the order of selecting the L0 vs. L1 motion vector is based on its merge index parity. With the proposed solution, the regular merge list can be directly used and there is no need to explicitly generate the uni-prediction merge candidate list for triangle prediction. Simulation results reported based on CTC show that the proposed solution incurs an average 0.01% and 0.09% luma coding efficiency loss respectively for random access and low delay B configuration, and no chroma coding efficiency loss relative to VTM-4.0.

# Introduction

In VTM-4.0[1], triangle prediction can be used to enable triangular shape prediction units for inter prediction. It is currently supported under merge mode, with its own merge candidate list construction process that is different from the regular merge candidate list construction.

Figure 1. Motion vectors used for triangle merge list construction

Figure 1 shows the block locations where motion vectors are derived for triangle merge list construction. Block 1 to 5 are spatial neighboring block and block 6 and 7 are collocated temporal block. These up-to seven motion vectors are first put into a neighboring motion vector list. To form a uni-prediction merge list for triangle prediction, this neighboring motion vector list is scanned, and uni-prediction motion vectors are first selected into the merge list. If the merge list is not full, L0 motion vectors of the bi-prediction motion vectors in the neighboring motion vector list are added into the merge list. Then L1 motion vectors of those bi-prediction motion vectors are added. After that, the average motion vector of L0 and L1 motion vectors are added. At last, uni-prediction zero motion vectors may be added to make the merge list full.

Overall, the merge list construction process is quite different from the regular merge list construction.

# Proposed method

According to this proposal, to maximize the common operations shared by different merge modes, the regular merge list is constructed and its motion vectors are used for triangle prediction.

Once the regular merge list is constructed, for each merge candidate in the regular merge candidate list, one and only one of its L0 or L1 motion vector is used for triangle prediction. In addition, the order of selecting the L0 vs. L1 motion vector is based on its merge index parity. More specifically, as shown in Figure 2, for a candidate with an even value index, its L0 motion vector is first chosen to be used for triangle prediction. If it is not available, its L1 motion vector would be used instead. For a candidate with an odd value index, its L1 motion vector is first chosen to be used for triangle prediction. If it is not available, its L0 motion vector would be used instead. In Figure 2, corresponding to each merge index, the motion vector marked with “x” is first selected for triangle prediction.

Figure 2. Uni-prediction MV selection for triangle prediction mode

With the proposed solution, the regular merge list can be directly used and there is no need to explicitly generate the uni-prediction merge candidate list for triangle prediction. At the decoder side, once a triangle merge index is received, the uni-prediction MV candidate can be easily located based on the signaled merge index value.

# Results

The proposed modification is integrated into VTM-4.0 and the results of the proposed implementation under CTC [2] are shown below against VTM4.0.

Table 1. Results of proposed method under Random Access

Table 2. Results of proposed method under Low delay B

# Conclusion

In this proposal, a new merge list construction process was proposed for triangle prediction. The proposed scheme reuses the regular merge list with no extra motion vector pruning. Based on the signaled merge index value, it is also easy to locate a uni-prediction MV candidate under triangle prediction mode. Simulation results based on CTC on VTM-4.0 show minor coding performance impact with the proposed solution. It is recommended to consider adoption of the proposed method into VVC for triangle prediction.

# Reference

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).