#### Derivation process for luma motion vectors for merge triangle mode

This process is only invoked when merge_triangle_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors mvLXA, and mvLXB

- the reference indices refIdxLXA and refIdxLXB

- the prediction list flags predFlagLXA and predFlagLXB

with X being 0 and 1 respectively.

The motion vectors mvLA and mvLB, the reference indices refIdxLA and refIdxLB and the prediction flags predFlagLA and predFlagLB are derived by the following ordered steps:

- The derivation process for merging candidates as specified in clause 8.5.2.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the refIdxLXN, predFlagLXN, mvLXN[ 0 ], mvLXN[ 1 ], and gbiIdxN from the mergeCandList with X being 0 or 1 and N being 0 to 5.

- The variables m and n, being the candidates in the merging candidate list mergeCandList, are derived using merge_triangle_idx0[ xCb ][ yCb ] and as merge_triangle_idx1[ xCb ][ yCb ] follows:

m = merge_triangle_idx0[ xCb ][ yCb ]		(8-491)

n = merge_triangle_idx1[ xCb ][ yCb ] + ( merge_triangle_idx1[ xCb ][ yCb ] >= m ) ? 1 : 0	(8-492)

- Set mvLXA, mvLXB to 0, set refIdxLXA, refIdxLXB to -1, set predFlagLXA and predFlagLXB to 0 with X being 0 and 1 respectivley.

- Let P = m&0x01, Q = 1-P,

if predFlagLPmergeCandList[m] == 1, set mvLPA = MvLPmergeCandList[m], refIdxLPA = refIdxLPmergeCandList[m], predFlagLPA = 1;

Else, set mvLQA = MvLQmergeCandList[m], refIdxLQA = refIdxLQmergeCandList[m], predFlagLQA = 1;

Let P = n&0x01, Q = 1-P,

if predFlagLPmergeCandList[n] == 1, set mvLPB = MvLPmergeCandList[n], refIdxLPB = refIdxLPmergeCandList[n], predFlagLPB = 1;

Else, set mvLQB = MvLQmergeCandList[n], refIdxLQB = refIdxLQmergeCandList[n], predFlagLQB = 1;

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-