| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0343-v2 |
| --- | --- |

| Title: | CE1: Unidirectional illumination compensation (CE1-5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Nan Hu, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents test results for the local illumination compensation (LIC) tool, which is tested in CE1. In the proposal, LIC is applied to AMVP, merge and MMVD tools, and it is disabled for other inter tools and bi-directional prediction. In the CE, two constraints are tested for the neighboring samples: no constraint (a) and only inter coded samples are utilized (b).

Test results show luma BD rate reduction of -0.43% with 113% encoder and 100% decoder runtime, and -0.35% with 115% encoder and 100% decoder runtime in RA configuration; and -0.42% with 111% encoder and 101% decoder run time and -0.36% with 114% encoder and 101% decoder run time in LB configuration for the tests CE1-5.1a and CE1-5.1b respectively.

# Introduction

Unidirectional LIC method was proposed in JVET-M0500, it uses linear regression to estimate linear model parameters. The input to the linear regression is reconstructed neighboring samples of the current and reference CUs.

Two constraints of using current CU neighboring samples are tested:

- No constraint

- Only inter coded samples are used. In this test, the neighboring samples of the current CU, which are not inter coded, are replaced with the corresponding (collocated) neighbor samples of the reference CU before deriving linear model parameters.

# Proposed method

As was agreed at the Marrakech JVET meeting, the LIC method is applied in the following configuration:

- Usage of reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP and IBC blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

LIC flag is included as a part of motion information in addition to MVs and reference indices. When merge candidate list is constructed, LIC flag is inherited from the neighbor blocks for merge candidates, but LIC flag is not used for motion vector pruning for simplification purpose.

LIC flag is not stored in the motion vector buffer of the reference picture, so LIC flag is always set equal to false for TMVP. LIC flag is also set equal to false for bi-directional merge candidates, such as par-wise average candidate, and zero motion candidates.

LIC flag is context coded with a single context, when LIC tool is not applicable, LIC flag is not signaled.

When in-loop luma reshaping is used, the inverse reshaping is applied to the neighbor samples of the current CU prior to LIC parameter derivation, since the current CU neighbors are in the reshaped domain, but the reference picture samples are in the original (non-reshaped) domain.

To derive LIC linear model parameters, linear least square method is utilized, it requires the following operations:

- Multiplications: 2 * min(width, height) + 4

- Additions: 4 * min(width, height) + 4

- Shifts: 12

To apply linear model 1 multiplication and 1 addition are used per sample, which can be done at the reconstruction stage when prediction is added to the residual.

# Test results

The proposed method was implemented on top of VTM-4.0 software and tested under common test conditions described in JVET-M1010.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.65% | 0.31% | 0.31% | 117% | 98% |
| Class A2 | -0.32% | -0.40% | -0.40% | 110% | 100% |
| Class B | -0.45% | -0.25% | -0.15% | 112% | 101% |
| Class C | -0.33% | -0.27% | -0.35% | 116% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.17% | -0.16% | 113% | 100% |
| Class D | -0.14% | -0.05% | -0.22% | 114% | 102% |
| Class F | -0.88% | -0.49% | -0.55% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.59% | -0.25% | -0.46% | 112% | 103% |
| Class C | -0.52% | 0.16% | -0.43% | 117% | 99% |
| Class E | -0.01% | 0.03% | -0.08% | 101% | 100% |
| Overall | -0.42% | -0.04% | -0.36% | 111% | 101% |
| Class D | -0.17% | -0.36% | -0.74% | 115% | 99% |
| Class F | -2.69% | -1.74% | -1.87% | 111% | 98% |

Test 1. Proposed LIC under constraint a. (Test CE1-5.1a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.18% | -0.37% | 120% | 98% |
| Class A2 | -0.24% | -0.28% | -0.30% | 109% | 98% |
| Class B | -0.33% | -0.22% | -0.16% | 114% | 101% |
| Class C | -0.29% | -0.43% | -0.41% | 117% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.28% | -0.30% | 115% | 100% |
| Class D | -0.15% | -0.37% | -0.20% | 115% | 102% |
| Class F | -0.71% | -0.47% | -0.51% | 110% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | -0.29% | -0.37% | 115% | 101% |
| Class C | -0.47% | -0.09% | -0.34% | 120% | 100% |
| Class E | 0.04% | 0.17% | -0.11% | 103% | 100% |
| Overall | -0.36% | -0.11% | -0.30% | 114% | 101% |
| Class D | -0.16% | -0.24% | -0.80% | 117% | 103% |
| Class F | -2.27% | -2.27% | -1.86% | 114% | 98% |

Test 2. Proposed LIC under constraint b. (Test CE1-5.1b).

The gain of the LIC method is much higher for the sequences that have illumination change. It was additionally tested on the following sequences having this property.

|  | Random access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences |  | Y | U | V |  | Y | U | V |  |
| Timelapse (1920x1080) |  | -4.05% | -4.06% | -1.51% |  | -7.29% | -4.67% | -2.16% |  |
| Crew (1280x720) |  | -3.38% | -0.06% | -1.69% |  |  |  |  |  |

# Patent rights declaration(s)e

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).