| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0347-v3 |
| --- | --- |

| Title: | CE7-related: Joint coding of chroma residuals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh K. Ramasubramonian Muhammed Coban Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution builds upon the joint chroma residual coding method that is tested in CE7-1. It is claimed that the CE7-1 method may result in chroma artifacts due to the approximation of the Cb and Cr residuals, because the approximation error is not further coded and, therefore, the lost chroma information can not be recovered. The proposed method processes the Cb and Cr residuals in an invertible manner, which is only lossy due to rounding, and is similar to jointly applying a 1D 2-point transform to the residuals. The Cb and Cr residuals are processed in the rescaled chroma domain before transforms, quantization, entropy coding. At the decoder-side, in case the signalled joint chroma residual coding mode is enabled for the transform block, the reconstruction of a Cb transform block consists of adding the two decoded Cb and Cr residuals to the predicted Cb block, while the reconstruction of a Cr transform block consists of subtracting the decoded Cb residual and adding the decoded Cr residual to the predicted Cr block. While the CE7-1 method signals a specific Qp offset value for the joint mode, which may be an encoder burden to determine this value, the proposed method does not require a specific Qp offset value.

The average BD-rates and runtimes are as follows:

- Qp values {22, 27, 32, 37}:

  - AI: -0.29% (Y), 0.20% (U), -0.01% (V) with runtimes 104% (EncT) and 102% (DecT)

  - RA: -0.21% (Y), 0.15% (U), 0.39% (V) with runtimes 102% (EncT) and 100% (DecT)

  - LDB: -0.14% (Y), 1.10% (U), -0.14% (V) with runtimes 102% (EncT) and 102% (DecT)

- Low-Qp values {2, 7, 12, 17}, for 100 frames:

  - AI: -0.11% (Y), 0.14% (U), 0.07% (V) with runtimes x% (EncT) and x% (DecT)

  - RA: -0.11% (Y), 0.27% (U), 0.30% (V) with runtimes x% (EncT) and x% (DecT)

  - LDB: -0.12% (Y), 0.24% (U), 0.18% (V) with runtimes x% (EncT) and x% (DecT)

The average BD-rates and runtimes are as follows with lambda tuning included that is identical to CE7-1:

- Qp values {22, 27, 32, 37}:

  - AI: -0.46% (Y), 0.69% (U), 0.44% (V) with runtimes x% (EncT) and x% (DecT)

  - RA: -0.33% (Y), 0.48% (U), 0.67% (V) with runtimes x% (EncT) and x% (DecT)

  - LDB: -0.16% (Y), 0.44% (U), -1.18% (V) with runtimes x% (EncT) and x% (DecT)

- Low-Qp values {2, 7, 12, 17}, for 100 frames:

  - AI: -0.28% (Y), 0.56% (U), 0.48% (V) with runtimes x% (EncT) and x% (DecT)

  - RA: -0.18% (Y), 0.40% (U), 0.44% (V) with runtimes x% (EncT) and x% (DecT)

  - LDB: -0.16% (Y), 0.28% (U), 0.07% (V) with runtimes x% (EncT) and x% (DecT)

It is reported that the highest coding gains are observed for sequence classes A and F.

Version history:

- v2: results updated

- v3: results updated

# Proposed joint chroma residual coding method

Table 1 compares aspects from the proposed method for joint coding of chroma residuals with the method proposed in the CE7-1 test (JVET-N0054).

Main benefits of the proposed method are:

- Invertibility property avoids chroma artifacts due to accidental use of the method.

- Avoids determining and signalling a mode-specific Qp offset value, which may be an encoder burden.

- Interesting coding efficiency benefits with low runtime impact (see experimental results).

Table 1: Comparison of aspects from the proposed method with the CE7-1 method

|  | Test CE7-1 | Current proposal |
| --- | --- | --- |
| Cb residual encoder processing (with resCb = origCb – predCb) | resJointCb = (resCb – resCr) / 2 | resJointCb = (resCb – resCr) / 2 |
| Cr residual encoder processing (with resCr = origCr – predCr) | 0 | resJointCr = (resCb + resCr) / 2 |
| Cb reconstruction (recCb) | recCb = predCb + resJointCb | recCb = predCb + resJointCb + resJointCr |
| Cr reconstruction (recCr) | recCr = predCr – resJointCb | recCr = predCr – resJointCb + resJointCr |
| Processing invertible? | No | Yes (lossy due to rounding only) |
| Qp offsets for resJoint coding | -1 | 0 |
| Lambda tuning | - Inter: IF (jointCbCr) THEN (0.6 × lambda) - Intra: IF (jointCbCr) THEN (0.6 × lambda) ELSE IF (sliceQP > 18) THEN (1.10 × lambda) ELSE (1.05 × lambda) | No (experimental results are provided with and without lambda tuning from CE7-1 for comparison) |
| Signalling of tu_cb_cr_joint_residual flag | residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {. if( cIdx == 2 && tu_cbf_cb[ x0 ][ y0 ] ) {. tu_cb_cr_joint_residual[ x0 ][ y0 ]. if( tu_cb_cr_joint_residual [ x0 ][ y0 ] ) {. return. }. }. …. | residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {. if( cIdx == 1 && tu_cbf_cb[ x0 ][ y0 ] ) {. tu_cb_cr_joint_residual[ x0 ][ y0 ]. }. …. |
| Qp offset signalling for joint residual mode | Yes | No |
| Chroma artifact examples (experiment forces joint residual mode selection; example: RaceHorses, AI) | QP=22 QP=22 | QP=22 QP=22 |

# Experimental results

The proposed technique has been implemented on top of VTM4.0. The experimental results under the AI, RA and LDB configurations are summarized in Table 4 for Qp values {22, 27, 32, 37} and in Table 5 for low-Qp values {2, 7, 12, 17}. The results in both tables are obtained without encoder lambda tuning.

The results obtained with lambda tuning from CE7-1, are reported in Table 6 for Qp values {22, 27, 32, 37} and in Table 7 for low-Qp values {2, 7, 12, 17}. Note that the tested lambda tuning is identical to that from CE7-1 and may not be optimal for the proposed method in this contribution. Additional results obtained with the CE7-1 method, excluding Qp/lambda tuning, are also provided in Table 8 for Qp values {22, 27, 32, 37} and in Table 9 for low-Qp values {2, 7, 12, 17}.

Table 2 and Table 3 summarize all results and include CE7-1 results (JVET-N0054) for comparison.

From these results it can be observed that the proposed method has interesting coding performance compared with CE7-1.

Table 2: Summary table comparing proposed method with CE7-1 method for Qp values {22, 27, 32, 37}.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Proposed method without CE7-1 lambda tuning | AI | -0.29% | 0.20% | -0.01% | 104% | 102% |
|  | RA | -0.21% | 0.15% | 0.39% | 102% | 100% |
|  | LDB | -0.14% | 1.10% | -0.14% | 102% | 102% |
| CE7-1 results without CE7-1 Qp/lambda tuning | AI | -0.30% | 0.16% | -1.02% |  |  |
|  | RA | -0.22% | 0.42% | -0.16% |  |  |
|  | LDB | -0.11% | -0.92% | -2.07% |  |  |
| Proposed method with CE7-1 lambda tuning | AI | -0.46% | 0.69% | 0.44% |  |  |
|  | RA | -0.33% | 0.48% | 0.67% |  |  |
|  | LDB | -0.16% | 0.44% | -1.18% |  |  |
| CE7-1 results with CE7-1 Qp/lambda tuning | AI | -0.38% | -0.49% | -1.52% | 103% | 100% |
|  | RA | -0.26% | -0.81% | -0.92% | 101% | 100% |
|  | LDB | -0.07% | -2.99% | -4.76% | 100% | 100% |

Table 3: Summary table comparing proposed method with CE7-1 method for low-Qp values {2, 7, 12, 17}.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Proposed method without CE7-1 lambda tuning | AI | -0.11% | 0.14% | 0.07% |  |  |
|  | RA | -0.11% | 0.27% | 0.30% |  |  |
|  | LDB | -0.12% | 0.24% | 0.18% |  |  |
| CE7-1 results without CE7-1 Qp/lambda tuning | AI | -0.13% | 0.21% | -0.02% |  |  |
|  | RA |  |  |  |  |  |
|  | LDB | -0.13% | 0.17% | -0.06% |  |  |
| Proposed method with CE7-1 lambda tuning | AI | -0.28% | 0.56% | 0.48% |  |  |
|  | RA | -0.18% | 0.40% | 0.44% |  |  |
|  | LDB | -0.16% | 0.28% | 0.07% |  |  |
| CE7-1 results with CE7-1 Qp/lambda tuning | AI | -0.28% | 0.57% | 0.27% | 108% | 99% |
|  | RA | -0.19% | 0.24% | -0.10% | 102% | 100% |
|  | LDB | -0.18% | 0.09% | -0.58% | 103% | 100% |

Table 4: Results obtained with proposed method for Qp values {22, 27, 32, 37} without encoder lambda tuning.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.02% | 0.46% | 104% | 102% |
| Class A2 | -0.51% | -0.09% | 0.93% | 107% | 97% |
| Class B | -0.18% | 0.08% | -0.15% | 103% | 103% |
| Class C | -0.33% | -0.01% | -0.65% | 105% | 104% |
| Class E | -0.35% | 1.21% | -0.34% | 103% | 103% |
| Overall | -0.29% | 0.20% | -0.01% | 104% | 102% |
| Class D | -0.31% | 0.17% | -0.67% | 105% | 100% |
| Class F | -0.47% | -0.74% | -1.41% | 104% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.05% | 1.69% | 104% | 101% |
| Class A2 | -0.30% | -0.54% | 0.82% | 102% | 99% |
| Class B | -0.14% | 0.21% | 0.09% | 101% | 100% |
| Class C | -0.24% | 0.76% | -0.54% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | 0.15% | 0.39% | 102% | 100% |
| Class D | -0.17% | 0.60% | -0.90% | 102% | 102% |
| Class F | -0.66% | -0.17% | -1.29% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | 0.27% | 0.02% | 102% | 105% |
| Class C | -0.19% | 0.63% | -1.18% | 102% | 100% |
| Class E | -0.05% | 3.11% | 0.98% | 100% | 99% |
| Overall | -0.14% | 1.10% | -0.14% | 102% | 102% |
| Class D | -0.14% | 1.82% | -1.09% | 102% | 95% |
| Class F | -0.54% | -0.27% | -1.86% | 103% | 99% |

Table 5: Results obtained with proposed method for Qp values {2, 7, 12, 17} without encoder lambda tuning.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.03% |  |  |
| Class A2 | -0.04% | 0.02% | 0.02% |  |  |
| Class B | -0.05% | 0.05% | 0.05% |  |  |
| Class C | -0.33% | 0.34% | 0.15% |  |  |
| Class E | -0.10% | 0.29% | 0.10% |  |  |
| Overall | -0.11% | 0.14% | 0.07% |  |  |
| Class D | -0.58% | 0.65% | 0.34% |  |  |
| Class F | -1.05% | 0.48% | 0.37% |  |  |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.06% | 0.21% |  |  |
| Class A2 | -0.04% | 0.04% | 0.06% |  |  |
| Class B | -0.08% | 0.27% | 0.32% |  |  |
| Class C | -0.28% | 0.58% | 0.52% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.27% | 0.30% |  |  |
| Class D | -0.42% | 0.68% | 0.26% |  |  |
| Class F | -0.87% | 0.77% | 0.66% |  |  |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.13% | 0.18% |  |  |
| Class C | -0.27% | 0.42% | 0.25% |  |  |
| Class E | -0.01% | 0.21% | 0.07% |  |  |
| Overall | -0.12% | 0.24% | 0.18% |  |  |
| Class D | -0.53% | 0.67% | 0.41% |  |  |
| Class F | -0.55% | 0.70% | 0.38% |  |  |

Table 6: Results obtained with proposed method for Qp values {22, 27, 32, 37} with CE7-1 lambda tuning.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | 1.15% | 1.49% |  |  |
| Class A2 | -0.94% | 0.40% | 1.82% |  |  |
| Class B | -0.29% | 0.75% | 0.32% |  |  |
| Class C | -0.41% | 0.07% | -0.53% |  |  |
| Class E | -0.40% | 1.26% | -0.48% |  |  |
| Overall | -0.46% | 0.69% | 0.44% |  |  |
| Class D | -0.36% | 0.24% | -0.57% |  |  |
| Class F | -0.47% | -1.04% | -1.75% |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | 0.45% | 2.03% |  |  |
| Class A2 | -0.50% | -0.68% | 1.27% |  |  |
| Class B | -0.20% | 0.81% | 0.22% |  |  |
| Class C | -0.25% | 0.98% | -0.25% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.33% | 0.48% | 0.67% |  |  |
| Class D | -0.20% | 1.02% | -0.76% |  |  |
| Class F | -0.57% | -0.65% | -1.89% |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.07% | -1.17% |  |  |
| Class C | -0.16% | 0.10% | -2.24% |  |  |
| Class E | -0.19% | 1.72% | 0.20% |  |  |
| Overall | -0.16% | 0.44% | -1.18% |  |  |
| Class D | -0.16% | 0.91% | -2.10% |  |  |
| Class F | -0.43% | -1.65% | -3.22% |  |  |

Table 7: Results obtained with proposed method for Qp values {2, 7, 12, 17} with CE7-1 lambda tuning.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.74% | 0.71% |  |  |
| Class A2 | -0.27% | 0.37% | 0.39% |  |  |
| Class B | -0.24% | 0.48% | 0.48% |  |  |
| Class C | -0.43% | 0.49% | 0.24% |  |  |
| Class E | -0.28% | 0.82% | 0.65% |  |  |
| Overall | -0.28% | 0.56% | 0.48% |  |  |
| Class D | -0.65% | 0.67% | 0.35% |  |  |
| Class F | -1.12% | 0.58% | 0.32% |  |  |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.53% | 0.68% |  |  |
| Class A2 | -0.15% | 0.22% | 0.24% |  |  |
| Class B | -0.15% | 0.42% | 0.46% |  |  |
| Class C | -0.31% | 0.42% | 0.39% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.18% | 0.40% | 0.44% |  |  |
| Class D | -0.44% | 0.57% | 0.22% |  |  |
| Class F | -0.89% | 0.66% | 0.49% |  |  |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.24% | 0.24% |  |  |
| Class C | -0.30% | 0.29% | -0.09% |  |  |
| Class E | -0.05% | 0.32% | 0.02% |  |  |
| Overall | -0.16% | 0.28% | 0.07% |  |  |
| Class D | -0.53% | 0.42% | 0.12% |  |  |
| Class F | -0.63% | 0.49% | 0.13% |  |  |

Table 8: Results obtained with CE7-1 method for Qp values {22, 27, 32, 37} without CE7-1 Qp/lambda tuning.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.06% | 0.36% |  |  |
| Class A2 | -0.51% | -0.28% | 0.47% |  |  |
| Class B | -0.22% | 0.20% | -1.35% |  |  |
| Class C | -0.32% | -0.17% | -2.11% |  |  |
| Class E | -0.33% | 1.18% | -1.92% |  |  |
| Overall | -0.30% | 0.16% | -1.02% |  |  |
| Class D | -0.30% | -0.25% | -2.07% |  |  |
| Class F | -0.43% | -0.86% | -2.37% |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.11% | 1.80% |  |  |
| Class A2 | -0.27% | -0.61% | 0.51% |  |  |
| Class B | -0.17% | 0.61% | -0.87% |  |  |
| Class C | -0.26% | 1.18% | -1.26% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.22% | 0.42% | -0.16% |  |  |
| Class D | -0.15% | 0.84% | -1.30% |  |  |
| Class F | -0.57% | -0.18% | -1.85% |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -1.56% | -1.83% |  |  |
| Class C | -0.15% | -1.05% | -2.37% |  |  |
| Class E | 0.00% | 0.33% | -2.05% |  |  |
| Overall | -0.11% | -0.92% | -2.07% |  |  |
| Class D | -0.06% | -1.85% | -2.65% |  |  |
| Class F | -0.41% | -1.71% | -1.78% |  |  |

Table 9: Results obtained with CE7-1 method for Qp values {2, 7, 12, 17} without CE7-1 Qp/lambda tuning.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.07% | 0.07% |  |  |
| Class A2 | -0.03% | 0.02% | 0.01% |  |  |
| Class B | -0.07% | 0.12% | 0.02% |  |  |
| Class C | -0.36% | 0.48% | -0.17% |  |  |
| Class E | -0.10% | 0.36% | 0.02% |  |  |
| Overall | -0.13% | 0.21% | -0.02% |  |  |
| Class D | -0.50% | 0.56% | -0.08% |  |  |
| Class F | -0.82% | 0.50% | -0.18% |  |  |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.10% | 0.29% | 0.19% |  |  |
| Class C | -0.29% | 0.44% | -0.02% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.36% | 0.21% | -0.23% |  |  |
| Class F | -0.65% | 0.44% | -0.01% |  |  |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.11% | 0.07% |  |  |
| Class C | -0.29% | 0.28% | -0.13% |  |  |
| Class E | -0.01% | 0.13% | -0.16% |  |  |
| Overall | -0.13% | 0.17% | -0.06% |  |  |
| Class D | -0.45% | 0.07% | -0.19% |  |  |
| Class F | -0.59% | 0.42% | -0.02% |  |  |

# Conclusion

It is recommended to adopt the proposed combined chroma residual coding method into VVC.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).