| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0349 |
| --- | --- |

| Title: | AHG17: On Parameter Set Design for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to keep following the design principle of using the same picture parameter set for all NAL units of a picture. It also proposes to adopt the decoder parameter set (DPS) that was proposed in JVET-M0101 and discussed in the BoG on high level syntax, but was accidentally missed in track review.

# Introduction

Parameter set design in the current VVC specification resembles the design in former video coding standards in two aspects: for a single-layer coded video sequence, there is

- one parameter set with sequence-level scope (SPS),

- one or more parameter sets with picture-level scope (PPS).

Although the picture parameter set has the scope of at least one picture, it is usually shared between multiple pictures. This is especially desirable because the PPS is a larger structure with many parameters, which is costly in terms of bit rate.

JVET-M0101 proposed a new parameter set, the Decoder Parameter Set (DPS) which is supposed to carry signalling related to a concatenation of CVSs or in other words on a bitstream-level. However, track review of the related HLS BoG report mistakenly excluded the recommendation to adopt the DPS and no track level conclusion was reached.

# Proposal

## Scope of picture parameter set

The authors propose to limit the minimum scope of the PPS to a picture. Sub-pictures (tile groups) should not use different picture parameter sets.

The absence of such a restriction incurs undesirable overhead, e.g. when used in context with the configurations investigated in CE12 (up to 96 tiles). Further, a value space problem arises for the identifiers of the PPSs (pps_pic_parameter_set_id).

## Decoder Parameter Set (DPS)

The authors propose to adopt the decoder parameter set based on JVET-M0101 as recommended for adoption by the HLS BoG during the previous meeting:

At this point of time, the syntax structure is proposed to contain the maximum number of sub-layers (as in HEVC) and a profile_level( ) syntax structure, which indicates a profile and a level of a decoder that is capable of decoding the bitstream and includes the constraints that apply to the bitstream.

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_reserved_three_2bits | u(2) |
| dps_max_sub_layers_minus1 | u(3) |
| dps_reserved_zero_7bits | u(7) |
| profile_level( dps_max_sub_layers_minus1 ) |  |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).