| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0350 |
| --- | --- |

| Title: | AHG17: HRD starting point |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution provides a stripped down basic Hypothetical Reference Decoder (HRD) design as starting point for VVC. It is based on the HRD model of HEVC, with the following simplifications:

- No sub-picture timing

- No alternative timing for removal of RASL pictures

- Bug-fix for splicing

Decoded picture buffer conformance text is not included, because the referenced syntax elements either do not exist in the VVC specification (e.g. no_output_of_prior_pics_flag) yet or are replaced by different techniques (e.g. RPS).

# Introduction

An HRD model is required to verify bitstream conformance. We propose to use a simplified model based on the HEVC text. This includes:

- HRD text (Annex C)

- Basic SEI syntax

- Buffering period and picture timing and filler payload SEI messages

- SPS VUI and HRD parameters syntax structures

- Filler Data NAL unit

Decoded picture buffer text is not included in this contribution.

## Sub-picture Timing

Although sub-picture timing is considered a necessary feature for ultra-low delay applications, it is considered that starting with a simplified HRD model is useful and once the foundation is settled, the work on sub-picture timing can start.

## Alternative Timing for removal of RASL pictures

The current VVC text specifies RASL and RADL pictures. Therefore, alternative timings as specified in HEVC should be also specified for VVC. However, it is asserted that there are further cases where AUs are dropped in a bitstream for which defining an HRD model and conformance would be beneficial. For instance, random access using DRAP pictures  could be added as normative decoder behavior.

It is asserted that a single solution that applies to both cases would be beneficial, but requires further study, which is highly encouraged. Therefore, the current proposed text focuses only on a simplified approach.

## Bug-fix for splicing

The HEVC specification determines, when concatenationFlag is set to 1, that the AuNominalRemovalTime for the splicing point under some circumstances (non-seamless splicing) may be higher than 
AuNominalRemovalTime[ prevNonDiscardablePic ]+ ( auCpbRemovalDelayDeltaMinus1 + 1 ).

The HEVC text specifies:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay =
		Max( ( auCpbRemovalDelayDeltaMinus1 + 1 ),	(C-10)
			Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick ) )
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

In the following Figure 1, it is shown a splicing operation that does not allow for seamless splicing due to the fact that the InitCpbRemovalDelay[ SchedSelIdx ] imposes a later removal time than that resulting in equi-distant CPB removal times.

Figure 1: Splicing example.

If instead of using the AU n-1 (AU 5) for derivation of the removal time of n=6, as indicated above in the figure, we use the non discardable picture (AU 4) it would result in the following:

taf6= trm4 + ∆ >= trm6 (underflow=non-seamless splicing)

trm6= taf5+InitCpbRemovalDelay

∆ =taf6 – trm4= Size/Bitrate – (trm4 – taf5)

trm4+Size/Bitrate – (trm4 – taf5)>= taf5+InitCpbRemovalDelay

Size/Bitrate – (trm4 – taf5)>= (taf5 – trm4) +InitCpbRemovalDelay

taf5+ Size/Bitrate = taf6 >= trm4 + (taf5 – trm4) +InitCpbRemovalDelay

As seen in the derived formula the part marked in yellow in C-10 above is not correct. A fix could be as follows:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 = 
		Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
			AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick ) 
									(C-10)
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 < 
		baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

# Proposal

HRD parameters and related VUI are proposed in JVET-N0353. We propose to adopt the syntax specified in this document.

The following sections contain the syntax tables for the additional NAL unit types and SEI messages.

## FD_NUT syntax

| filler_data_rbsp( ) { | Descriptor |
| --- | --- |
| while( next_bits( 8 ) = = 0xFF ) |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| rbsp_trailing_bits( ) |  |
| } |  |

The filler data RBSP contains bytes whose value shall be equal to 0xFF. No normative decoding process is specified for a filler data RBSP.

## SEI message syntax

| sei_rbsp( ) { | Descriptor |
| --- | --- |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadType += 255 |  |
| } |  |
| last_payload_type_byte | u(8) |
| payloadType += last_payload_type_byte |  |
| payloadSize = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadSize += 255 |  |
| } |  |
| last_payload_size_byte | u(8) |
| payloadSize += last_payload_size_byte |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Modified buffering period SEI and picture timing SEI messages are proposed in JVET-N0353. We propose to adopt the syntax in this proposal instead of the original HEVC syntax.

| filler_payload( payloadSize ) { | Descriptor |
| --- | --- |
| for( k = 0; k < payloadSize; k++) |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| } |  |

SEI payload semantics

General SEI payload semantics

reserved_payload_extension_data shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of reserved_payload_extension_data. When present, the length, in bits, of reserved_payload_extension_data is equal to 8 * payloadSize − nEarlierBits − nPayloadZeroBits − 1, where nEarlierBits is the number of bits in the sei_payload( ) syntax structure that precede the reserved_payload_extension_data syntax element and nPayloadZeroBits is the number of payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure.

payload_bit_equal_to_one shall be equal to 1.

payload_bit_equal_to_zero shall be equal to 0.

NOTE 1 – SEI messages with the same value of payloadType are conceptually the same SEI message regardless of whether they are contained in prefix or suffix SEI NAL units.

NOTE 2 – For SEI messages with payloadType in the range of 0 to 47, inclusive, that are specified in this Specification, the payloadType values are aligned with similar SEI messages specified in Rec. ITU-T H.264 | ISO/IEC 14496-10.

The semantics and persistence scope for each SEI message are specified in the semantics specification for each particular SEI message.

NOTE 4 – Persistence information for SEI messages is informatively summarized in Table XXX.1.

| Table XXX.1 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| Buffering period | The remainder of the bitstream |
| Picture timing | The access unit containing the SEI message |
| Filler payload | The access unit containing the SEI message |

In the following subclauses of this annex, the following applies:

–	The current SEI message refers to the particular SEI message.

–	The current access unit refers to the access unit containing the current SEI message.

Filler payload SEI message semantics

This SEI message contains a series of payloadSize bytes of value 0xFF, which can be discarded.

ff_byte shall be a byte having the value 0xFF.

## HRD related Baseline Text

The following HRD text contains change marks relative to HEVC.

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard.)

General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams or bitstream subsets are subject to HRD conformance checking for this Specification. The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all access units in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B).

NOTE 1 – Decoders conforming to profiles specified in Annex A do not use NAL units with nuh_layer_id greater than 0 (e.g., access unit delimiter NAL units with nuh_layer_id greater than 0) for access unit boundary detection, except for identification of whether a NAL unit is a VCL or non-VCL NAL unit. Consequently, hypothetical reference decoder (HRD) parameters carried in non-scalable-nested buffering period, picture timing and decoding unit information SEI messages apply to access units that are identified based on such access unit boundary detection.

Figure C.1 shows the types of bitstream conformance points checked by the HRD.

Figure C.1 – Structure of byte streams and NAL unit streams for HRD conformance checks

[NOTE: Figure to be updated]

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic clauses of clause 7, Annexes D and E.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through the hrd_parameters( ) syntax structure, which may be part of the SPS syntax structure or the VPS syntax structure.

Two sets of bitstream conformance tests are needed for checking the conformance of a bitstream, which is referred to as the entire bitstream, denoted as entireBitstream. The first set of bitstream conformance tests are for testing the conformance of the entire bitstream and its temporal subsets, regardless of whether there is a layer set specified by the active VPS that contains all the nuh_layer_id values of VCL NAL units present in the entire bitstream. The second set of bitstream conformance tests are for testing the conformance of the layer sets specified by the active VPS and their temporal subsets. For all these tests, only the base layer pictures (i.e., pictures with nuh_layer_id equal to 0) are decoded and other pictures are ignored by the decoder when the decoding process is invoked.

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this clause:

- An operation point under test, denoted as TargetOp, is selected by selecting a layer identifier list OpLayerIdList and a target highest TemporalId value OpTid. The layer identifier list OpLayerIdList of TargetOp either consists of all the nuh_layer_id values of the VCL NAL units present in entireBitstream or consists of all the nuh_layer_id values of a layer set specified by the active VPS. The value of OpTid is in the range of 0 to sps_max_sub_layers_minus1, inclusive. The values of OpLayerIdList and OpTid areis  such that the sub-bitstream BitstreamToDecode that is the output by invoking the sub-bitstream extraction process as specified in clause 10 with entireBitstream and OpTid and OpLayerIdList as inputs satisfy both of the following conditions:

–	There is at least one VCL NAL unit in BitstreamToDecode with nuh_layer_id equal to each of the nuh_layer_id values in OpLayerIdList.

–	There is at least one VCL NAL unit with TemporalId equal to OpTid in BitstreamToDecode.

- TargetDecLayerIdList is set equal to OpLayerIdList of TargetOp and HighestTid is set equal to OpTid of TargetOp.

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected. If TargetDecLayerIdList contains all nuh_layer_id values present in entireBitstream, tThe hrd_parameters( ) syntax structure in the active SPS (or provided through an external means not specified in this Specification) is selected. Otherwise, the hrd_parameters( ) syntax structure in the active VPS (or provided through some external means not specified in this Specification) that applies to TargetOp is selected. Within the selected hrd_parameters( ) syntax structure, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) is selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- An access unit associated with a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as access unit 0. If TargetDecLayerIdList contains all nuh_layer_id values present in entireBitstream, the assocciated buffering period SEI message shall be either a non-scalable-nested SEI message or provided by external means. Otherwise, the assocciated buffering period SEI message shall be either a scalable-nested SEI message or provided by external means.

- When sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the access unit level (in which case the variable SubPicHrdFlag is set equal to 0) or at the sub-picture level (in which case the variable SubPicHrdFlag is set equal to 1). Otherwise, SubPicHrdFlag is set equal to 0 and the CPB is scheduled to operate at the partition unit level.

- For each access unit in BitstreamToDecode starting from access unit 0, the buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and the picture timing SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and when SubPicHrdFlag is equal to 1 and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, the decoding unit information SEI messages (present in BitstreamToDecode or available through external means not specified in this Specification) that are associated with decoding units in the access unit and apply to TargetOp are selected. If TargetDecLayerIdList contains all nuh_layer_id values present in entireBitstream, tThe selected buffering period, and picture timing and decoding unit information SEI messages shall be either non-scalable-nested SEI messages or provided by external means. Otherwise, the selected buffering period, picture timing and decoding unit information SEI messages shall be either scalable-nested SEI messages or provided by external means.

- A value of SchedSelIdx is selected. The selected SchedSelIdx shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where cpb_cnt_minus1[ HighestTid ] is found in the hrd_parameters( ) syntax structure as selected above.

- When the coded picture in access unit 0 has nal_unit_type equal to CRA_NUT or BLA_W_LP and irap_cpb_params_present_flag in the selected buffering period SEI message is equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and the RASL access units associated with access unit 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 *  n2 ( n2 * 2 + n3 ) * n4, where the values of n0, n1, and n2, n3 and n4 are specified as follows:

–	n0 is derived as follows:

–	If BitstreamToDecode is a Type I bitstream, n0 is equal to 1.

–	Otherwise (BitstreamToDecode is a Type II bitstream), n0 is equal to 2.

–	n1 is equal to cpb_cnt_minus1[ HighestTid ] + 1.

–	n2 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which both of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT or BLA_W_LP for the VCL NAL units.

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 1.

–	n23 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which one or both of the following conditions are true:

–	nal_unit_type is equal to neither CRA_NUT nor BLA_W_LP for the VCL NAL units.

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 0.

–	n4 is derived as follows:

–	If sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 0, n4 is equal to 1.

–	Otherwise, n4 is equal to 2.

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure C.1, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( nal_hrd_parameters_present_flag )" is selected), the test is conducted at the Type II conformance point shown in Figure C.1, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 2 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C.1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C.1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] for the variable bit rate (VBR) case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a CVS conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2 through 10, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item f) of clause A.4.2 but also fall within the bounds set for VCL HRD parameters for profile conformance in item e) of clause A.4.2, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item e) of clause A.4.2.

All VPSs, SPSs and PPSs referred to in the VCL NAL units and the corresponding buffering period, and picture timing and decoding unit information SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification.

In Annexes C, D and E, the specification for "presence" of non-VCL NAL units that contain VPSs, SPSs, PPSs, buffering period SEI messages, or picture timing SEI messages or decoding unit information SEI messages is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of such a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of such a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax as specified in this Specification.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this clause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data are supplied by some other means not specified in this Specification.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C.2.

Figure C.2 – HRD buffer model

[NOTE: Figure to be updated]

For each bitstream conformance test, the CPB size (number of bits) is CpbSize[ SchedSelIdx ] as specified in clause E.3.3, where SchedSelIdx and the HRD parameters are specified above in this clause. The DPB size (number of picture storage buffers) is sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1.

If SubPicHrdFlag is equal to 0, the HRD operates at access unit level and each decoding unit is an access unit. Otherwise the HRD operates at sub-picture level and each decoding unit is a subset of an access unit.

NOTE 5 – If the HRD operates at access unit level, each time when some bits are removed from the CPB, a decoding unit that is an entire access unit is removed from the CPB. Otherwise (the HRD operates at sub-picture level), each time when some bits are removed from the CPB, a decoding unit that is a subset of an access unit is removed from the CPB. Regardless of whether the HRD operates at access unt level or sub-picture level, each time when some picture is output from the DPB, an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

The following is specified for expressing the constraints in this annex:

–	Each access unit is referred to as access unit n, where the number n identifies the particular access unit. Access unit 0 is selected per step 4 above. The value of n is incremented by 1 for each subsequent access unit in decoding order.

–	Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

NOTE 6 – The numbering of decoding units is relative to the first decoding unit in access unit 0.

–	Picture n refers to the coded picture or the decoded picture of access unit n.

The HRD operates as follows:

–	The HRD is initialized at decodingaccess unit 0, with both the CPB and the DPB being set to be empty (the DPB fullness is set equal to 0).

NOTE 7 – After initialization, the HRD is not initialized again by subsequent buffering period SEI messages.

–	Data associated with decoding units that flow into the CPB according to a specified arrival schedule are delivered by the hypothetical stream scheduler (HSS).

–	The data associated with each decodingaccess unit are removed and decoded instantaneously by the instantaneous decoding process at the CPB removal time of the decodingaccess unit.

–	Each decoded picture is placed in the DPB.

–	A decoded picture is removed from the DPB when it becomes no longer needed for inter prediction reference and no longer needed for output.

For each bitstream conformance test, the operation of the CPB is specified in clause C.2, the instantaneous decoder operation is specified in clauses 2 through 10, the operation of the DPB is specified in clause C.3 and the output cropping is specified in clauses C.3.3 and C.5.2.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in clauses E.2.2 and E.3.2. The HRD is initialized as specified by the buffering period SEI message specified in clauses D.2.2 and D.3.2. The removal timing of decodingaccess units from the CPB and output timing of decoded pictures from the DPB is specified using information in picture timing SEI messages (specified in clauses D.2.3 and D.3.3) or in decoding unit information SEI messages (specified in clauses D.2.22 and D.3.22). All timing information relating to a specific decodingaccess unit shall arrive prior to the CPB removal time of the decodingaccess unit.

The requirements for bitstream conformance are specified in clause C.4 and the HRD is used to check conformance of bitstreams as specified above in this clause and to check conformance of decoders as specified in clause C.5.

NOTE 8 – While conformance is guaranteed under the assumption that all picture-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is performed with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decodingaccess unit is not necessarily an integer.

The variable ClockTick is derived as follows and is called a clock tick:

ClockTick = vui_num_units_in_tick ÷ vui_time_scale	(C-1)

The variable ClockSubTick is derived as follows and is called a clock sub-tick:

ClockSubTick = ClockTick ÷ ( tick_divisor_minus2 + 2 )	(C-2)

  - Operation of coded picture buffer

    - General

The specifications in this clause apply independently to each set of coded picture buffer (CPB) parameters that is present and to both the Type I and Type II conformance points shown in Figure C.1 and the set of CPB parameters is selected as specified in clause C.1.

    - Timing of decoding unit arrival

If SubPicHrdFlag is equal to 0, the variable subPicParamsFlag is set equal to 0 and tThe process specified in the remainder of this clause is invoked with a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n.

Otherwise (SubPicHrdFlag is equal to 1), the process specified in the remainder of this clause is first invoked with the variable subPicParamsFlag set equal to 0 and a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n, and then invoked with subPicParamsFlag set equal to 1 and a decoding unit being considered as a subset of an access unit, for derivation of the initial and final CPB arrival times for the decoding units in access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1 or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1:

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1 and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	The value of subPicParamsFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

The time at which the first bit of decodingaccess unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

The initial arrival time of decodingaccess unit m is derived as follows:

–	If the decodingaccess unit is decodingaccess unit 0 (i.e., when m is equal to 0), initArrivalTime[ 0 ] is set equal to 0.

–	Otherwise (the decodingaccess unit is decodingaccess unit m with m > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decodingaccess unit m is equal to the final arrival time (which is derived below) of decodingaccess unit m − 1, i.e.,

if( !subPicParamsFlag )
	initArrivalTime[ m ] = AuFinalArrivalTime[ m − 1 ]	(C-3)
else
	initArrivalTime[ m ] = DuFinalArrivalTime[ m − 1 ]

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decodingaccess unit m is derived as follows:

if( !subPicParamsFlag )
	initArrivalTime[ m ] = Max( AuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(C-4)
else
	initArrivalTime[ m ] = Max( DuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

if( !subPicParamsFlag )
	tmpNominalRemovalTime = AuNominalRemovalTime[ m ]	(C-5)
else
	tmpNominalRemovalTime = DuNominalRemovalTime[ m ]

where AuNominalRemovalTime[ m ] and DuNominalRemovalTime[ m ] areis the nominal CPB removal time of access unit m and decoding unit m, respectively, as specified in clause C.2.3.

–	If decodingaccess unit m is not the first decodingaccess unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] ) ÷ 90000	(C-6)

–	Otherwise (decodingaccess unit m is the first decodingaccess unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 )	(C-7)

The final arrival time for decodingaccess unit m is derived as follows:

if( !subPicParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷ BitRate[ SchedSelIdx ]	(C-8)
else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷ BitRate[ SchedSelIdx ]

where sizeInbits[ m ] is the size in bits of decodingaccess unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C.1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the selected hrd_parameters( ) syntax structures for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the selected hrd_parameters( ) syntax structures for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the current access unit.

    - Timing of decodingaccess unit removal and decoding of decodingaccess unit

The variables InitCpbRemovalDelay[ SchedSelIdx ], and InitCpbRemovalDelayOffset[ SchedSelIdx ], CpbDelayOffset and DpbDelayOffset are derived as follows:

–	If one or more of the following conditions are true, CpbDelayOffset is set equal to the value of the buffering period SEI message syntax element cpb_delay_offset, DpbDelayOffset is set equal to the value of the buffering period SEI message syntax element dpb_delay_offset, and InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause C.1:

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1 and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause C.1, CpbDelayOffset and DpbDelayOffset are both set equal to 0.

The nominal removal time of the access unit n from the CPB is specified as follows:

–	If access unit n is the access unit with n equal to 0 (the access unit that initializes the HRD), the nominal removal time of the access unit from the CPB is specified by:

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000	(C-9)

–	Otherwise, the following applies:

–	When access unit n is the first access unit of a buffering period that does not initialize the HRD, the following applies:

The nominal removal time of the access unit n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 = 
		Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
			AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick ) 
									(C-10)
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 < 
		baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

where AuNominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first access unit of the previous buffering period, AuNominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL, or RADL or SLNR picture, AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n and concatenationFlag and auCpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and au_cpb_removal_delay_delta_minus1, respectively, in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the values of CpbDelayOffset and DpbDelayOffset are updated as follows:

–	If one or more of the following conditions are true, CpbDelayOffset is set equal to the value of the buffering period SEI message syntax element cpb_delay_offset, and DpbDelayOffset is set equal to the value of the buffering period SEI message syntax element dpb_delay_offset, where the buffering period SEI message containing the syntax elements is selected as specified in clause C.1:

–	Access unit n is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit n is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1 and UseAltCpbParamsFlag for access unit n is equal to 1.

–	Otherwise, CpbDelayOffset and DpbDelayOffset are both set equal to 0.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset )	(C-11)

where AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period and AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n.

When SubPicHrdFlag is equal to 1, the following applies:

–	The variable duCpbRemovalDelayInc is derived as follows:

–	If sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_spt_cpb_removal_delay_increment in the decoding unit information SEI message, selected as specified in clause C.1, associated with decoding unit m.

–	Otherwise, if du_common_cpb_removal_delay_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_cpb_removal_delay_increment_minus1[ i ] + 1 for decoding unit m in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n, where the value of i is 0 for the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in the access unit that contains decoding unit m, 1 for the subsequent num_nalus_in_du_minus1[ 1 ] + 1 NAL units in the same access unit, 2 for the subsequent num_nalus_in_du_minus1[ 2 ] + 1 NAL units in the same access unit, etc.

–	Otherwise, duCpbRemovalDelayInc is set equal to the value of du_common_cpb_removal_delay_increment_minus1 + 1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n.

–	The nominal removal time of decoding unit m from the CPB is specified as follows, where AuNominalRemovalTime[ n ] is the nominal removal time of access unit n:

–	If decoding unit m is the last decoding unit in access unit n, the nominal removal time of decoding unit m DuNominalRemovalTime[ m ] is set equal to AuNominalRemovalTime[ n ].

–	Otherwise (decoding unit m is not the last decoding unit in access unit n), the nominal removal time of decoding unit m DuNominalRemovalTime[ m ] is derived as follows:

if( sub_pic_cpb_params_in_pic_timing_sei_flag )
	DuNominalRemovalTime[ m ] = DuNominalRemovalTime[ m + 1 ] −
		ClockSubTick * duCpbRemovalDelayInc	(C-12)
else
	DuNominalRemovalTime[ m ] = AuNominalRemovalTime[ n ] −
		ClockSubTick * duCpbRemovalDelayInc

If SubPicHrdFlag is equal to 0, tThe removal time of access unit n from the CPB is specified as follows, where AuFinalArrivalTime[ n ] and AuNominalRemovalTime[ n ] are the final CPB arrival time and nominal CPB removal time, respectively, of access unit n:

if( !low_delay_hrd_flag[ HighestTid ]  | |  AuNominalRemovalTime[ n ]  >=  AuFinalArrivalTime[ n ] )
	AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ]
else				(C-13)
	AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ] + ClockTick *
		Ceil( ( AuFinalArrivalTime[ n ] − AuNominalRemovalTime[ n ] ) ÷ ClockTick )

NOTE 1 – When low_delay_hrd_flag[ HighestTid ] is equal to 1 and AuNominalRemovalTime[ n ] is less than AuFinalArrivalTime[ n ], the size of access unit n is so large that it prevents removal at the nominal removal time.

Otherwise (SubPicHrdFlag is equal to 1), the removal time of decoding unit m from the CPB is specified as follows:

if( !low_delay_hrd_flag[ HighestTid ]  | |  DuNominalRemovalTime[ m ]  >=  DuFinalArrivalTime[ m ] )
	DuCpbRemovalTime[ m ] = DuNominalRemovalTime[ m ]
else				(C-14)
	DuCpbRemovalTime[ m ] = DuFinalArrivalTime[ m ]

NOTE 2 – When low_delay_hrd_flag[ HighestTid ] is equal to 1 and DuNominalRemovalTime[ m ] is less than DuFinalArrivalTime[ m ], the size of decoding unit m is so large that it prevents removal at the nominal removal time.

If SubPicHrdFlag is equal to 0, aAt the CPB removal time of access unit n, the access unit is instantaneously decoded.

Otherwise (SubPicHrdFlag is equal to 1), at the CPB removal time of decoding unit m, the decoding unit is instantaneously decoded, and when decoding unit m is the last decoding unit of access unit n, the following applies:

–	Picture n is considered as decoded.

–	The final CPB arrival time of access unit n, i.e., AuFinalArrivalTime[ n ], is set equal to the final CPB arrival time of the last decoding unit in access unit n, i.e., DuFinalArrivalTime[ m ].

–	The nominal CPB removal time of access unit n, i.e., AuNominalRemovalTime[ n ], is set equal to the nominal CPB removal time of the last decoding unit in access unit n, i.e., DuNominalRemovalTime[ m ].

–	The CPB removal time of access unit n, i.e., AuCpbRemovalTime[ m ], is set equal to the CPB removal time of the last decoding unit in access unit n, i.e., DuCpbRemovalTime[ m ].

  - Operation of the decoded picture buffer

TBD

  - Bitstream conformance

TBD

All of the following conditions shall be fulfilled for each of the bitstream conformance tests:

- For each access unit n, with n greater than 0, associated with a buffering period SEI message, let the variable deltaTime90k[ n ] be specified as follows:

deltaTime90k[ n ] = 90000 * ( AuNominalRemovalTime[ n ] − AuFinalArrivalTime[ n − 1 ] )	(C-17)

The value of InitCpbRemovalDelay[ SchedSelIdx ] is constrained as follows:

–	If cbr_flag[ SchedSelIdx ] is equal to 0, the following condition shall be true:

InitCpbRemovalDelay[ SchedSelIdx ]  <=  Ceil( deltaTime90k[ n ] )	(C-18)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1), the following condition shall be true:

Floor( deltaTime90k[ n ] ) <= InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( deltaTime90k[ n ] )	(C-19)

NOTE 1 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialize the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialize the HRD, as the HRD may be initialized at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is greater than the CPB size. The CPB shall never overflow.

- When low_delay_hrd_flag[ HighestTid ] is equal to 0, the CPB shall never underflow. A CPB underflow is specified as follows:

–	If SubHrdFlag is equal to 0, aA CPB underflow is specified as the condition in which the nominal CPB removal time of access unit n AuNominalRemovalTime[ n ] is less than the final CPB arrival time of access unit n AuFinalArrivalTime[ n ] for at least one value of n.

–	Otherwise (SubPicHrdFlag is equal to 1), a CPB underflow is specified as the condition in which the nominal CPB removal time of decoding unit m DuNominalRemovalTime[ m ] is less than the final CPB arrival time of decoding unit m DuFinalArrivalTime[ m ] for at least one value of m.

- When SubPicHrdFlag is equal to 1, low_delay_hrd_flag[ HighestTid ] is equal to 1 and the nominal removal time of a decoding unit m of access unit n is less than the final CPB arrival time of decoding unit m (i.e., DuNominalRemovalTime[ m ] < DuFinalArrivalTime[ m ]), the nominal removal time of access unit n shall be less than the final CPB arrival time of access unit n (i.e., AuNominalRemovalTime[ n ] < AuFinalArrivalTime[ n ]).

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order) shall satisfy the constraints on AuNominalRemovalTime[ n ] and AuCpbRemovalTime[ n ] expressed in clauses TBD.

TBD

  - Decoder conformance

    - General

TBD

    - Operation of the output order DPB

TBD

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).