| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0351 |
| --- | --- |

| Title: | AHG17: HRD parameters for temporal scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution proposes to indicate CPB removal times for operation points using a different number of temporal sub-layers within the buffering period SEI message or picture timing SEI message. Since removal times follow a pattern, these patterns can be included in the buffering period SEI or directly coded in the picture timing SEI.

# Introduction

Temporal scalability support requires substitution of buffering period SEI messages and picture timing SEI messages so that the defined HRD model leads to a conforming bitstream. Especially, when the level of a sub-layer is lower than that of the whole original CVS with TemporalIds up to sps_max_sub_layers_minus1.

The substitution of the mentioned SEI messages is, for instance, described in a NOTE in Section 10 of HEVC in the sub-bitstream extraction process.

NOTE 3 – A "smart" bitstream extractor may include appropriate non-scalable-nested buffering picture SEI messages, non-scalable-nested picture timing SEI messages and non-scalable-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as scalable-nested SEI messages in the original bitstream.

It is asserted that temporal scalability is a valuable feature that should be supported from the beginning in VVC and that it would be preferred not to require having multiple Buffering Period SEI messages and Pictures Timing SEI messages, some of which are encapsulated into scalable-nested SEI messages that need to be extracted. It is the view of the authors that a solution explicitly specifying derivation of the timings and reducing overhead of multiple SEI message would be preferable.

# Proposal

The removal times of a bitstream encoded with a GOP of 8 are illustrated in Figure 1 with three sub-layers corresponding to 30fps, 60fps and 120fps.

Frame rate 30 fps:

Frame rate 60fps:

Frame rate 120fps:

Figure 1: Removal times for a hierarchical GOP with sub-layers corresponding to 30fps, 60fps and 120fps.

The calculated CPB removal times above show a recognizable pattern with a limited set of delta values between them as shown in the following table:

| 30fps | 60fps | 120fps | Delta(60,30) | Delta(120,30) or Delta(120,60) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| NA | NA | 4/120 | NA | NA |
| NA | 3/60 | 3/120 | NA | 3/120 |
| NA | NA | 5/120 | NA | NA |
| 2/30 | 2/60 | 2/120 | 0 | 0 |
| NA | NA | 7/120 | NA | NA |
| NA | 4/60 | 6/120 | NA | 2/120 |
| NA | NA | 8/120 | NA | NA |
| 1/30 | 1/60 | 1/120 | 1/60 | 3/120 |
| NA | NA | 12/120 | NA | NA |
| NA | 7/60 | 11/120 | NA | 3/120 |
| NA | NA | 13/120 | NA | NA |
| 4/30 | 6/60 | 10/120 | 2/60 | 6/120 |
| NA | NA | 15/120 | NA | NA |
| NA | 8/60 | 14/120 | NA | 2/120 |
| NA | NA | 16/120 | NA | NA |
| 3/30 | 5/60 | 9/120 | 1/60 | 3/120 |
| NA | NA | 20/120 | NA | NA |
| NA | 11/60 | 19/120 | NA | 3/120 |
| NA | NA | 21/120 | NA | NA |
| 6/30 | 10/60 | 18/120 | 2/60 | 6/120 |
| NA | NA | 23/120 | NA | NA |
| NA | 12/60 | 22/120 | NA | 2/120 |
| NA | NA | 24/120 | NA | NA |
| 5/30 | 9/60 | 17/120 | 1/60 | 3/120 |
| NA | NA | 28/120 | NA | NA |
| NA | 15/60 | 27/120 | NA | 3/120 |
| NA | NA | 29/120 | NA | NA |
| 8/30 | 14/60 | 26/120 | 2/60 | 6/120 |
| NA | NA | 31/120 | NA | NA |
| NA | 16/60 | 30/120 | NA | 2/120 |
| NA | NA | 32/120 | NA | NA |
| 7/30 | 13/60 | 25/120 | 1/60 | 3/120 |

The delta values of the removal times for picture belonging to the lowest sub-layer when present at higher framerates are 0, 1/60, 3/120, 2/60 and 6/120. The delta values of the removal times for picture belonging to the second sub-layer when present at higher framerates are 3/120 and 6/120.

It is therefore proposed to indicate the delta values visible from the above pattern in the buffering period SEI message and a reference to a specific delta value in the picture timing SEI message to allow derivation of the correct removal times.

However, when scene cuts happen such a pattern might be interrupted as illustrated in Figure 2:

Figure 2: Hierarchical GOP at 120 fps with exemplary scene cut at the yellow dashed line.

| 30fps | 60fps | 120fps | Delta(60,30) | Delta(120,30) or Delta(120,60) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| NA | NA | 4/120 | NA | NA |
| NA | 3/60 | 3/120 | NA | 3/120 |
| NA | NA | 5/120 | NA | NA |
| 2/30 | 2/60 | 2/120 | 0 | 0 |
| NA | NA | 7/120 | NA | NA |
| NA | 4/60 | 6/120 | NA | 2/120 |
| NA | NA | 8/120 | NA | NA |
| 1/30 | 1/60 | 1/120 | 1/60 | 3/120 |
| NA | NA | 12/120 | NA | NA |
| NA | 7/60 | 11/120 | NA | 3/120 |
| NA | NA | 13/120 | NA | NA |
| 4/30 | 6/60 | 10/120 | 2/60 | 6/120 |
| NA | NA | 15/120 | NA | NA |
| NA | 8/60 | 14/120 | NA | 2/120 |
| NA | NA | 16/120 | NA | NA |
| 3/30 | 5/60 | 9/120 | 1/60 | 3/120 |
| NA | NA | 18/120 | NA | NA |
| NA | 9/60 | 17/120 | NA | 3/120 |
| NA | NA | 2/120 | NA | NA |
| 1/30 | 1/60 | 1/120 | 1/60 | 3/120 |
| NA | NA | 4/120 | NA | NA |
| NA | 2/60 | 3/120 | NA | 1/120 |
| NA | NA | 5/120 | NA | NA |
| 5/30 | 10/60 | 19/120 | 0 | 1/120 |
| NA | NA | 9/120 | NA | NA |
| NA | 5/60 | 8/120 | NA | 2/120 |
| NA | NA | 10/120 | NA | NA |
| 3/30 | 4/60 | 7/120 | 2/60 | 5/120 |
| NA | NA | 12/120 | NA | NA |
| NA | 6/60 | 11/120 | NA | 1/120 |
| NA | NA | 13/120 | NA | NA |
| 2/30 | 3/60 | 6/120 | 1/60 | 2/120 |
| NA | NA | 17/120 | NA | NA |
| NA | 9/60 | 16/120 | NA | 2/120 |
| NA | NA | 18/120 | NA | NA |
| 5/30 | 8/60 | 15/120 | 2/60 | 5/120 |
| NA | NA | 20/120 | NA | NA |
| NA | 10/60 | 19/120 | NA | 1/120 |
| NA | NA | 21/120 | NA | NA |
| 4/30 | 7/60 | 14/120 | 1/60 | 2/120 |

In such a case, it can be seen that the pattern might not apply to the pictures for which the GOP structure is not aligned (see values highlighted in bold font).

Therefore, the proposal is to signal the patterns applicable for a buffering period in the buffering period SEI message, as shown below with the basic buffering period SEI message in [JVET-N0353] as reference:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_seq_parameter_set_id | ue(v) |
| concatenation_flag | u(1) |
| au_cpb_removal_delay_delta_minus1 | u(v) |
| … |  |
| au_cpb_removal_offset_pattern_present_flag | u(1) |
| if( au_cpb_removal_offset_pattern_present_flag ) { |  |
| num_cpb_removal_offsets_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_offsets_minus1 ; i++ ) |  |
| au_cpb_removal_pattern_offset[ i ] | u(v) |
| } |  |
| if( NalHrdBpPresentFlag ) { |  |
| for( i = 0; i < CpbCnt; i++ ) { |  |
| nal_initial_cpb_removal_delay[ i ] | u(v) |
| nal_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( i = 0; i < CpbCnt; i++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

au_cpb_removal_offset_pattern_present_flag specifies the presence of CPB removal time offset patterns. When au_cpb_removal_offset_pattern_present_flag is equal to 1, the buffering period SEI message contains CPB removal time offset patterns. When au_cpb_removal_offset_pattern_present_flag is equal to 0, no CPB removal time offset patterns are present in the buffering period SEI message.

num_cpb_removal_offsets_minus1 plus 1 specifies the number of syntax element au_cpb_removal_pattern_offset[ i ] following in the buffering period SEI message. The value of num_cpb_removal_offsets_minus1 shall be in the range of 0 to 15, inclusive.

au_cpb_removal_pattern_offset[ i ] specifies the CPB removal delay offset of the i-th entry in the CPB removal time offset pattern.

In each picture timing SEI message it is indicated, whether and which au_cpb_removal_pattern_offset described in the buffering period SEI message is to be applied for a specific value of target TemporalIds, i.e. OpTid.

As mentioned before, since for some pictures an offset might not be described with a pattern in the buffering period SEI message, e.g. due to scene cut, a flag is added to indicate whether an offset is to be derived using one of the signalled values of au_cpb_removal_pattern_offset or if the offset value is explicitly signalled within the picture timing SEI message.

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| au_cpb_removal_delay_per_sub_layer_flag | u(1) |
| if( au_cpb_removal_delay_per_sub_layer ) |  |
| buffering_period_pattern_flag | u(1) |
| if( !au_cpb_removal_delay_per_sub_layer ¦¦ buffering_period _pattern_flag ) |  |
| au_cpb_removal_delay_minus1 | u(v) |
| if( au_cpb_removal_delay_per_sub_layer) |  |
| num_sub_layer_cpb_removal_delays_minus1 | ue(v) |
| for( i = 0; i < num_sub_layer_cpb_removal_delays_minus1 + buffering_period _pattern_flag?0:1 ; i++ ) { |  |
| if( buffering_period _pattern_flag ) |  |
| au_cpb_removal_pattern_idx[ i ] | u(v) |
| else |  |
| sub_layer_au_cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| pic_dpb_output_delay | u(v) |
| } |  |
| } |  |

au_cpb_removal_delay_per_sub_layer_flag specifies the presence HRD timing information for two or more target TemporalIds. When au_cpb_removal_delay_per_sub_layer_flag is equal to 1, the buffering period SEI message contains CPB removal delays for two or more target TemporalIds.

buffering_period_pattern_flag plus 1 specifies the presence of the au_cpb_removal_pattern_idx[ i ] in the picture timing SEI message.

au_cpb_removal_pattern_offset[ i ] specifies the CPB removal delay offset of the i-th entry in the CPB removal time offset pattern.

au_cpb_removal_pattern_idx[ i ] specifies the index j of the au_cpb_removal_pattern_offset[ j ] that applies to a OpTid equal to TemporalId of the picture timing SEI message plus i + 1.

sub_layer_au_cpb_removal_delay_minus1[ i ] specifies the number of clock ticks between the CPB removal time of the access unit associated with the picture timing SEI message and the preceding acces unit in decoding order that contained a buffering period SEI message that applies to a OpTid equal to TemporalId of the picture timing SEI message plus i.

The following text changes are relative to the on the HRD description proposed in JVET-N0350.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * AuCpbRemovalDelayVal	(C-11)

where AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period and AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n derived as follows:

–	If au_cpb_removal_delay_per_sub_layer is equal to 0, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit m.

–	Otherwise, if buffering_period _pattern_flag is equal to 0, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1[ i ] in the picture timing SEI message with i set to OpTid – TemporalId of the picture timing SEI message, selected as specified in clause C.1, associated with access unit m.

–	Otherwise, if OpTid is equal to TemporalId of the picture timing SEI message, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit m.

–	Otherwise (OpTid is higher than TemporalId of the picture timing SEI message), AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1 in the picture timing SEI message - au_cpb_removal_pattern_offset[ i ] in the buffering period SEI message with i set to au_cpb_removal_pattern_idx[ j ] and with j set to OpTid – TemporalId – 1 of the picture timing SEI message, selected as specified in clause C.1, associated with access unit m.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).