| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0356 |
| --- | --- |

| Title: | CE12-2: Encoder-side tile group restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Valeri George Karsten Sühring Thomas Schierl | Email: | {forename.surname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results of CE12-2 encoder-side tile group restriction for temporally independent tile group coding.

# Introduction

The CE12-2 test features encoder-side tile group restrictions for temporally independent tile group coding based on VTM 4.0.1. A more detailed test description can be found in the CE document M1032 and the related test conditions document M0870.

Bug#218 was identified after VTM4.0.1 release and was fixed with MR#369. Hence, the commit be55eee8bd3d12bc23016a3eea738ea747fadb84 is used for CE12-2 test.

# Experiment

In summary, the 360° video test sequences SkateboardInLot, ChairLift, KiteFlite, Harbor, Trolley and GasLamp are used in CMP format with a face size of 1536x1536 and a total resolution of 4608x3072 equivalent to 6k. Two tiling setups were investigated, namely 6x4 and 12x8 uniformly sized tiles, which results in tile sizes of 768x768 (i.e. 24 tiles) or 384x384 (i.e. 96 tiles) respectively.

ALF, reshaper and BIO were disabled for the CE12-2 test while for the CE12 anchor, only ALF and reshaper are disabled. Furthermore, each of the tiles in CE12-2 is packaged into an individual tile group while for the anchor, all tiles are packaged into a single tile group per picture.

# Results

6x4 Tiles, RA, CE12-2 Encoder-side MCTS

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Chairlift | 13,83% | 13,40% | 12,80% |
| GasLamp | 4,66% | 3,79% | 4,41% |
| Harbor | 4,20% | 3,89% | 2,95% |
| KiteFlite | 3,30% | 3,44% | 3,35% |
| SkateboardInLot | 12,16% | 15,35% | 17,41% |
| Trolley | 2,44% | 2,38% | 2,77% |
| Average | 6,77% | 7,04% | 7,28% |

12x8 Tiles, RA, CE12-2 Encoder-side MCTS

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Chairlift | 33,54% | 35,70% | 33,50% |
| GasLamp | 13,90% | 13,39% | 13,82% |
| Harbor | 12,36% | 12,65% | 12,20% |
| KiteFlite | 7,08% | 7,80% | 7,66% |
| SkateboardInLot | 25,55% | 33,09% | 38,28% |
| Trolley | 6,35% | 6,82% | 7,71% |
| Average | 16,46% | 18,24% | 18,86% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).