| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0358-v3 |
| --- | --- |

| Title: | Non-CE3: Harmonization of CBF coding for intra sub-partition coding mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, R. Sjöberg | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a method to explicit signal the CBF for each sub-partition when the current block is coded with intra sub-partition (ISP) coding mode. The proposed method removes one restriction on encoder and harmonizes the decode CBF for all sub-partitions. The proposed method has a luma BDR impact for AI/RA/LDB of 0.04%/0.00%/0.01% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/100%/99% compared to VTM-4.0.

# Introduction

In the current VVC, when given a certain block coded with intra sub-partition (ISP) coding mode, the decoder will assume that at least one of the sub-partitions has a non-zero CBF. If n is the number of sub-partitions, the decode CBF for the n-th sub-partition is different from the first n-1 sub-partitions:

- The CBFs for the first n-1 sub-partitions are derived from decoding elements in the bitstream.

- The CBF for the n-th sub-partition is inferred to be 1 when the first n-1 sub-partitions have produced a zero CBF. Otherwise the CBF for the n-th sub-partition is derived from decoding elements in the bitstream.

The restriction of at least one of the sub-partitions has a non-zero CBF disables the encoder to be able to select a certain block coded with ISP coding mode where each sub-partition has a zero CBF.

# Proposed method

This contribution proposes to explicit signal the CBF for each sub-partition when the current block is coded with ISP coding mode. The proposed method eliminates the restriction on encoder. The encoder is able to select a certain block coded with ISP coding mode where each sub-partition has a zero CBF.

From a statistic of simulation with the proposed method, when the current block is coded with ISP coding mode and the current block is divided into 4 sub-partitions, the occurrence probability of a block where each sub-partition has a zero CBF is higher than the occurrence probability of some cases where at least one sub-partition has non-zero CBF.

| CBF combination | Probability | CBF combination | Probability | CBF combination | Probability | CBF combination | Probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 6.7% | 0100 | 3.2% | 1000 | 6.4% | 1100 | 4.8% |
| 0001 | 15.0% | 0101 | 2.4% | 1001 | 3.3% | 1101 | 3.7% |
| 0010 | 3.0% | 0110 | 3.2% | 1010 | 1.6% | 1110 | 5.0% |
| 0011 | 8.2% | 0111 | 8.3% | 1011 | 3.7% | 1111 | 21.5% |

The proposed method harmonizes the encode / decode process of CBF for each sub-partition. The current VVC WD can be simplified as:

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |

The JVET AHG report: Tool reporting procedure (AHG13) [2] reports the ISP tool off has a Luma BDR impact for AI/RA/LDB of 0.61%/0.26%/0.13%. The proposed method has a Luma BDR impact for AI/RA/LDB of 0.04%/0.00%/0.01% compared to VTM4.0. There is marginal impact for AI configuration and almost no degradation for RA and LDB configuration.

# Simulation Result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | 0,04% | 0,07% | 94% | 100% |
| Class A2 | 0,03% | 0,15% | 0,02% | 92% | 100% |
| Class B | 0,04% | 0,06% | 0,10% | 91% | 101% |
| Class C | 0,05% | -0,03% | 0,00% | 81% | 101% |
| Class E | 0,07% | 0,09% | 0,10% | 88% | 101% |
| Overall | 0,04% | 0,06% | 0,06% | 89% | 100% |
| Class D | 0,02% | 0,13% | 0,01% | 78% | 101% |
| Class F | 0,08% | 0,05% | -0,09% | 87% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,06% | -0,06% | 88% | 100% |
| Class A2 | -0,02% | 0,10% | 0,00% | 90% | 101% |
| Class B | 0,01% | -0,04% | -0,10% | 83% | 100% |
| Class C | 0,03% | -0,26% | -0,06% | 82% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | -0,05% | -0,06% | 85% | 100% |
| Class D | -0,03% | -0,12% | 0,39% | 83% | 99% |
| Class F | 0,02% | -0,02% | 0,06% | 89% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,11% | -0,37% | 97% | 99% |
| Class C | -0,02% | 0,32% | -0,06% | 86% | 99% |
| Class E | 0,10% | 0,22% | 0,15% | 80% | 98% |
| Overall | 0,01% | 0,11% | -0,14% | 89% | 99% |
| Class D | -0,05% | -0,21% | -0,64% | 74% | 99% |
| Class F | -0,03% | 0,32% | -0,07% | 87% | 99% |

The encoding time is not reliable due to simulation for anchor and test run on different cluster. The decoder time is measured on same machine.

According to the cross-check report from Huawei, the encoding time is ~99% and decoding time is ~99%.

# Conclusions

We propose to adopt explicit signal the CBF for each sub-partition when the current block is coded with intra sub-partition (ISP) coding mode to VVC and VTM to harmonize the CBF coding and eliminate the encoder restriction.

# References

[1]. JVET-M1001: Versatile Video Coding (Draft 4)

[2]. JVET-M0102: CE3: Intra Sub-Partitions Coding Mode

[3]. JVET AHG report: Tool reporting procedure (AHG13)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).