| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0359 |
| --- | --- |

| Title: | CE11-related: On MV threshold for deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the use of a sub-sample MV component difference threshold for determination of boundary strength in deblocking instead of the luma sample threshold currently used in VVC. The motivation is too avoid passing block artifacts without deblocking. The modification is on top of fixing deblocking of SBT boundaries which not are filtered in VTM-4.0. The proposal gives a benefit in subjective video quality.

# Introduction

When identical reference frames are used in motion compensation for both blocks adjacent to a prediction block boundary the motion vector components of the blocks are compared. If the difference is equal or larger than 1 (in luma samples) the boundary strength is set to 1 and deblocking can be applied to the boundary.

You may think that a threshold of 1 luma sample is sufficient however this contribution asserts that also a sub-sample difference in a motion vector component can produce a visible block boundary.

# Problem description

Consider a gradient in a reference picture. Then consider that this region is referenced by two blocks, block P and block Q, in the current picture, see Figure 2, where block P derives the prediction block by motion compensation the top half of the region and block Q derives the prediction block by motion compensation of the bottom half of the region.

Figure 1: A region of a reference picture				Figure 2: Block P and block Q in current picture

In figure 3 the result of applying motion compensation using VTM interpolation filter with a difference in the horizontal component of the motion vectors of less than one pixel for block P and Q when the vertical component of the motion vectors are identical. It can be seen that even a sub-sample difference in a motion vector component between block P and Q can result in boundary artifacts.

Figure 3: Illustration of two prediction blocks after motion compensation with a given difference in only the horizontal component of the motion vectors. Top left: 1/4 luma sample., Top right: 1/2 luma sample, Bottom left: 3/4 luma sample, Bottom right: 1 luma sample

# Proposed solution

It is proposed to set the MV component difference threshold for determination of boundary strength to 1 in units of quarter luma samples to enable more efficient reduction of blocking artifacts. This is a very conservative threshold but ensures that we not pass any blocking artifacts without deblocking. It could be beneficial to have the threshold signalled in the bitstream to be more flexible and let the encoder decide how conservative it wants to be.

# Results

## Subjective examples

The subjective benefit of the proposal is illustrated for two examples from FourPeople for LDP coding using QP=39. Significant reduction in block artifact can be seen.

Figure 4: FourPeople at POC 171 for QP=39 using LDB. Motion vector difference of 13/16 for the horizontal component and 12/16 for the vertical component. Left: VTM-4.0. Right: Proposal

Figure 5: FourPeople at POC 171 for QP=39 using LDB. Motion vector difference of 13/16 for the horizontal component and 2/16 for the vertical component. Left: VTM-4.0. Right: Proposal

## Objective results vs VTM-4.0

Following table presents CTC results for SBT sub-block deblocking and MV threshold modification over VTM-4.0. To be completed when simulations finish

|  | Random access Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,06% | 0,04% | -0,02% | 99% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0,10% | 0,01% | -0,03% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,16% | -0,09% | -0,01% | 101% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,05% | 0,08% | -0,14% | 98% | 102% |
| Class E | 0,18% | -0,32% | -0,53% | 100% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,03% | 0,16% | -0,18% | 103% | 107% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Following table presents CTC results for SBT sub-block deblocking over VTM-4.0. To be completed when simulations finish

|  | Random access Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,02% | 0,28% | -0,15% | 97% | 102% |
| Class E | -0,04% | -0,04% | -0,22% | 99% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,00% | 0,05% | -0,23% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

We recommend adoption of a sub-sample MV component difference threshold for determining boundary strength for deblocking to VVC draft. This ensures that blocking artifacts that origins from neighbouring blocks with small differences in motion also are deblocked.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).