| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0360 |
| --- | --- |

| Title: | CE6-1: Unification between MTS and Transform Skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-1, which proposes a method for the unification between MTS and Transform Skip (TS) mode. With the proposed method, transform skip mode is unified with identity transform such that one-dimensional transform skip mode using identity transform is also enabled. Compared to VTM-4.0, experimental results report 0.80%, 0.62% and 0.41% coding gain for TGM sequences, under AI, RA and LDB configurations, respectively. However, it is also reported that 0.19%, 0.08% and 0.04% coding loss is observed for natural sequences under AI, RA and LDB configurations with 95%, 100% and 100% enc-time, respectively.

# Proposal

In CE6-1, Transform Skip (TS) mode and MTS are unified in a way that TS mode is implemented using Identity Transform (IDT) which shares the same quantization scheme. In addition, one-dimensional transform skip is also enabled by replacing (DST-7, DCT-8) and (DCT-8, DST-7) in MTS with (DCT-2, IDT) and (IDT, DCT-2), respectively, and the last transform candidate, i.e., (DCT-8, DCT-8), is removed from MTS.

Moreover, for more efficient encoder intra mode selection taking TS mode into consideration, the SATD cost measurement in rough intra mode decision process is replaced by the minimum of SATD and SAD. This aspect has also been proposed in JVET-N0363 [1].

# Experiment results

The proposed methods are implemented on top of VTM-4.0. The experiment is performed under CTC condition [2], and CTC with inter MTS enabled [3].

Table 1: Coding performance of CE6-1 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | -0.08% | 0.00% | 96% | 97% | 0.08% | -0.14% | 0.05% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.30% | -0.03% | -0.19% | 93% | 99% | 0.06% | -0.04% | 0.00% | 101% | 100% |  |  |  |  |  |
| Class B | 0.20% | -0.29% | -0.24% | 95% | 98% | 0.11% | -0.05% | -0.15% | 100% | 99% | 0.03% | -0.37% | -0.05% | 101% | 101% |
| Class C | 0.13% | -0.28% | -0.29% | 95% | 100% | 0.05% | -0.30% | -0.30% | 100% | 101% | -0.04% | 0.16% | -0.11% | 99% | 100% |
| Class E | 0.12% | -0.21% | -0.31% | 94% | 101% |  |  |  |  |  | 0.17% | 0.23% | 0.95% | 100% | 101% |
| Overall | 0.19% | -0.20% | -0.21% | 95% | 99% | 0.08% | -0.13% | -0.12% | 100% | 100% | 0.04% | -0.04% | 0.18% | 100% | 101% |
| Class D | 0.09% | -0.34% | -0.48% | 96% | 101% | 0.01% | -0.15% | -0.34% | 102% | 102% | -0.03% | -0.43% | -0.83% | 101% | 100% |
| Class F | -0.26% | -0.26% | -0.15% | 97% | 100% | -0.23% | -0.24% | -0.32% | 101% | 102% | -0.34% | -0.55% | -0.80% | 100% | 100% |
| Class TGM | -0.80% | -0.38% | -0.41% | 96% | 98% | -0.62% | -0.43% | -0.48% | 101% | 99% | -0.41% | -0.55% | -0.49% | 103% | 103% |

Table 2: Coding performance of CE6-1 when inter MTS is enabled.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This contribution reports the test results of CE6-1 related to unification between MTS and Transform Skip mode. Comparing to VTM-4.0, coding gains are observed for TGM sequences and small coding loss is also observed for natural sequences with 5% enc-time saving for AI configuration.

# References

- X. Zhao, X. Li, S. Liu, “CE6-related: Modified encoder decision for transform skip,” JVET-N0363, 14th Meeting: Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- X. Zhao, H. E. Egilmez, “CE6-related: Modified encoder decision for transform skip,” JVET-M1026, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).