| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0361-v2 |
| --- | --- |

| Title: | CE7-2: Reduced number of context-coded bins |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE7-2, which proposes further reduction on the averaged maximum number of context-coded bins per 4x4 sub-block for transform coefficient coding. With the proposed method, the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region is further reduced such that the averaged maximum number of context-coded bins per 4x4 sub-block is reduced from 32 to 30. Compared to VTM-4.0, experimental results report 0.03%, 0.01% and 0.02% luma BD-rate for AI, RA and LDB configurations, respectively.

# Proposal

In VVC, the maximum number of context-coded bins is constrained to 32 for each 4x4 sub-block regardless of the frequency position of the 4x4 sub-block, so the averaged maximum number of context-coded bins per 16 samples is 32. Since the number of the context-coded bins in high frequency region is usually smaller than that in low frequency region, this contribution proposes to reduce the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region such that the averaged maximum number of context-coded bins per 16 samples is reduced from 32 to 30.

In CE7-2, to limit the worst-case number of context-coded bins, the maximum number of context-coded bins maxCtxBinsNum for 4x4 sub-block in TU is determined as follows:

If (CGX + CGY < CGDiag)

maxCtxBinsNum = 32

Else If(CGX + CGY > CGDiag)

maxCtxBinsNum = 28

Else

maxCtxBinsNum = 30

where (CGX, CGY) is the 4x4 sub-block position and CGDiag is calculated as follows with TU width and height:

maxCGX = (width >> 2) – 1;

maxCGY = (height >> 2) – 1;

CGDiag = (maxCGX + maxCGY) >> 1

An example for 8x16 and 16x16 TU is illustrated in Fig. 1. In this example, TU is grouped into 3 sets by CGDiag: The first group consists of sub-blocks in the top-left region (white region), the second group consists of sub-blocks in middle region (red region) and the third group consists of sub-blocks in the bottom-right region (grey region). The maximum number of context-coded bins for sub-blocks in the first group is 32, the maximum number of context-coded bins for sub-blocks in second group is 30, and the maximum number of context-coded bins for the sub-blocks in the third group is 28. So, the average maximum number of context-coded bins per 4x4 sub-block is 30.

Figure 1. Illustration of grouping 4x4 sub-blocks in 16x16 TU.

# Experiment results

The proposed method is implemented on top of VTM-4.0. The experiment is performed under CTC condition [1] and Low QP configuration as defined in JVET-M01027 [2].

Table 1: Coding performance of CE7-2 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.01% | 102% | 100% | 0.01% | 0.06% | -0.07% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.02% | 0.09% | 0.00% | 100% | 99% | 0.00% | -0.09% | -0.06% | 101% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.06% | 0.05% | 99% | 99% | 0.01% | 0.00% | -0.15% | 100% | 100% | -0.02% | 0.06% | -0.07% | 101% | 100% |
| Class C | 0.06% | 0.05% | 0.13% | 99% | 101% | 0.03% | -0.08% | -0.08% | 99% | 98% | 0.01% | 0.27% | 0.22% | 99% | 97% |
| Class E | 0.01% | 0.10% | 0.00% | 97% | 99% |  |  |  |  |  | 0.09% | 0.99% | -0.16% | 101% | 102% |
| Overall | 0.03% | 0.07% | 0.04% | 99% | 100% | 0.01% | -0.03% | -0.10% | 100% | 99% | 0.02% | 0.36% | 0.00% | 100% | 99% |
| Class D | 0.05% | -0.06% | -0.06% | 101% | 96% | 0.04% | -0.09% | -0.10% | 100% | 97% | 0.01% | -0.16% | 0.20% | 100% | 102% |
| Class F | 0.11% | 0.15% | 0.09% | 101% | 101% | 0.11% | 0.04% | -0.09% | 100% | 99% | -0.07% | 0.54% | -0.26% | 99% | 99% |

Table 2: Coding performance of CE7-2 under Low QP configuration.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.27% | 0.25% | 99% | 102% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.18% | 0.21% | 0.22% | 98% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.13% | 0.14% | 99% | 101% | 0.14% | 0.21% | 0.21% | 102% | 102% |  |  |  |  |  |
| Class C | 0.06% | 0.09% | 0.11% | 99% | 97% | 0.13% | 0.19% | 0.22% | 100% | 99% | 0.14% | 0.29% | 0.26% | 100% | 101% |
| Class E | 0.04% | 0.13% | 0.11% | 98% | 96% |  |  |  |  |  | 0.08% | 0.12% | 0.12% | 100% | 99% |
| Overall | 0.11% | 0.16% | 0.16% | 99% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.06% | 0.08% | 97% | 98% | 0.12% | 0.20% | 0.10% | 99% | 99% | 0.11% | 0.21% | 0.20% | 102% | 101% |
| Class F | 0.10% | 0.17% | 0.16% | 100% | 98% | 0.17% | 0.17% | 0.10% | 100% | 100% | 0.19% | 0.35% | 0.37% | 99% | 99% |

# Conclusions

This contribution reports the test results of CE7-2 related to reduced context coded bins for transform coefficient coding. Comparing to VTM-4.0, average 0.03%, 0.01% and 0.02% luma BD-rate is reported for AI, RA and LDB configurations, respectively. It is proposed to adopt the proposed method into next version of VVC draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” JVET-M1027, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).