| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0363-v2 |
| --- | --- |

| Title: | CE6-related: Modified encoder decision for transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to use the minimum of SAD (Sum of Absolute Difference) and SATD (Sum of Absolute Transform Difference) as the cost measurement for the first round of intra mode pre-selection at the encoder. With the proposed method, for AI (All Intra), RA (Random Access) and LDB (Low-Delay B) configurations, average 0.00%, 0.02% and 0.03% gain is achieved for natural sequences used in CTC test condition, average 0.39%, 0.36% and 0.25% gain is achieved for TGM sequences, respectively. When IBC is turned off, with the proposed method, average 1.80%, 1.21% and 0.45% gain is achieved for TGM sequences for AI, RA and LDB configurations, respectively.

# Introduction

In VTM-4.0, the intra mode decision applies a two-stage intra prediction mode selection. In the first stage, SATD is used in the cost measurement in a rough mode decision to first identify a few potential intra prediction modes. Then in the second stage, the R-D cost is used as the measurement to finally decide which intra prediction mode is used for the actual coding of current block.

However, in VTM-4.0, transform skip mode (TSM) is applied up to 32x32 block, and a transform-domain cost measurement, i.e., SATD, may not match with TSM efficiently since TSM applies quantization and entropy coding directly on the spatial-domain.

# Proposal

It is proposed that, in the first stage of intra mode decision at encoder, both SAD and SATD are calculated, and the minimum value, which replaces SATD, is used in the cost measurement. The proposed cost measurement is described below:

VTM-4.0: Cost = SATD + λ×modeBits

Proposal: Cost = min(SAD, SATD) + λ×modeBits

where modeBits represents the coding bits of intra prediction modes.

# Experiment results

The experiment is performed on top of VTM-4.0 using both CTC condition and CTC without IBC, the anchor used for these experiments is VTM-4.0. Detailed results for each experiment are reported in Table 1 and Table 2, respectively. It is observed that the proposed method has almost no impact on the encoder run-time.

Table 1: Coding gain of proposed method versus VTM-4.0 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.01% | 100% | 97% | 0.00% | 0.00% | -0.01% | 101% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.14% | -0.02% | 102% | 102% | -0.01% | 0.07% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.02% | 100% | 101% | 0.00% | -0.11% | -0.11% | 101% | 100% | 0.00% | -0.24% | -0.25% | 102% | 103% |
| Class C | -0.01% | -0.11% | 0.01% | 101% | 101% | -0.05% | -0.14% | -0.13% | 101% | 101% | -0.05% | 0.26% | -0.01% | 99% | 99% |
| Class E | -0.01% | 0.01% | -0.04% | 99% | 99% |  |  |  |  |  | -0.03% | 0.77% | 0.78% | 103% | 104% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% | -0.02% | -0.06% | -0.06% | 101% | 100% | -0.03% | 0.18% | 0.09% | 101% | 102% |
| Class D | 0.00% | -0.07% | -0.08% | 102% | 103% | -0.06% | -0.26% | -0.16% | 102% | 102% | -0.01% | 0.14% | -0.51% | 101% | 100% |
| Class F | -0.14% | -0.09% | -0.17% | 100% | 100% | -0.18% | -0.19% | -0.31% | 101% | 101% | -0.24% | -0.38% | -0.67% | 99% | 100% |
| Class TGM | -0.39% | -0.24% | -0.23% | 98% | 99% | -0.36% | -0.32% | -0.30% | 101% | 100% | -0.25% | -0.34% | -0.26% | 102% | 101% |

Table 2: Coding gain of proposed method versus VTM-4.0 under CTC with IBC off.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.01% | 101% | 100% | 0.00% | 0.00% | -0.01% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.14% | -0.02% | 102% | 102% | -0.01% | 0.07% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.02% | 99% | 101% | 0.00% | -0.11% | -0.11% | 101% | 101% | 0.00% | -0.24% | -0.25% | 102% | 103% |
| Class C | -0.01% | -0.11% | 0.01% | 102% | 102% | -0.05% | -0.14% | -0.13% | 102% | 102% | -0.05% | 0.26% | -0.01% | 99% | 101% |
| Class E | -0.01% | 0.01% | -0.04% | 97% | 99% |  |  |  |  |  | -0.03% | 0.77% | 0.78% | 102% | 104% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 101% | -0.02% | -0.06% | -0.06% | 101% | 101% | -0.03% | 0.18% | 0.09% | 101% | 102% |
| Class D | 0.00% | -0.07% | -0.08% | 102% | 102% | -0.06% | -0.26% | -0.16% | 102% | 103% | -0.01% | 0.14% | -0.51% | 99% | 99% |
| Class F | -0.59% | -0.40% | -0.53% | 98% | 101% | -0.50% | -0.41% | -0.29% | 101% | 102% | -0.18% | -0.03% | 0.31% | 101% | 102% |
| Class TGM | -1.80% | -1.38% | -1.43% | 100% | 101% | -1.21% | -1.11% | -1.11% | 103% | 101% | -0.45% | -0.70% | -0.78% | 104% | 104% |

# Conclusions

This proposal suggests a modified cost measurement for intra mode decision of VTM sw. With the proposed method, cost measurement is selected among the minimum of SAD and SATD to better match the different transform schemes that may be applied on the prediction residual. It is recommended to adopt the proposed changes into next version of VTM sw.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).