| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0365 |
| --- | --- |

| Title: | CE6-related: MPM based non-separable secondary transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an MPM-based non-separable secondary transform (NSST) scheme on top of CE6-3.1d. With the proposed method, NSST is applied only for most probable modes (MPMs), or only for MPMs excluding non-angular modes (i.e., Planar and DC modes). Compared to VTM-4.0, experimental results reports 1.04%, 0.xx% and 0.xx% coding gain for AI, RA and LDB configurations with 118% enc-time when CE6-3.1d is only applied for MPMs, and 0.82%, 0.xx% and 0.xx% coding gain for AI, RA and LDB configurations with 109% enc-time when CE6-3.1d is only applied for angular MPMs (excluding Planar and DC modes).

# Problem Statement and Proposal

In CE6-3, a non-separable secondary transform (called reduced-size transform (RST)) is applied using 4x4 and 8x8 non-separable secondary transform for small and larger blocks, respectively. The transform set is selected based on intra prediction mode according to pre-defined mapping rule. Totally 4 transform sets are used, and each transform set includes 2 RST kernels, and the selection of transform kernel, as indicated by an index, is signalled. In CE6-3.1d, for 8x8 RST, 48x16 matrices are used to reduce memory cost for storing transform kernels, which is 8 KB with CE6-3.1d.

With the proposed RST, the encoder needs to make decision which RST kernel is selected, which introduces relatively high encoder complexity, around 130% for AI configuration.

To reduce the run-time, two methods are proposed. In the first method, namely Method 1, NSST is only applied for MPMs. In the second method, namely Method 2, NSST is applied only for MPMs without Planar or DC modes, which further reduces the memory cost from 8KB to 6KB.

# Experiment results

The proposed methods are implemented on top of VTM-4.0. The experiment is performed under CTC condition [1], and the anchor is VTM-4.0. Detailed results for Method 1 and 2 are reported in Table 1 and Table 2, respectively.

Table 1: Coding performance of Method 1.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.60% | -0.43% | -0.34% | 119% | 95% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -1.05% | 0.38% | 0.71% | 121% | 97% |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.78% | -0.05% | -0.88% | 119% | 95% |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.91% | -0.15% | -0.86% | 118% | 98% |  |  |  |  |  |  |  |  |  |  |
| Class E | -1.07% | -1.06% | -1.75% | 109% | 94% |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.04% | -0.23% | -0.67% | 118% | 96% |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.64% | -0.22% | -0.85% | 121% | 101% |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.90% | -0.05% | -0.28% | 112% | 96% |  |  |  |  |  |  |  |  |  |  |

Table 2: Coding performance of Method 2.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.01% | -0.56% | -0.34% | 111% | 95% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -0.93% | 0.28% | 0.56% | 112% | 97% |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | -0.12% | -1.01% | 110% | 94% |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.84% | -0.22% | -0.87% | 111% | 97% |  |  |  |  |  |  |  |  |  |  |
| Class E | -0.84% | -1.06% | -1.77% | 103% | 94% |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.82% | -0.30% | -0.73% | 109% | 95% |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.60% | -0.27% | -0.91% | 112% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.84% | -0.07% | -0.30% | 106% | 97% |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This proposal provides further complexity reduction for CE6-3.1d related to non-separable secondary transform. Comparing to VTM-4.0, the proposed method 1 achieves -1.04% coding gain for AI configuration with 118% enc-time and 8KB memory cost, and the proposed method 2 achieves -0.82% coding gain for AI configuration with 109% enc-time and 6KB memory cost. It is recommended to further study the proposed changes in the next Core Experiment.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).