| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0368 |
| --- | --- |

| Title: | An implementation of adaptive color transform in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiaozhong Xu, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xiaozhongxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution provides an implementation of adaptive color-space transform on top of VTM-4.0. With the proposed method, the processing of prediction and residual coding can be done in a different color space. The proposed method can be switched on and off for each CU using a flag explicitly signaled at CU-level. Experimental results show that, comparing to VTM-4.0 with support of chroma 4:4:4 format as proposed in JVET-N0367, the proposed method achieves up to 14.1% coding gain for RGB video content under All Intra (AI) configuration when IBC, Dualtree and In-loop Reshaper is turned off.

# Proposal

For efficient coding of RGB video content, the in-loop Adaptive Colour Transform (ACT) was adopted into SCM2.0, where ACT is operated in the residue domain, and a CU-level flag is signaled to indicate the usage of the color-space transform.

In this contribution, we propose an implementation of ACT on top of VTM-4.0. Different from the ACT in SCM which is performed between prediction and transform, the proposed implementation performs ACT before prediction and after pixel reconstruction. Due to the modified processing order of ACT, a different colour-space transform is also proposed to equalize the dynamic range of pixel value after applying colour-space transform.

## Framework

At the encoder, the proposed ACT is performed before prediction, and the reference samples and input original samples are both mapped to a different colour space in case ACT is applied for current CU. For pixel reconstruction, in case ACT is applied, the reference samples are mapped to the alternative color space before being used for prediction, and reconstructed samples are then mapped back to the original colour space. The encoder and decoder flow chart using the proposed ACT scheme is described in

Figure 1: Flow chart of proposed encoding and decoding loop using ACT.

## Transform

On top of the color transform process defined in SCM, the 2nd and 3rd components are further offset by a constant c after and before the color transform for forward and backward transform, respectively. The constant c is derived as 1<<(bitDepth-1), where bitDepth refers to the bit-depth of input sample.

Forward transform:		YCgCo=14121-12-120-2×RGB+0cc

Backward transform:	RGB=1-111101-1-1×YCg-cCo-c

The purpose of adding the offset is to keep the same dynamic range of the sample values after color transform.

## Signaling

The proposed method adds a CU-level flag which is signaled to indicate whether ACT is applied or not. Only one context is applied for this flag.

# Experiments

Simulations are performed under CTC condition [1] with serval coding tools disabled. The disabled tools include IBC, In-loop Reshaper and Dualtree. The test materials are same as defined in the CTC used in Screen Content Coding (SCC) [2]. The anchor is VTM-4.0 with the support of chroma format 4:4:4 and the test is the anchor with the proposed implementation of ACT.

Table 1: Coding performance of proposed ACT versus VTM-4.0 with chroma format 4:4:4 support.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -12.5% | -21.5% | -17.0% |
| RGB, mixed content, 1440p & 1080p | -12.7% | -24.3% | -17.9% |
| RGB, Animation, 720p | -2.7% | -9.0% | -7.2% |
| RGB, camera captured, 1080p | -6.9% | -13.9% | -7.7% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.6% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | -0.1% | -0.5% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 183% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusions

This proposal provides an implementation of adaptive color transform on top of VTM-4.0. Coding gains are reported for RGB video content. It is recommended to further study the proposed adaptive color transform scheme in Core Experiment.

# References

- L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm), X. Xiu, Y. He, and Y. Ye (InterDigital), “SCCE5 Test 3.2.1: In-loop color-space transform,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0147, 30 June – 9 July 2014.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- On test conditions for screen content coding,” JCTVC-U1015, 21st Meeting: Warsaw, PL, 19–26 June 2015.

- X. Zhao, X. Li, X. Xu, S. Liu, “Support of chroma 4:4:4 format for VVC”, JVET-N0467, 14th meeting, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).