| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0369-v1 |
| --- | --- |

| Title: | CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution provides supplemental results for CE2-4.8 (JVET-N0068). Different from algorithm tested in JVET-N0068, in this variant the reference block bounding box size is computed for 2x2 sub-block vectors for both bi-directional and unidirectional affine mode.

For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  2*17*17 for 2x2 sub-block vectors of unidirectional affine mode, the average BD-differences compared to the VTM4.0 are as follows:

- For Test1 in which the fallback mode is not triggered, the BD-rate difference is 0.01% in RA and 0.00% in LD_B, respectively.

- For Test2 in which the fallback mode is triggered, the BD-rate difference is -0.01% in RA and -0.01% in LD_B, respectively.

# Algorithm description

To impose the same constraint proposed in  [4] on the decoder side, the following algorithm is tested. The decoder checks whether the following conditions are satisfied:

- If a PU uses bi-directional affine mode (i.e. bi-pred), the reference block bounding box size bxW4*bxH4 is less than or equal to Thred4.

- Otherwise, if the PU is coded in unidirectional affine mode, the reference block bounding box size bxW4*bxH4 is less than or equal to 2*Thred4.

If yes, the decoder generates the sub-block vectors of the PU in the same way as defined in the VTM4.0. Otherwise, all the sub-block vectors of the PU are set to the same vector which is computed at the center position of the PU by using the affine motion model.

As defined in JVET-M0049, the width and height of the reference block bounding box of 2x2 sub-block vectors, i.e. bxW4,bxH4, are computed by:

Figure 2: Illustration of reference block bounding box of 2x2 sub-block motion vectors in a PU coded affine mode

bxW4=max0, 41+a, 4c,41+a+4c-min0, 41+a, 4c,41+a+4c+11bxH4=max0, 4b, 41+d,4b+41+d-min 0, 4b, 41+d,4b+41+d+11

Where the affine motion model is defined as:

vx=ax+cy+evy=bx+dy+f

The threshold for a PU coded in is defined as:

Thred4=(15+δx)*(15+δy)

Where δx and δy are parameters used for controlling the threshold.

# Experimental results

Tests were carried out by using the VTM4.0 software, and the common test conditions (CTC) defined in [1] was followed. In the tests, the threshold of the luma reference block bounding box is set to Thred4=17*17 for 2x2 sub-block vectors of the affine mode (i.e. setting δx=δy=2).

The following two tests were carried out:

- Test1: the encoder controls the affine motion estimation so that the fallback mode is never triggered in the bitstreams.

- Test2: the encoder allows the use of fallback mode if the reference block bounding box sizes exceed the pre-defined thresholds.

As shown in Table 1, for Test1 in which the fallback mode is not triggered, the BD-rate difference is 0.01% in RA and 0.00% in LD_B, respectively. For Test 2 in which the fallback mode is triggered (see Table 2), the BD-rate difference is -0.01% in RA and -0.01% in LD_B, respectively. Triggering the fallback mode on the encoder side appears to be providing slightly better coding efficiency.

Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.12% | 91% | 94% |
| Class A2 | 0.02% | 0.00% | 0.05% | 96% | 99% |
| Class B | 0.01% | 0.02% | 0.09% | 94% | 93% |
| Class C | 0.01% | 0.00% | -0.05% | 94% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.05% | 94% | 95% |
| Class D | 0.00% | 0.03% | 0.02% | 93% | 97% |
| Class F (optional) | 0.04% | 0.06% | 0.01% | 92% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.10% | -0.08% | 95% | 105% |
| Class C | 0.00% | 0.19% | 0.16% | 93% | 91% |
| Class E | -0.03% | 0.33% | -0.16% | 82% | 87% |
| Overall | 0.00% | 0.10% | -0.02% | 91% | 96% |
| Class D | 0.00% | -0.03% | -0.59% | 101% | 99% |
| Class F (optional) | -0.10% | -0.45% | -0.08% | 99% | 96% |

Table 1: Summary results of Test1 ( δx=δy=2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.07% | 93% | 93% |
| Class A2 | -0.02% | -0.11% | 0.00% | 88% | 98% |
| Class B | -0.01% | -0.07% | 0.03% | 91% | 92% |
| Class C | -0.01% | -0.08% | -0.08% | 90% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | 0.00% | 91% | 93% |
| Class D | 0.00% | -0.04% | -0.08% | 89% | 95% |
| Class F (optional) | -0.04% | -0.02% | -0.01% | 95% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.02% | -0.13% | 92% | 101% |
| Class C | -0.05% | 0.10% | 0.03% | 93% | 99% |
| Class E | 0.08% | 0.23% | -0.41% | 89% | 97% |
| Overall | -0.01% | 0.08% | -0.15% | 92% | 100% |
| Class D | -0.01% | -0.19% | -0.66% | 96% | 118% |
| Class F (optional) | -0.18% | -0.15% | -0.37% | 113% | 97% |

Table 2: Summary results of Test2 ( δx=δy=2)

# Memory bandwidth measurements

We conducted a memory bandwidth study by running both the software of the proposal and the VTM4.0 with a commercial motion compensation cache model integrated. The Random Access (RA) summary results of Test 1 and Test 2 are provided in the Table 3 and Table 4.

As shown in Tables below, the restrictions do not have an impact on memory bandwidth consumption when compared to VTM4.0 (i.e. w/o restrictions) according the “MBW_diff” measurements.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.12% | -0.06% | -0.04% | -0.32% | 91% | 105% |
| Class A2 | 0.02% | 0.00% | 0.05% | -0.09% | -0.19% | 0.32% | 96% | 112% |
| Class B | 0.01% | 0.02% | 0.09% | -0.02% | 0.00% | -0.19% | 94% | 104% |
| Class C | 0.01% | 0.00% | -0.05% | 0.10% | 0.09% | 0.93% | 94% | 107% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.05% | -0.03% | -0.03% | 0.93% | 94% | 107% |
| Class D | 0.00% | 0.03% | 0.02% | -0.04% | -0.02% | 0.21% | 93% | 109% |
| Class F (optional) | 0.04% | 0.06% | 0.01% | -0.06% | 0.00% | 0.62% | 92% | 110% |

Table 3: Memory bandwidth measurement for Test 1 ( δx=δy=2)

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.07% | -0.02% | 0.00% | 0.00% | 93% | 104% |
| Class A2 | -0.02% | -0.11% | 0.00% | 0.01% | 0.00% | 0.63% | 88% | 111% |
| Class B | -0.01% | -0.07% | 0.03% | 0.02% | 0.04% | -0.93% | 91% | 108% |
| Class C | -0.01% | -0.08% | -0.08% | 0.01% | 0.07% | 0.00% | 90% | 110% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.07% | 0.00% | 0.01% | 0.04% | 0.63% | 91% | 108% |
| Class D | 0.00% | -0.04% | -0.08% | 0.00% | 0.04% | 0.21% | 89% | 110% |
| Class F (optional) | -0.04% | -0.02% | -0.01% | 0.05% | 0.11% | -0.31% | 95% | 109% |

Table 4: Memory bandwidth measurement for Test 2 ( δx=δy=2)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage increase relative to VTM4.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage increase relative to VTM4.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage increase relative to VTM4.0.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-M1001-v6, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[3] Y. He, C.-Y. Chen and C-C Chen, “Description of Core Experiment 6 (CE2): Sub-block based motion prediction”, JVET-M1022, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[4] M. Zhou, “A restriction on memory bandwidth consumption of affine mode”, JVET-M0049, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[5] M. Zhou, “On restriction of memory bandwidth consumption of affine mode (CE2-4.8)”, JVET-N0068, the 14th JVET meeting, Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-M1001-v6

In section 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors,

Text for Tes t1 and Test 2 as decoder conformance check

Replace

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-776)

yPosCb = 2 + ( ySbIdx << 2 )		(8-777)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-778)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-779)

with (new items marked in yellow)

The variables bxWX4,bxHX4are derived as follows:

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY  (8-xxx)

bxWX4=(maxW4-minW4≫11)+11bxHX4=(maxH4-minH4≫11)+11                                                            (8-xxx)

The variable isFallbackModeTriggered is set equal to 1, and is updated as follows:

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, and bxWX4*bxHX4 is less than or equal to 289,  isFallbackModeTriggered is set equal to 0.

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, and bxWX4*bxHX4 is less than or equal to 578,  isFallbackModeTriggered is set equal to 0.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

  - If  isFallbackModeTriggered is equal to 1, the variables  xPosCb and yPosCb are derived as follows:

xPosCb =  (cbWidth>>1)		(8-776)

yPosCb =  (cbHeight>>1)		(8-777)

  - Otherwise, the variables  xPosCb and yPosCb are derived as follows:

xPosCb = 2 + ( xSbIdx << 2 )		(8-776)

yPosCb = 2 + ( ySbIdx << 2 )		(8-777)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-778)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-779)

Text for Test 1 as bitstream restriction

Add (new items marked in yellow)

It is a requirement of bitstream conformance that

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, bxWX4*bxHX4 shall be  less than or equal to 289

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, bxWX4*bxHX4 shall be  less than or equal to 578

Where the values of (bxWX4,bxHX4) are computed by:

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY  (8-xxx)

bxWX4=(maxW4-minW4≫11)+11bxHX4=(maxH4-minH4≫11)+11                                                            (8-xxx)