| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0370-v1 |
| --- | --- |

| Title: | CE1: Intra reference sample deblocking (CE1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, R. Sjöberg, J. Ström | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Angular intra prediction modes can create annoying artifacts along the prediction direction for large blocks due to use of reference samples which are populated from a boundary between two large blocks.

This contribution reports the test results of two tests CE1.6.1 Intra reference sample deblocking (JVET-M0138) and CE1.6.2 HEVC strong reference sample smoothing. The two CE1.6 tests are designed to eliminate the discontinuity between reference samples. The two CE1.6 tests are asserted to significantly reduce artifacts from angular intra prediction.

CE1.6.1 has a luma BDR impact for AI/RA/LDB of -0.03%/-0.02%/0.00% compared to VTM-4.0 and decoding time impact for AI/RA/LDB of 100%/100%99% compared to VTM-4.0.

CE1.6.2 has a luma BDR impact for AI/RA/LDB of 0.02%/0.00%/-0.01% compared to VTM-4.0 and decoding time impact for AI/RA/LDB of 101%/100%100% compared to VTM-4.0.

# Introduction

In Versatile Video Coding (VVC), blocking artifacts in previously decoded CUs may introduce artifacts along the intra prediction direction in a current CU. The value discontinuity of reference samples can not be eliminated sufficiently by the [1 2 1] / 4 low-pass filter or 4-tap interpolation filter. Figure 1 shows an example of artifacts along the intra prediction direction coded with VTM 4.0. The example is from Tango2 in RA configuration with QP39, the POC is 16 and the slice QP is 35. The artifacts in yellow circle is for a 32x64 CU with intra prediction mode 38.

Figure 1, an example of visual artifact for a 32x64 CU with intra prediction mode 38

# Proposed method

## Intra reference sample deblocking

JVET-M0138 presents an Intra Reference Sample Deblocking (IRSDB) method for angular intra prediction of blocks with both width and height equal to or larger than 32.

IRSDB is designed to symmetrically deblock 7 unfiltered reference samples on respective side of a block boundary to eliminate the discontinuity between reference samples which are populated from two neighboring large blocks. IRSDB first define a set of checkpoints that includes the following positions:

- at the sample to the top-left of the current block

- at each position (X, Y) above the current block that X is divisible by 32

- at each position (X, Y) to the left of the current block that is Y divisible by 32

Figure 2 shows example of a set of checkpoint positions for a 32x64 block.

Figure 2.  Example of a set of checkpoint positions for a 32x64 block

At each checkpoint, the block to the left or below of the checkpoint is block P, the block to the right or above of the checkpoint is block Q. For filter decision and operation, we use 8 reference samples p0,i to p7,i in block P, where p0,i is nearest to checkpoint and 8 reference samples q0,i to q7,i in block Q, where q0,i is nearest to checkpoint. Figure 3 shows the block P, block Q and reference sample pn,i and qn,i when checkpoint positions at top-left, above and left of current CU, wherein n ranges from 0 to 7 and i = multiple reference line index.

Figure 3. An example of block P, block Q for checkpoint positions at top-left, above and left of current CU

At each checkpoint, the unfiltered reference samples are examined to check whether or not to apply filtering to the reference samples pn,i and qn,i for the checkpoint:

- abs(p0,i-q0,i) < 2.5*tc, where tc value depends on the slice QP as in VTM deblocking.

- abs(p0,i-p3,i-p4,i+p7,i) + abs(q0,i-q3,i-q4,i+q7,i) < (3 * beta / 32), where beta value depends on the slice QP similar to VTM deblocking.

For the checkpoint is to the top-left of the current block:

p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos-1-i, yPos),  (xPos-1-i, yPos+3), (xPos-1-i, yPos+4), (xPos-1-i, yPos+7), (xPos, yPos-1-i), (xPos+3, yPos-1-i), (xPos+4, yPos-1-i) and  (xPos+7,yPos-1-i).

For a checkpoint to the left of current CU at position (xPos-1-i, yPos+yO+32*k) where k ranges from 1 to (2*CUheight/32-1):

p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos-1-i, yPos+yO+32*k), (xPos-1-i, yPos+yO+32*k+3), (xPos-1-i, yPos+yO+32*k+4), (xPos-1-i, yPos+yO+32*k+7), (xPos-1-i, yPos+yO+32*k-1), (xPos-1-i, yPos+yO+32*k-4), (xPos-1-i, yPos+yO+32*k-5) and (xPos-1-i, yPos+yO+32*k-8).

For a checkpoint to the above of current CU at position (xPos+xO+32*k,yPos-1-i) where k ranges from 1 to (2*CUwidth/32-1):

p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos+xO+32*k-1, yPos-1-i), (xPos+xO+32*k-4, yPos-1-i), (xPos+xO+32*k-5,yPos-1-i), (xPos+xO+32*k-7, yPos-1-i), (xPos+xO+32*k,yPos-1-i), (xPos+xO+32*k+3,yPos-1-i), (xPos+xO+32*k+4,yPos-1-i) and (xPos+xO+32*k+7,yPos-1-i).

Where the top-left sample of the current block is at (xPos,yPos), i = multi reference line index, xO is the initial offset of xPos for checkpoint position at above of current CU, yO is the initial offset of yPos for checkpoint position at left of current CU, xO is equal to 16 when CU width is equal to 32 and xPos % 32 is not equal to 0, otherwise xO is equal to 0. yO is equal to 16 when CU height is equal to 32 and yPos % 32 is not equal to 0, otherwise yO is equal to 0.

The IRSDB filter operation:

- For intra reference samples pn,i and qn,i for n = 0 to 6 on each side of the checkpoint are then replaced by linear interpolation as follows:

  - pn,i´=(fn*Middle+64-fn*P)+32)≫6)

  - qn,i´=(fn*Middle+64-fn*Q+32)≫6)

where f, Middle, P and Q are given below:

fn=59-n*9,  can also be described as f=59,50,41,32,23,14,5

Middle=(2*p0,i+q0,i+p1,i+q1,i+p2,i+q2,i+p3,i+q3,i+p4,i+q4,i+p5,i+q5,i+p6,i+q6,i+8)≫4

P=(p6,i+p7,i+1)≫1,      Q=(q6,i+q7,i+1)≫1

Where the positions of the unfiltered samples pn,i and qn,i,

wherein n ranges from 0 to 6, i = multiple reference line index, are given by:

- pn,i = (xPos-1-i,yPos+n), qn,i = (xPos+n, yPos-1-i) for a top-left checkpoint,

- pn,i = (xPos-1-i, yPos+yO+32*k+n), qn,i = (xPos-1-i, yPos+yO+32*k-1-n) for a left checkpoint at k,

- pn,i = (xPos+xO+32*k-1-n, yPos-1-i), qn,i = (xPos+xO+32*k+n, yPos-1-i) for an above checkpoint at k,

where the top-left sample of the current block is at (xPos,yPos),

wherein, xO is equal to 16 when CUwidth is equal to 32 and xPos % 32 is not equal to 0, otherwise xO is equal to 0.

wherein, yO is equal to 16 when CUheight is equal to 32 and yPos % 32 is not equal to 0, otherwise yO is equal to 0.

- When the top-left checkpoint is decided to apply the IRSDB filter, the reference samples to the top-left of the current block is filtered as follows:

reftop-leftxPos+a,yPos+b´=Middle,

  - Multi reference line index is 0, reftop-left is the reference sample at top-left of current block where both a and b are equal to -1.

  - Multi reference line index > 0, reftop-left are the reference samples at top-left of current block where both a and b are equal to -i-1, and i samples at below of the top-left reference sample where a is equal to -i-1 and b ranges from -1 to -i and i samples at right of the top-left reference sample where a ranges from -1 to -i and b is equal to -i-1, where i = multi reference line index.

The IRSDB reference samples are further filtered by [1 2 1] / 4 low-pass filter or 4-tap interpolation filter depending on the intra prediction mode. [ref JVET-L0628]

The directional intra-prediction mode is classified into one of the following groups:

  - vertical or horizontal modes (HOR_IDX, VER_IDX),

  - diagonal modes that represent angles which are multiple of 45 degree (2, DIA_IDX, VDIA_IDX),

  - remaining directional modes;

|  | Reference sample filter |
| --- | --- |
| Group A | IRSDB |
| Group B | IRSDB + [1 2 1] / 4 low-pass filter |
| Group C | IRSDB + 4-tap interpolation filter |

## HEVC strong reference sample smoothing

HEVC strong reference sample smoothing filter is a method in HEVC which not is present in VVC but exists in VTM (default turned off). The HEVC strong reference sample smoothing filter is applied to the luma reference samples of large blocks that have both a block width and a block height larger than or equal to 32 luma samples. The HEVC strong reference sample smoothing filter firstly test the signal activity. If the signal activity for both reference samples at the left and above the current block are below a threshold, the reference samples are modified by a linear interpolation between the top-right reference sample and the top-left reference sample, and by a linear interpolation between the bottom-left reference sample and the top-left reference sample.

In the CE test, the HEVC strong reference sample smoothing implementation is aligned with HEVC to only modify twice as many samples as the block width minus 1 for above reference samples and twice as many samples as the block height minus 1 for left reference samples both modifications based on linear interpolation between the top-left reference sample and a sample at a distance of twice the block width respectively twice the block height from the top-reference sample. Figure 4 shows the modified reference samples for a 32×64 block when multiple reference line index is 0, 1 or 3.

Figure 4. Modified samples for a 32×64 block when multiple reference line index is 0, 1 or 3

The unfiltered reference samples are examined to check whether or not to apply HEVC strong reference sample smoothing to the current block:

- abs(ptop-left-2×pmiddle-left+pbottom-left)<(1≪(b-5))

- abs(ptop-left-2×ptop-middle+ptop-right)<(1≪(b-5))

Wherein,

ptop-left=p-1-refIdx[-1-refIdx]

pbottom-left=p-1-refIdxrefH-1-refIdx

pmiddle-left=p-1-refIdx[(refH≫1)-1]

ptop-right=prefW-1-refIdx[-1-refIdx]

ptop-middle=p(refW≫1)-1[-1-refIdx]

Wherein,

b specifies the sample bit depth,

refIdx specifies the multiple reference line index,

the W specifies block width,

the H specifies the block height,

refW = 2×W specifies number of reference samples at the above of current block,

refH = 2×H specifies number of reference samples at the left of current block,

pxy specifies the unfiltered reference samples

When the HEVC strong reference sample smoothing is decided to be applied to the current block, the modified reference samples p'xy are derived by follows:

- Generate reference samples by linear interpolation between pbottom-left and ptop-left :

p'-1-refIdxy=

refH-(y+1+refIdx)×ptop-left+(y+1+refIdx)×pbottom-left+refH≫1≫log2refH

y in range of[-refIdx,…,0,1,2,…,refH-2-refIdx]

- Generate reference samples by linear interpolation between ptop-left and prop-right :

p'x-1-refIdx=refW-x+1+refIdx×ptop-left+x+1+refIdx×ptop-right+refW≫1≫log2refWx in range of -refIdx,…,0,1,2,…,refW-2-refIdx

Wherein,

p-1-refIdx-1-refIdx specifies the reference sample at the top-left corner of current block,

p[refW+max1,WH×refIdx-1]-1-refIdx specifies the reference sample at the top-right corner of current block,

p-1-refIdx[refH+max1,HW×refIdx-1] species the reference sample at bottom-left corner of current block.

The HEVC strong reference sample smoothing modified reference samples are further filtered by [1 2 1] / 4 low-pass filter or 4-tap interpolation filter depending on the intra prediction mode. [ref JVET-L0628]

The directional intra-prediction mode is classified into one of the following groups:

- vertical or horizontal modes (HOR_IDX, VER_IDX),

- diagonal modes that represent angles which are multiple of 45 degree (2, DIA_IDX, VDIA_IDX),

- remaining directional modes;

|  | Reference sample filter |
| --- | --- |
| Group A | HEVC strong reference sample smoothing |
| *Group B | HEVC strong reference sample smoothing + [1 2 1] / 4 low-pass filter |
| Group C | HEVC strong reference sample smoothing + 4-tap interpolation filter |

(* For Group B, to be aliened with the IRSDB test, the HEVC strong reference sample smoothing modified reference samples are further filtered by [1 2 1] / 4 low-pass filter.

Test results for NOT applying the [1 2 1] / 4 low-pass filter to the HEVC strong reference sample smoothing modified reference sample is reported as additional test results. The additional test results do not show noticeable difference compared to the reported CE test result of CE1.6.2 for both objective and subjective performance.)

# Simulation result

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE1.6.1 (IRSDB) Over VTM-4.0 |  |  |  |  | CE1.6.2 (HEVC) Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0,13% | -0,04% | -0,09% | 98% | 100% | 0,05% | 0,06% | 0,05% | 98% | 101% |
| Class A2 | -0,02% | 0,10% | -0,03% | 97% | 100% | 0,02% | 0,20% | 0,00% | 99% | 101% |
| Class B | -0,01% | -0,03% | 0,02% | 86% | 100% | 0,02% | 0,00% | 0,05% | 99% | 99% |
| Class C | 0,00% | 0,04% | 0,04% | 96% | 100% | 0,01% | 0,00% | 0,00% | 93% | 102% |
| Class E | -0,03% | -0,01% | -0,05% | 94% | 101% | -0,03% | 0,04% | -0,08% | 93% | 100% |
| Overall | -0,03% | 0,01% | -0,02% | 93% | 100% | 0,02% | 0,05% | 0,01% | 96% | 101% |
| Class D | 0,01% | -0,03% | -0,04% | 79% | 101% | 0,00% | -0,04% | -0,06% | 87% | 99% |
| Class F | 0,01% | -0,03% | -0,10% | 102% | 100% | 0,02% | -0,09% | -0,02% | 96% | 99% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE1.6.1 (IRSDB) Over VTM-4.0 |  |  |  |  | CE1.6.2 (HEVC) Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,15% | -0,10% | 100% | 100% | 0,02% | -0,05% | 0,04% | 110% | 100% |
| Class A2 | -0,02% | 0,01% | -0,02% | 97% | 99% | -0,03% | 0,06% | 0,06% | 105% | 97% |
| Class B | 0,00% | 0,03% | -0,15% | 91% | 102% | 0,02% | 0,08% | -0,11% | 102% | 100% |
| Class C | 0,00% | 0,01% | -0,07% | 90% | 100% | 0,00% | -0,04% | -0,09% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,01% | -0,09% | 94% | 100% | 0,00% | 0,02% | -0,04% | 104% | 100% |
| Class D | 0,00% | -0,12% | 0,10% | 88% | 99% | -0,02% | -0,14% | -0,13% | 101% | 100% |
| Class F | 0,00% | -0,01% | -0,05% | 94% | 102% | 0,00% | -0,04% | -0,14% | 93% | 100% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE1.6.1 (IRSDB) Over VTM-4.0 |  |  |  |  | CE1.6.2 (HEVC) Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0,01% | -0,13% | 0,06% | 100% | 100% | 0,00% | -0,25% | -0,37% | 100% | 100% |
| Class C | -0,02% | 0,16% | 0,20% | 99% | 100% | -0,04% | 0,28% | -0,05% | 103% | 101% |
| Class E | 0,05% | 2,08% | 1,65% | 95% | 98% | -0,01% | 1,02% | 0,57% | 102% | 100% |
| Overall | 0,00% | 0,52% | 0,51% | 98% | 99% | -0,01% | 0,25% | -0,03% | 101% | 100% |
| Class D | -0,03% | 0,14% | -0,15% | 96% | 100% | 0,05% | 0,09% | 0,15% | 96% | 99% |
| Class F | -0,11% | -0,26% | -0,59% | 99% | 101% | -0,13% | 0,40% | -0,28% | 102% | 100% |

# Complexity analysis

The IRSDB and HEVC strong reference sample smoothing can be applied to block size 32 × 32, 32 × 64 and 64 × 64. The computational complexity in term of the total number of operations for each block size are as follows:

| Block size | Test# | Computational complexity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | #multiplications | #additions | #shifts | #checks | Total number |
| 32 × 32 | 1.6.1 (IRSDB) | 88 | 290 | 56 | 17 | 451 |
|  | 1.6.2 (HEVC) | 253 | 514 | 128 | 5 | 900 |
| 32 × 64 | 1.6.1 (IRSDB) | 172 | 575 | 107 | 32 | 886 |
|  | 1.6.2 (HEVC) | 381 | 770 | 192 | 5 | 1348 |
| 64 × 64 | 1.6.1 (IRSDB) | 200 | 670 | 124 | 37 | 1031 |
|  | 1.6.2 (HEVC) | 509 | 1026 | 256 | 5 | 1796 |

## HEVC strong reference sample smoothing computational complexity analysis

- Deriving filter decision: 1 multiplication, 10 additions, 2 shifts, 5 checks

- Deriving modified sample:

  - For one modified sample: 2 multiplications, 4 additions, 1 shift

  - Number of modified samples: (2 × width – 1 + 2 × height - 1)

- Summery for each block size:

  - 32 × 32: 253 multiplications, 514 additions, 128 shifts, 5 checks

  - 32 × 64: 381 multiplications, 770 additions, 192 shifts, 5 checks

  - 64 × 64: 509 multiplications, 1026 additions, 256 shifts, 5 checks

## IRSDB computational complexity analysis

- Deriving tc and beta: 4 multiplications, 5 additions, 5 shifts, 2 checks

- Deriving filter decision:

  - For each checkpoint: 21 additions, 5 checks

  - Number of checkpoints:

    - 3 checkpoints for block size 32 × 32

    - 6 checkpoints for block size 32 × 64

    - 7 checkpoints for block size 64 × 64

- Deriving modified sample:

  - For each checkpoint:

    - Deriving P, Q and Middle: 18 additions, 3 shifts

    - Deriving one modified sample: 2 multiplications, 4 additions, 1 shift

    - Number of modified samples: 14

  - Number of checkpoints:

    - 3 checkpoints for block size 32 × 32

    - 6 checkpoints for block size 32 × 64

    - 7 checkpoints for block size 64 × 64

- Summery for each block size:

  - 32 × 32: 88 multiplications, 290 additions, 56 shifts, 17 checks

  - 32 × 64: 172 multiplications, 575 additions, 107 shifts, 32 checks

  - 64 × 64: 200 multiplications, 670 additions, 124 shifts, 37 checks

## Complexity analysis report

Complexity measures report according to [ref JVET-M1021] :

| Test# | filter shape | Computational complexity |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | #multiplications per sample of the block | #additions per sample of the block | #shifts per sample of the block | #checks per sample of the block |
| 1.6.1 | 2-taps | 0.086 | 0.283 | 0.055 | 0.017 |
| 1.6.2 | 2-taps | 0.247 | 0.502 | 0.125 | 0.005 |

| Test# | filter shape | Computational complexity |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | #multiplications per referece sample | #additions per referece sample | #shifts per reference sample | #checks per reference sample |
| 1.6.1 | 2-taps | 0.891 | 2.979 | 0.554 | 0.166 |
| 1.6.2 | 2-taps | 1.980 | 3.992 | 0.996 | 0.039 |

| Test# | Sequential operation to get one sample processed, e,g, S=A+B+C+D is 2 sequential additions | Operations before block processing | Precision of multiply | Parallel friendly (each sample can be filtered independently from other samples) |
| --- | --- | --- | --- | --- |
| 1.6.1 | For filter decision: 3 multi / 7 add / 2 shift / 3 check For filter operation: 1 multi / 16 add / 2 shift | Get tc, beta from LUT based on slice QP | 6×10 multiplication (6-bit filter coefficient + 10-bit sample) | Yes |
| 1.6.2 | For filter decision: 1 multi / 3 add / 1 shift / 3 check For filter operation: 1 multi / 3 add / 1 shift | None | 7×10 multiplication (7-bit filter coefficient + 10-bit sample) | Yes |

| Test# | Number of clock cycles | memory requirement | How to derive filter coeffs | Min. and max. CU Size for applying filters |
| --- | --- | --- | --- | --- |
| 1.6.1 | 2 | 7 values of 6-bit unsigned integer for filter coefficients | Derive from LUT | Width >= 32 and Height >= 32 |
| 1.6.2 | 2 | None | Linear combination of distance to corner samples. | Width >= 32 and Height >= 32 |

# Conclusions

This contribution reports the performance of intra reference sample deblocking and HEVC strong reference sample smoothing on VTM4.0.

Both CE1.6.1 (IRSDB) and CE1.6.2 (HEVC strong reference sample smoothing) have minor effect on BD-rate on average. However, it can be noted that there is a noticeable difference of about 0.2% for class A1 for AI and 0.1% RA in favour of IRSDB over HEVC strong reference sample smoothing.

The IRSDB has less computational complexity than the HEVC strong reference sample smoothing. The total number of computational operations (multiplications, additions, shits, checks) for IRSDB is 50%, 66% and 57% compared to the total number of operations for HEVC strong reference sample smoothing when the block size equals to 32 × 32, 32 × 64 and 64 × 64 respectively.

Both the CE1.6.1 (IRSDB) and the CE1.6.2 (HEVC strong reference sample smoothing) has better subjective performance than VTM4.0. We assert that CE1.6.1 (IRSDB) achieves better subjective performance than the CE1.6.2 (HEVC strong reference sample smoothing).

We recommend to adopt intra reference sample deblocking (IRSDB) for angular intra luma prediction to VVC and VTM to improve subjective performance of intra prediction of large blocks.

# References

[1]. JVET-M0138: Non-CE3: Intra reference sample deblocking

[2]. JVET-L0628: CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter

[3]. JVET-L0072: CE11: Long deblocking filters for luma (CE11.1.1) and for both luma and chroma (CE11.1.9)

[4]. JVET-M1021: Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).