| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0371 |
| --- | --- |

| Title: | Non-CE3: directional intra prediction with varying angle |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Fabrice Urban and Fabien Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes an intra prediction tool which considers multiple non-overlapping sub-partitions for the directional intra prediction of luma blocks, with slightly different directions, while keeping the entire parent block structure for transform and entropy coding. On top of VTM-4.0, the simulation results reportedly show luma BD-rate variation of -0.11% and -0.05% on average in All Intra (AI) and Random Access (RA) configuration.

# Introduction

In this contribution, intra predicted coding blocks with a directional mode can be split for the prediction step only, the resulting subblocks being assigned slightly different directions.

# Description

The proposed method enables the encoder to partition a luma coding block into two parts, for the prediction stage only. The direction of splitting is derived from the intra prediction direction of the parent coding block. If the direction is rather vertical, i.e. predMode >= DIA_IDX, the block is split horizontally (one top part and one bottom part). Else, if the direction is rather horizontal, i.e. predMode < DIA_IDX, the block is split vertically (one left part and one right part).

Then the directional prediction mode of the part 1, i.e. the top or left part, can have the value predMode+1 or predMode-1. The other part has prediction value predMode.

The encoder then signals the above decision using the following variable-length code:

- 0: no split

- 10: predMode-1

- 11: predMode+1.

In this contribution, the split is located at the fourth of the size of the parent block. The first part has ¼ of the block surface and the second part as ¾ of the parent block, as shown in Figure 1.

Notice that the target pixels in the second part are not predicted directly with their prediction mode, but also consider the prediction mode of the first part pixels, to keep the prediction consistent on the second part, i.e. bottom or right part. That is, the predictor for a target pixel in the second part will be determined based on both the prediction modes of the second part and the first part. This is shown in Figure 1, where the reference sample which is used as predictor for the depicted target pixel is derived considering the directions, with prediction mode values V-1 and V, over Part 1 and Part 2, respectively.

Part 1

Part 2

target pixel

predictor

V- 1

V

Figure 1: exemplary rectangular block with MRL index is 1, the rather vertical prediction is decreased by 1 for the top part

The proposed method does not require to reconstruct the partitions sequentially, i.e. the prediction of both parts relies on the same reference sample arrays which are those considered for the parent block. The prediction can then be performed at once.

# Interactions with Multi-Reference Line (MRL) and intra sub-partitions (ISP)

VTM-4.0 includes Intra Sub Partitions (ISP) as described in [1], which enable the encoder to further split a coding block, each sub partitions being coded independently but sharing the same intra prediction mode.

In this contribution, only the blocks which use the second or the fourth reference line, as per the MRL tool, are treated with the proposed prediction method. Therefore, if the current block has MRL index equal to 0, there is no encoding for mode partitioning. The decoder will assume a value of zero (no split for mode partitioning) if the decoded value of MRL index is 0. In this case, there is no conflict with ISP since ISP is applied only to the blocks having MRL index 0. Note that the MRL index is encoded before the ISP index in VTM-4.0. It is proposed to encode the mode partitioning index following the ISP index. The decoder thus decodes the MRL index first, then decodes the ISP index if MRL index is equal to zero or decodes the mode partitioning index otherwise.

# Simulation results

The following tables report the results of the simulations which were conducted using the VTM-4.0 reference software, under the common test conditions described in JVET-M1010 [2].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.01% | 111% | 101% |
| Class A2 | -0.08% | 0.03% | -0.05% | 108% | 99% |
| Class B | -0.16% | -0.12% | -0.07% | 108% | 100% |
| Class C | -0.12% | -0.10% | -0.03% | 109% | 101% |
| Class E | -0.17% | -0.07% | -0.12% | 111% | 102% |
| Overall | -0.11% | -0.05% | -0.05% | 109% | 100% |
| Class D | 0.01% | -0.09% | -0.18% | 108% | 100% |
| Class F | -0.07% | -0.01% | -0.15% | 106% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.01% | 105% | 101% |
| Class A2 | -0.11% | 0.12% | -0.05% | 104% | 99% |
| Class B | -0.05% | -0.05% | -0.16% | 101% | 97% |
| Class C | -0.04% | -0.18% | -0.32% | 95% | 91% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.05% | -0.15% | 101% | 97% |
| Class D | 0.03% | -0.21% | -0.41% | 93% | 90% |
| Class F | 0.01% | -0.31% | -0.32% | 101% | 104% |

# References

[1]	S. De-Luxán-Hernández, et al., “JVET-M0102 CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2).” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan-2019.

[2]	Franck Bossen, Jill Boyce, Xiang Li, Vadim Seregin, and Karsten Suehring, “JVET-M1010 JVET common test conditions and software reference configurations.” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan-2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).