| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0372-v1 |
| --- | --- |

| Title: | Non-CE3: ISP with independent sub-partitions for certain block sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Benjamin Bross, Tung Nguyen, Valeri George, Benno Stabernack, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of an extension of the Intra Sub-Partitions (ISP) coding mode to 4×4 blocks and a modification in the number of sub-partitions of 4×8 and 8×4 blocks. In order to keep the worst-case scenario throughput of 16 samples/cycle, the new sub-partitions are independent from each other. The experimental results show a gain of 0,10% and 0.47% for CTC and class F respectively for the AI configuration and a gain of 0,01% and 0.25% for CTC and class F respectively in the RA case. The impact on the encoding run-time is 102% in the AI case and 100% in the RA one.

Additional test results are provided on the effect of extending the proposed concept to all ISP blocks that produce sub-partitions with a width smaller than 4 on the current ISP design. The experimental results show a gain of 0.05% and 0.44% for CTC and class F respectively for the AI configuration and a gain of 0.24% on class F (no changes on CTC) in the RA case.

On the other hand, the impact of the complete removal of sub-partitions with a width smaller than 4 of the current ISP design has also been tested for comparison reasons. This additional information shows a loss of 0.14% and 0.31% for CTC and class F respectively for the AI configuration and a loss of 0.04% and 0.23% for CTC and class F respectively in the RA case.

All simulations have been run on top of VTM-4.0.1.

# Introduction

As presented in [1], the Intra Sub-Partitions (ISP) coding mode divides luma intra-predicted blocks vertically or horizontally into 2 or 4 equal-size sub-partitions according to the CU dimensions. Table I shows the different possibilities.

Table 1: Number of sub-partitions created by ISP according to the CU dimensions in the current VVC Draft

| Block size | Number of sub-partitions |
| --- | --- |
| 4×4 | Not divided |
| 4×8 and 8×4 | 2 |
| All other cases | 4 |

Each sub-partition is predicted, transformed, quantized and the entropy coded coefficients are sent to the decoder. Then, the reconstructed samples of a sub-partition are used to generate the prediction of the next sub-partition, but the intra mode used is shared among all sub-partitions.

Since this process lies in the intra-prediction critical path (in the general case a sub-partition cannot be decoded until the previous sub-partition reconstructed samples have been obtained), it is necessary to enforce a minimum of 16 samples in each sub-partition, which guarantees a worst-case scenario throughput of 16 samples/cycle. This is, in fact, the reason why 4×8 and 8×4 blocks have 2 sub-partitions (instead of 4) and 4×4 blocks cannot be sub-divided.

This contribution presents an expansion of ISP that allows its usage on 4×4 blocks and modifies the number of sub-partitions in the case of 4×8 and 8×4 blocks, while keeping the 16 samples/cycle throughput constraint. This goal can be achieved by producing sub-partitions that are independent from each other, that is, by not using the reconstructed samples of a sub-partition to the predict the next one.

# ISP extension to new block sizes

The modifications of ISP introduced in this contribution affect 4×4, 8×4 and 4×8 blocks.

## 4×4 blocks

The block is divided into four 4×1 (hor. split) or 1×4 (ver. split) sub-partitions. However, the prediction signal of each of them is generated using the neighbour samples of the CU boundaries as reference samples, as shown in Figure 1. Consequently, each sub-partition is independent from each other and all of them can be processed in parallel in single step.

Figure 1: Example of the vertical split of a 4×4 block. The reference samples used to generate each 1×4 prediction are only CU boundary samples. Note that the prediction is the same as for a non-ISP case.

Note that the prediction samples of all sub-partitions are the same as the ones that would be generated for a block not using ISP (excluding the reference samples and PDPC filtering operations that are disabled for all ISP blocks). The difference between a 4×4 block using ISP and one not using it lies in the transform (in the ISP case there are four 1-D transforms and in the non-ISP case there is a single 4×4 transform) and the entropy coding of the coefficients.

## 8×4 and 4×8 blocks

In the new ISP design the number of sub-partitions will be 4 instead of 2. However, the prediction signals of sub-partitions 2 and 4 cannot be generated using the reconstructed samples of sub-partitions 1 and 3 respectively, as shown in the examples of Figure 2 and Figure 3. Consequently, sub-partition 2 is independent from sub-partition 1 and analogously sub-partition 4 is independent from 3. Hence, a 4×8 or an 8×4 block could be processed in 2 cycles, which is equivalent to a throughout of 16 samples/cycle.

Figure 2: Example of the vertical split of a 4×8 block into four 1×8 sub-partitions. In this case the reconstructed samples from sub-partition 2 can be used to predict sub-partition 3, but the reconstructed samples from sub-partitions 1 and 3 cannot be used to predict 2 and 4 respectively.

Figure 3: Example of the horizontal split of a 4×8 block into four 4×2 sub-partitions. The reconstructed samples from sub-partitions 2 can be used to predict sub-partition 3, but the reconstructed samples from sub-partitions 1 and 3 cannot be used to predict 2 and 4 respectively.

# Experimental results

According to the common test conditions [2], the proposed method is evaluated for intra-only (AI), random-access (RA) and low-delay-b (LDB) configurations with the VTM-4.0.1 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler.

Table 1: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.05% | 102% | 100% |
| Class A2 | -0.01% | -0.03% | 0.02% | 101% | 101% |
| Class B | -0.05% | -0.05% | -0.13% | 102% | 101% |
| Class C | -0.22% | -0.14% | -0.20% | 102% | 103% |
| Class E | -0.19% | -0.13% | -0.11% | 102% | 100% |
| Overall | -0.10% | -0.07% | -0.09% | 102% | 101% |
| Class D | -0.22% | -0.20% | -0.22% | 102% | 103% |
| Class F | -0.47% | -0.44% | -0.32% | 102% | 103% |

Table 2: Results for the RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 100% | 100% |
| Class A2 | 0.01% | 0.08% | -0.07% | 100% | 100% |
| Class B | -0.01% | 0.10% | 0.00% | 100% | 100% |
| Class C | -0.04% | 0.11% | 0.15% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.08% | 0.04% | 100% | 100% |
| Class D | -0.10% | -0.20% | -0.07% | 100% | 100% |
| Class F | -0.25% | -0.07% | -0.23% | 100% | 100% |

Table 3: Results for the LDB configuration

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.10% | -0.43% | 100% | 100% |
| Class C | 0.03% | -0.15% | -0.26% | 100% | 102% |
| Class E | -0.04% | -1.21% | -1.07% | 99% | 100% |
| Overall | -0.01% | -0.31% | -0.53% | 100% | 101% |
| Class D | 0.01% | -0.44% | -0.19% | 101% | 100% |
| Class F | -0.04% | 0.22% | 0.22% | 100% | 101% |

# Additional information

## Usage of sub-partitions with a width smaller than 4

The usage of sub-partitions with a width smaller than 4 has been mentioned on the JVET reflector as a potential hardware concern, given that samples are typically allocated in a raster scanner fashion and certain implementations write the output of reconstructed samples in groups. In fact, typically a number of samples will be collected and written back to the external memory by a cache or buffer management component, which is part of the memory subsystem of state-of-art decoder implementations. This is important since DDR memories work with burst wise access rather than with access of small chunks of data to reach the highest throughput. Since this applies not only for the presented tool, a decoder architecture needs to take care of this behavior and has to provide a buffer functionality anyways, which can be used as well for rearranging pixel data into raster scan order when writing back.

In the following additional information is provided to evaluate the impact of the usage of these sub-partitions.

    - Removal of sub-partitions with a width smaller than 4

This modification affects the vertical split of 4×N and 8×N blocks as follows:

- 4×N: There is no vertical split anymore. Consequently, whenever ISP is used on one of these blocks, the decoder infers that a horizontal split is used and hence it is not necessary to parse the split flag syntax element.

- 8×N: The vertical split produces 2 sub-partitions instead of 4.

Note that this modified ISP approach was proposed as Test 1.1.2 in the CE3 of the January 2019 Marrakech Meeting in [1]. The results of this approach on top of VTM-4.0.1 are presented in Table 5, Table 6 and Table 7.

Table 4: Results without sub-partitions with a width smaller than 4 for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.03% | 100% | 99% |
| Class A2 | 0.01% | -0.04% | -0.01% | 99% | 102% |
| Class B | 0.06% | 0.08% | 0.09% | 99% | 100% |
| Class C | 0.21% | 0.21% | 0.18% | 99% | 100% |
| Class E | 0.42% | 0.35% | 0.26% | 99% | 98% |
| Overall | 0.14% | 0.12% | 0.10% | 99% | 100% |
| Class D | 0.25% | 0.28% | 0.12% | 99% | 100% |
| Class F | 0.31% | 0.16% | 0.22% | 100% | 100% |

Table 5: Results without sub-partitions with a width smaller than 4 for the RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.06% | 101% | 101% |
| Class A2 | 0.04% | -0.04% | 0.05% | 101% | 101% |
| Class B | 0.03% | 0.26% | 0.10% | 100% | 100% |
| Class C | 0.09% | 0.27% | 0.42% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.13% | 0.14% | 100% | 100% |
| Class D | 0.13% | 0.13% | -0.19% | 100% | 101% |
| Class F | 0.23% | 0.13% | 0.17% | 100% | 99% |

Table 6: Results without sub-partitions with a width smaller than 4 for the LDB configuration

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.22% | -0.36% | 100% | 100% |
| Class C | 0.00% | -0.26% | -0.20% | 100% | 102% |
| Class E | 0.03% | -0.84% | 0.29% | 100% | 101% |
| Overall | 0.02% | -0.20% | -0.14% | 100% | 101% |
| Class D | 0.01% | 0.69% | -0.58% | 100% | 100% |
| Class F | 0.14% | 0.42% | 0.17% | 101% | 102% |

### Extension of the ISP with independent sub-partitions approach to sub-partitions with a width smaller than 4

In this case sub-partitions with a width smaller than 4 are not removed from the ISP structure. Instead, the same design as the one introduced in this contribution for 4×4, 8×4 and 4×8 is applied to them. Two different cases are distinguished:

#### The vertical split of 4×N blocks is treated the same way as the vertical split of 4×4 explained in section 2.1. For example, a 4×32 block could be vertically divided into four independent 1×32 sub-partitions.

- The vertical split of 8×N blocks is treated the same way as the vertical split of 8×4 blocks explained in section 2.2. For instance, an 8×16 block could be divided into four 2×16 sub-partitions, where sub-partitions 2 and 4 could not use the reconstructed samples of sub-partitions 1 and 3 respectively to generate its corresponding prediction signals.

This method allows writing reconstructed samples in groups of at least 4×4 samples in all the vertical split cases (in the case of the horizontal split the minimum is still 16×1). The results of this approach (using independent sub-partitions on 4×4, 4×8, 8×4 and the vertical split of 4×N and 8×N blocks) on top of VTM-4.0.1 are presented in Table 7, Table 8 and Table 9.

Table 7: Results on using independent sub-partitions on 4×4, 4×8, 8×4 and the vertical split of 4×N and 8×N blocks for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.02% | 101% | 101% |
| Class A2 | 0.01% | -0.04% | 0.00% | 101% | 101% |
| Class B | -0.02% | 0.06% | -0.01% | 101% | 101% |
| Class C | -0.13% | -0.05% | 0.00% | 101% | 102% |
| Class E | -0.08% | -0.01% | -0.07% | 101% | 99% |
| Overall | -0.05% | 0.00% | -0.01% | 101% | 101% |
| Class D | -0.19% | -0.10% | -0.11% | 101% | 102% |
| Class F | -0.44% | -0.41% | -0.27% | 101% | 103% |

Table 8: Results on using independent sub-partitions on 4×4, 4×8, 8×4 and the vertical split of 4×N and 8×N blocks for the RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.09% | 101% | 100% |
| Class A2 | 0.02% | 0.15% | 0.01% | 101% | 101% |
| Class B | 0.00% | 0.22% | 0.16% | 100% | 99% |
| Class C | -0.04% | 0.20% | 0.32% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.14% | 0.16% | 100% | 100% |
| Class D | -0.04% | -0.03% | -0.23% | 100% | 100% |
| Class F | -0.24% | -0.18% | -0.36% | 100% | 100% |

Table 9: Results on using independent sub-partitions on 4×4, 4×8, 8×4 and the vertical split of 4×N and 8×N blocks for the LDB configuration

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.11% | -0.36% | 100% | 100% |
| Class C | -0.01% | -0.36% | -0.34% | 100% | 102% |
| Class E | -0.03% | -0.23% | 0.18% | 100% | 101% |
| Overall | -0.02% | -0.13% | -0.22% | 100% | 101% |
| Class D | 0.00% | 0.68% | -0.21% | 100% | 101% |
| Class F | -0.09% | 0.60% | -0.05% | 100% | 101% |

# Conclusion

The reported results show a gain in CTC and Class F with a very little (AI) or no impact (RA and LDB) in the encoding run-time.

Regarding the additional information presented, the experimental results show that the removal of the sub-partitions with a width smaller than 4 produces a noticeable loss, especially in the case of Classes C and E in AI and Class F in all configurations. Besides, this loss does not come with a significant reduction in the encoding run-time.

On the other hand, the extension of the new ISP design to blocks producing sub-partitions with a width smaller than 4 has a better performance than the complete removal of these sub-partitions in terms of BD-Rate gain, given that it does not produce any loss and it produces a significant gain in class F with a very small impact on the encoding run-time.

# References

| [1] | S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)," in Document JVET-M0102, Marrakech, MA, 2019. |
| --- | --- |
| [2] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. (. Sühring, "JVET common test conditions and software reference configurations for SDR video," Document JVET-M1010, Marrakech, MA, 2019. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).