| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 17–29 Mar. 2019 | Document: JVET-N0377 |
| --- | --- |

| Title: | CE2-5.1: Affine HMVP with modified affine inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: Tel: Email: | +1-650-798-3300 guichunli@tencent.com 1-201-816-2000 jie.zhao@lge.com |
| Source: | Tencent & LG Electronics |  |  |

_____________________________

# Abstract

This document reports results of CE2-5.1, “Affine HMVP with modified affine inheritance”. In affine HMVP, a table of affine HMVP candidates is maintained and updated similar to the HMVP table for regular merge candidates. In test CE2-5.1.a, the affine HMVP candidates are applied in inherited affine merge list generation to replace the original affine inheritance from spatial neighbours. In test CE2-5.1.b, affine inheritance from line buffer in current VVC is used in addition to CE2-5.1.a. It was asserted that in both subtests, the storage requirements for affine inheritance were reduced by 84.85%.

Result summaries are reported as follows:

- CE2-5.1.a: 0.08% on RA, 0.09% on LDB;

- CE2-5.1.b: 0.03% on RA, 0.05% on LDB.

# Introduction

Affine HMVP was previously proposed in JVET-M0125 [4] and JVET-M0432 [5]. A table to store affine motion information is updated after coding an affine coded CU, and the affine HMVP table is used in inherited affine merge candidate generation. The number of entries in affine HMVP table is 5. For each entry of affine HMVP table, the following data are stored:

Table 1.1 - Storage for each entry of Affine HMVP table

| Data Elements |  | Bits |
| --- | --- | --- |
| 3 Affine CPMVs for 2 Lists | 36*2*3 | 216 |
| Affine type | 1 | 1 |
| Position (x, y) | 5*2 | 10 |
| CU Width and height | 3*2 | 6 |
| Total |  | 233 |

For all 5 affine HMVP table entries, the total bits stored are 5 * 233 = 1165 bits.

In current VVC working draft, the optimized local buffer design for affine mode requires storing affine information for 1 row (16) and 1 column (16) of 8x8 blocks with 1 additional buffer for corner positions. The total required storage is (16 + 16 + 1) * 244 = 7689bits.

The percentage of required local buffer for affine blocks saved by the proposed affine HMVP method can be calculated as (7689– 1165 ) / 7689, which is a significant saving of 84.85%.

The proposed method replaced the spatial availability check by using affine information from the affine HMVP buffer. The affine HMVP buffer is updated after each affined coded CU, in the same way as the HMVP buffer for CUs coded in regular inter mode.

# Subtests

## Test CE2-5.1.a: Replace spatial affine inheritance with inheritance from affine HMVP

In test CE2-5.1.a, it was tested to replace the existing inherited affine merge candidates with candidates inherited from affine HMVP. The method reportedly reduced affine related memory storage significantly. Due to the fact that the affine HMVP buffer is reset at the beginning of each CTU row, the affine motion information from blocks above current CTU will not be used in affine HMVP. The number of inherited affine merge candidates from the HMVP buffer is limited to 2.

Table 2.1 - Complexity analysis of CE2-5.1.a

| Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer for (bits per 8x8) | Additional logics |
| --- | --- | --- |
| 1165 bits | 0 | Affine HMVP table update |

## Test CE2-5.1.b: Test 2.5.1a + affine inheritance from line buffer

In test CE2-5.1.b, a combined method is tested to keep affine inheritance from affine HMVP as in CE2-5.1.a, and the original affine inheritance from motion data line buffer when neighboring candidate is above the current CTU boundary. This combination can keep the benefit of low memory requirement the same as in CE2-5.1.b for affine model inheritance from affine HMVP, while utilizing the available affine motion models from above CTU. When the current block is located at the top of a CTU, the total number of inherited affine merge candidates is limited to 2, otherwise, inheritance from affine HMVP buffer is limited to 2.

Table 2.2 - Complexity analysis of CE2-5.1.b

| Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer for (bits per 8x8) | Additional logics |
| --- | --- | --- |
| 1165 bits | no change | Affine HMVP table update |

# Experimental Results

The proposed combined methods were implemented based on VTM-4.0, and were tested following common test conditions [2].

Table 3.1 Test CE2-5.1.a Results

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | -0.03% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.11% | 0.04% | 0.12% | 100% | 100% |  |  |  |  |  |
| Class B | 0.08% | -0.08% | 0.11% | 100% | 99% | 0.14% | 0.21% | -0.20% | 100% | 97% |
| Class C | 0.09% | 0.08% | -0.14% | 100% | 101% | 0.07% | 0.26% | 0.02% | 100% | 98% |
| Class E |  |  |  |  |  | 0.01% | -0.30% | -0.55% | 101% | 98% |
| Overall | 0.08% | 0.00% | 0.02% | 100% | 100% | 0.09% | 0.10% | -0.22% | 100% | 98% |
| Class D | 0.13% | 0.16% | 0.03% | 100% | 101% | 0.28% | -0.20% | -0.75% | 100% | 99% |
| Class F | 0.03% | 0.06% | -0.06% | 100% | 100% | -0.02% | -0.38% | 0.48% | 99% | 99% |

Table 3.2 Test CE2-5.1.b Results

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | 0.04% | 99% | 100% |  |  |  |  |  |
| Class A2 | 0.02% | -0.11% | 0.05% | 100% | 99% |  |  |  |  |  |
| Class B | 0.03% | -0.04% | -0.02% | 99% | 99% | 0.09% | -0.03% | -0.04% | 100% | 97% |
| Class C | 0.06% | 0.07% | -0.13% | 100% | 101% | 0.03% | 0.21% | 0.18% | 99% | 99% |
| Class E |  |  |  |  |  | 0.03% | -0.03% | 0.16% | 99% | 99% |
| Overall | 0.03% | 0.00% | -0.02% | 99% | 100% | 0.05% | 0.05% | 0.08% | 100% | 98% |
| Class D | 0.12% | 0.10% | 0.02% | 100% | 102% | 0.22% | 0.05% | -0.27% | 100% | 101% |
| Class F | 0.00% | 0.04% | -0.07% | 100% | 101% | 0.01% | -0.19% | -0.06% | 101% | 101% |

# Conclusion

It was observed that in CE2-5.1.a, affine model inheritance only from affine HMVP, significantly reduced local storage requirement and removed the requirement of motion data line buffer for affine, with a small coding performance impact. Compared to VTM-4.0, the BD-rate changes are 0.08% and 0.09% on RA and LDB, respectively.

The test CE2-5.1.b, affine model inheritance from affine HMVP and motion data line buffer, keeps the reduction of local memory requirement the same as in CE2-5.1.a, while further reduces the coding performance impact. Compared to VTM-4.0, the BD-rate changes are 0.03% and 0.05% on RA and LDB, respectively.

It is proposed to adopt CE2-5.1.b to VVC and VTM.

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVT-M1001, Marrakech, MA, Jan 2019.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-M1010 Marrakech, MA, Jan 2019.

- C.-C. Chen, Y. He, H. Liu, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction”, JVT-M1022, Marrakech, MA, Jan 2019.

- J. Zhao, S. Kim, G. Li, X. Xu, X. Li, S. Liu, “CE2: History Based Affine Motion Candidate (Test 2.2.3)”, Document of Joint Video Experts Team (JVET), JVET-M0125, Marrakech, MA, Jan 2019.

- G. Li, X. Xu, X. Li, S. Liu, J. Zhao, S. Kim, “CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer”, Document of Joint Video Experts Team (JVET), JVET-M0432, Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).