| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0380 |
| --- | --- |

| Title: | CE4-related: Fix of MMVD signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiang Li, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it was proposed to fix an issue in MMVD signalling, i.e., MMVD base index is inferred to be equal to 0 without signalling when maximum allowed number of merge candidates is set to 1. Simulations were performed with VTM-4.0.1 as anchor. The proposed change has no impact under CTC test conditions (maximum number of merge candidates > 1). When maximum number of merge candidates is set to 1, it was observed that BD-rate change is 0.02% with 93% encoding time for RA, and BD-rate change is -0.16% and 97% encoding times for LB.

# Introduction

In current VVC working draft and VTM-4.0.1, merge with motion vector difference (MMVD) is used. MMVD can use up to 2 merge candidates as base merge candidates. And the base merge candidate index is signalled in bitstream with syntax element “mmvd_merge_flag”.

Below is the description of the semantic of “mmvd_merge_flag” which is the index for base candidates, as described in 7.4.7.8 of current working draft of VVC [1]:

“mmvd_merge_flag[ x0 ][ y0 ] specifies whether the first (0) or the second (1) candidate in the merging candidate list is used with the motion vector difference derived from mmvd_distance_idx[ x0 ][ y0 ] and mmvd_direction_idx[ x0 ][ y0 ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.”

There’s a potential issue. When the maximum number of merge candidate is set to 1, the second candidate in the merging candidate list does not exist, thus the second base candidate of MMVD becomes undefined. In current software implementation VTM-4.0.1, the second base candidate of MMVD is always generated, and when the second merging candidate does not exist, zero MVs will be used. This does not align with the working draft text.

# Proposed method

It is proposed to remove the zero MV filling on the second MMVD base candidate when the second merging candidate is not available, and to use conditional signalling of “mmvd_merge_flag”, depending on the maximum number of merging candidate used. When MMVD is used (mmvd_flag is true) and the maximum number of merging candidate is equal to 1, “mmvd_merge_flag” is not signalled and inferred to be equal to 0.

# Experimental Results

Proposed method is implemented based on VTM-4.0.1. Test was conducted with maximum number of merge candidates set to 1, all other settings are the same as CTC[2].

Table 3.1 Simulation Result

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | -0.05% | 93% | 99% |  |  |  |  |  |
| Class A2 | 0.05% | -0.05% | 0.02% | 94% | 100% |  |  |  |  |  |
| Class B | 0.01% | -0.03% | -0.03% | 93% | 100% | -0.09% | -0.21% | 0.00% | 96% | 97% |
| Class C | 0.03% | 0.09% | 0.11% | 94% | 101% | -0.19% | -0.33% | -0.24% | 98% | 100% |
| Class E |  |  |  |  |  | -0.24% | -0.68% | -1.08% | 97% | 97% |
| Overall | 0.02% | 0.00% | 0.01% | 93% | 100% | -0.16% | -0.37% | -0.35% | 97% | 98% |
| Class D | 0.12% | 0.08% | 0.04% | 94% | 99% | -0.16% | -0.10% | 0.14% | 100% | 105% |
| Class F | -0.01% | 0.04% | 0.09% | 93% | 100% | -0.06% | 0.21% | -0.15% | 99% | 103% |

It can be observed from Table 3.1 that the proposed method shows similar coding efficiency in RA with about 7% encoding time reduction and 0.16% BD-rate gain for LDB with about 3% encoding time reduction when compared to VTM-4.0.1 with max number of merge candidates as 1. The time saving is mainly from less MMVD candidates search during encoding. The coding gain for LDB is from the signalling bit saving of mmvd_merge_flag.

# Working Draft Text Changes

The changes based on JVET-M1001-v6 are highlighted below.

7.3.6.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| … |  |

7.4.7.7	Merge data semantics

…

mmvd_merge_flag[ x0 ][ y0 ] specifies whether the first (0) or the second (1) candidate in the merging candidate list is used with the motion vector difference derived from mmvd_distance_idx[ x0 ][ y0 ] and mmvd_direction_idx[ x0 ][ y0 ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mmvd_distance_idx[ x0 ][ y0 ] specifies the index used to derive MmvdDistance[ x0 ][ y0 ] as specified in Table 7-11. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

…

# Conclusion

It was observed that the proposed fix on MMVD signalling doesn’t have impact on BD-rate performance in common test conditions. When max number of merging candidate is set to 1, the proposed method has 0.02% BD-rate change on Random Access configuration with 93% encoding time, and -0.16% BD-rate change on Low Delay B configuration with 97% encoding time.

It is proposed to adopt the proposed fix on MMVD signalling into VVC and VTM.

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVT-M1001, Marrakech, MA, Jan 2019.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-M1010 Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).