| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0381 |
| --- | --- |

| Title: | Native CABAC initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharplabs.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

A new arithmetic coding engine was adopted at the 13th JVET meeting. To derive the initial state for each context, a state variable is first computed according to the same rules used for the AVC and HEVC engines. This state variable is then mapped to reflect the state representation of the new engine. It is proposed to have a “native” initialization process that doesn’t require such mapping.

# Problem Statement

A new arithmetic coding engine was adopted at the 13th JVET meeting. To derive the initial state for each context, a state variable is first computed according to the same rules as for the AVC and HEVC engines. This state variable is then mapped to reflect the state representation of the new engine. This mapping is achieved with the use of a 256-entry lookup table. Such a large lookup is unnecessary and undesirable.

# Proposed solution

It is proposed to change the initialization process to:

From the 8 bit table entry initValue, two 4-bit variables slopeIdx and offsetIdx are derived as follows:

slopeIdx = initValue  >>  4

offsetIdx = initValue & 15

The variables m and n, used in the initialization of context variables, are derived from slopeIdx and offsetIdx as follows:

m = slopeIdx - 8

n = ( offsetIdx  - 8 ) * 25

The variable preCtxStateIdx is derived from TileGroupQpY, which is derived as provided above,  as follows:

preCtxStateIdx = m * Clip3( 0, 51, TileGroupQpY) + n

mps = preCtxStateIdx < 0  ? 0 : 1

preCtxStateIdx = 3 * min(255, mps ? preCtxStateIdx  : –preCtxStateIdx-1)

state = ((255 – (   preCtxStateIdx & 127)) * 32) >> (preCtxStateIdx    >> 7)

state = mps ? 16383 – p : p

The values of pStateL and pStateH are derived as follows:

pStateL = state >> 4

pStateH = state

## Results

BD rate results show that the proposed initialization method doesn’t have a significant impact on coding efficiency.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.02% | 0.09% | -0.06% | -0.02% | 100% | 98% |
| Class A2 | -0.02% | 0.15% | -0.04% | -0.02% | 98% | 97% |
| Class B | -0.01% | -0.09% | 0.01% | -0.02% | 100% | 101% |
| Class C | 0.01% | -0.09% | -0.09% | -0.01% | 102% | 101% |
| Class E | -0.02% | -0.03% | -0.02% | -0.02% | 99% | 100% |
| Overall | -0.01% | -0.01% | -0.04% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.05% | -0.23% | -0.01% | 101% | 100% |
| Class F | 0.03% | -0.11% | -0.25% | 0.01% | 100% | 99% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | -0.07% | -0.06% | -0.03% | 101% | 98% |
| Class A2 | -0.03% | 0.13% | -0.02% | -0.02% | 98% | 98% |
| Class B | -0.02% | 0.01% | -0.10% | -0.02% | 100% | 100% |
| Class C | -0.04% | -0.15% | -0.23% | -0.06% | 98% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.11% | -0.03% | 99% | 99% |
| Class D | -0.03% | -0.40% | -0.28% | -0.05% | 99% | 99% |
| Class F | -0.01% | -0.18% | -0.14% | -0.02% | 99% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | -0.35% | -0.58% | -0.08% | 97% | 98% |
| Class C | -0.13% | -0.02% | -0.42% | -0.14% | 99% | 97% |
| Class E | -0.16% | 1.20% | 0.53% | -0.06% | 100% | 99% |
| Overall | -0.10% | 0.15% | -0.25% | -0.10% | 98% | 98% |
| Class D | -0.09% | 0.29% | -0.97% | -0.11% | 100% | 102% |
| Class F | -0.14% | 0.21% | -0.24% | -0.13% | 98% | 99% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | -0.25% | -0.72% | -0.07% | 98% | 98% |
| Class C | -0.06% | -0.58% | -0.81% | -0.13% | 101% | 101% |
| Class E | -0.12% | 1.16% | 1.24% | 0.01% | 101% | 102% |
| Overall | -0.07% | -0.01% | -0.26% | -0.07% | 100% | 100% |
| Class D | 0.00% | 0.96% | 0.50% | 0.02% | 100% | 102% |
| Class F | -0.05% | -0.43% | -0.40% | -0.03% | 99% | 100% |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).