| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0382-v1 |
| --- | --- |

| Title: | CE8-related: Unified intra block copy block vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a unified block vector prediction list is constructed for both merge mode and AMVP mode in intra block copy mode. It is asserted to be simpler than either of the two modes in their original processes.

Simulation results report that, by using a unified candidate list for both merge and AMVP mode in IBC:

- -0.05%/0.00% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -0.13%/0.06% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

Simulation results report that, by using the proposed simpiflied list for merge mode in IBC:

- -0.02%/0.00% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -0.12%/-0.13% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

# Problem Statement

In current VVC specification, intra block copy mode is separate from inter mode. For that purpose, a separate merge candidate list and separate BV predictor list (AMVP mode) for IBC will be used. Both lists are formed only by block vectors.

Currently, the merge candiate list for IBC consists of the following elements in order:

- 5 spatial neigboring positions

- 5 HMVP entries

- 1 pairewise average

Note that when IBC merge candidate list is not full, there is no default candiates to fill up the entries in the list, which potentially will cause problems in implementations.

The AMVP predictor list for IBC, which is not the same as the merge candidate list, consists of the following elements:

- First avaliable candidate from left two neighbors

- First available candiate from top three neighbors

- 5 HMVP entries

- Up to two zero candidates (invalid)

Note also that, even when shared merge list region is applied, for each of small blocks coded in AMVP mode, different BV predictor list for each block should still be constructed.

It is desirable to have a simplified BV predictor derivation process for IBC mode.

# Proposed method

In this proposal, the BV predictors for merge mode and AMVP mode in IBC will share a common predictor list, which consist of the following elements:

- 2 spatial neigboring positions (A0, B0 as in Fig. 1)

- 5 HMVP entries

- Default vectors{(-2w, 0), (0, -2h), (-w, 0), (0, -h), (-2w, -2h), (-w, -h)} (w, h refers to the width and height of current block)

For merge mode, up to first 6 entries of this list will be used; for AMVP mode, the first 2 entries of this list will be used. And the list conforms with the shared merge list region requirement (shared the same list within the SMR).

Fig. 1: Five spatial neighboring candidates

# Simulation results

The proposed method is implemented on top of VTM-4.0.1. Simulation is performed using the common test condition as defined in JVET-M1010. Only Class F and Class TGM are tested as IBC is turned off for all other sequences in the CTC.

Table I: unified merge and amvp candidate list for IBC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.05% | -0.12% | -0.04% | 98% | 98% |
| Class SCC 1080p | -0.13% | -0.10% | -0.14% | 97% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | 0.00% | -0.04% | -0.14% | 99% | 97% |
| Class SCC 1080p | 0.06% | 0.02% | 0.01% | 100% | 100% |

Table II: simplified merge list for IBC (AMVP mode unchanged)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.02% | -0.10% | -0.01% | 102% | 102% |
| Class SCC 1080p | -0.12% | -0.10% | -0.10% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | 0.00% | -0.16% | -0.18% | 102% | 102% |
| Class SCC 1080p | -0.13% | -0.09% | -0.13% | 102% | 101% |

There appears to be some issue with the behaviour of an encoder optimization adoption from the last meeting (hash based ME). By turning off this tool, the results are shown as follows:

Table III: unified merge and amvp candidate list for IBC (HashME=0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.05% | -0.12% | -0.04% | 97% | 95% |
| Class SCC 1080p | -0.13% | -0.10% | -0.14% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.02% | -0.08% | -0.15% | 101% | 101% |
| Class SCC 1080p | -0.14% | -0.17% | -0.07% | 100% | 100% |

Table VI: simplified merge list for IBC (AMVP mode unchanged, HashME=0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.02% | -0.10% | -0.01% | 97% | 95% |
| Class SCC 1080p | -0.12% | -0.10% | -0.10% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.05% | -0.22% | -0.23% | 101% | 100% |
| Class SCC 1080p | -0.14% | -0.16% | -0.09% | 100% | 99% |

# Conclusion

In this proposal, a simplified BV prediction list is proposed. Simulation results show minor coding performance changes for the proposed method, either when applied to merge mode only, or to both merge and AMVP predictor candidate list. It is recommended that methods in this proposal be adopted into next VVC working draft and VTM software release.

# Specification Text

Based on JVET-M1001-v7.

TBA

# References

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).