| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0382-v2 |
| --- | --- |

| Title: | CE8-related: Unified intra block copy block vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a unified block vector prediction list is constructed for both merge mode and AMVP mode in intra block copy mode. It is asserted to be simpler than either of the two modes in their original processes.

Simulation results report that, by using a unified candidate list for both merge and AMVP mode in IBC:

- -0.05%/-0.03%/-0.09% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-4.0.1 anchor.

- -0.13%/-0.11%/-0.15% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-4.0.1 anchor.

Simulation results report that, by using the proposed simpiflied list for merge mode in IBC:

- -0.02%/-0.06%/-0.17% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-4.0.1 anchor.

- -0.12%/-0.11%/-0.08% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-4.0.1 anchor.

# Problem Statement

In current VVC specification, intra block copy mode is separate from inter mode. For that purpose, a separate merge candidate list and separate BV predictor list (AMVP mode) for IBC will be used. Both lists are formed only by block vectors.

Currently, the merge candiate list for IBC consists of the following elements in order:

- 5 spatial neighboring positions

- 5 HMVP entries

- 1 pairwise average

Note that when IBC merge candidate list is not full, there is no default candiates to fill up the entries in the list, which potentially will cause problems in implementations.

The AMVP predictor list for IBC, which is not the same as the merge candidate list, consists of the following elements:

- First available candidate from left two neighbors

- First available candidate from top three neighbors

- 5 HMVP entries

- Up to two zero candidates (invalid)

Note also that, even when shared merge list region is applied, for each of small blocks coded in AMVP mode, different BV predictor list for each block should still be constructed.

It is desirable to have a simplified BV predictor derivation process for IBC mode.

# Proposed method

In this proposal, the BV predictors for merge mode and AMVP mode in IBC will share a common predictor list, which consist of the following elements:

- 2 spatial neighboring positions (A1, B1 as in Fig. 1)

- 5 HMVP entries

- Default vectors{(-2w, 0), (0, -2h), (-w, 0), (0, -h), (-2w, -2h), (-w, -h)} (w, h refers to the width and height of current block)

For merge mode, up to first 6 entries of this list will be used; for AMVP mode, the first 2 entries of this list will be used. And the list conforms with the shared merge list region requirement (shared the same list within the SMR).

Fig. 1: Five spatial neighboring candidates

# Simulation results

The proposed method is implemented on top of VTM-4.0.1. Simulation is performed using the common test condition as defined in JVET-M1010. Only Class F and Class TGM are tested as IBC is turned off for all other sequences in the CTC.
 Bug fixes related to HashME search in ticket #222 are applied to both anchor and tested data.

Table I: unified merge and amvp candidate list for IBC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.12% | -0.04% | 98% | 101% |
| Class TGM | -0.13% | -0.10% | -0.14% | 98% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.03% | -0.05% | -0.16% | 101% | 101% |
| Class TGM | -0.11% | -0.13% | -0.14% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | -0.01% | 0.54% | 101% | 98% |
| Class TGM | -0.15% | -0.10% | -0.17% | 100% | 100% |

Table II: simplified merge list for IBC (AMVP mode unchanged)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | -0.10% | -0.01% | 102% | 102% |
| Class TGM | -0.12% | -0.10% | -0.10% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.19% | -0.25% | 102% | 101% |
| Class TGM | -0.11% | -0.07% | -0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM401 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.17% | -0.19% | 0.46% | 102% | 100% |
| Class TGM | -0.08% | -0.14% | -0.18% | 99% | 101% |

# Conclusion

In this proposal, a simplified BV prediction list is proposed. Simulation results show minor coding performance changes for the proposed method, either when applied to merge mode only, or to both merge and AMVP predictor candidate list. It is recommended that methods in this proposal be adopted into next VVC working draft and VTM software release.

# Specification Text

Based on JVET-M1001-v6.

### Derivation process for motion vector components for IBC blocks

#### 8.6.2.1 General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vector in 1/16 fractional-sample accuracy mvL.

The luma motion vector mvL is derived as follows:

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vector prediction s for merge mode as specified in clause 8.6.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vector mvL.

- Otherwise, the following applies:

- The variable mvd is derived as follows:

mvd[ 0 ] = MvdL0[ xCb ][ yCb ][ 0 ]		(8-967)

mvd[ 1 ] = MvdL0[ xCb ][ yCb ][ 1 ]		(8-210)

- The derivation process for luma motion vector prediction in clause 8.6.2.6 8.6.2.2 is invoked with the luma coding block location ( xCb, yCb ), the coding block width cbWidth and the coding block height cbHeight as inputs, and the output being mvp.

……

#### 8.6.2.2 Derivation process for luma motion vector for motion vector prediction merge mode

……

- The derivation process for motion vector prediction merging candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the outputs being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 and the motion vectors mvA0, mvA1, mvB0, and mvB1 and mvB2.

- The merging motion vector candidate list, mergeMvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeMvCandList [ i++ ] = mvA1
if( availableFlagB1 )
	mergeMvCandList [ i++ ] = mvB1
if( availableFlagB0 ) 		(8-976)
	mergeMvCandList [ i++ ] = mvB0
if( availableFlagA0 )
	mergeMvCandList [ i++ ] = mvA0
if( availableFlagB2 )
	mergeMvCandList [ i++ ] = mvB2

……

- When numCurrMergeCand is less than MaxNumMergeCand, the derivation process for default motion vector prediction candidates specified in clause 8.6.2.x is invoked with the mergeMvCandList, the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight, and numCurrMergeCand as inputs, and the output is assigned to mergeMvCandList.

- The following assignments are made:

- If merge_flag[ xCb ][ yCb ] is equal to 1,

mvL[ 0 ] = mergeMvCandList[ merge_idx[ xCb ][ yCb ] ][ 0 ]	(8-977)

mvL[ 1 ] = mergeMvCandList[ merge_idx[ xCb ][ yCb ] ][ 1 ]	(8-978)

- Otherwise ( merge_flag[ xCb ][ yCb ] is equal to 0 ),

mvL[ 0 ] = mergeMvCandList[ mvp_l0_flag[ xCb ][ yCb ] ][ 0 ]	(8-977)

mvL[ 1 ] = mergeMvCandList[ mvp_l0_flag[ xCb ][ yCb ] ][ 1 ]	(8-978)

#### 8.6.2.x Derivation process for default motion vector prediction candidates

Inputs to this process are:

- a motion vector candidate list mergeCandList,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the modified motion vector candidate list mergeCandList.

The variable dMvpIdx is set equal to 0.

The array defaultMvCandidate is defined as follows:

defaultMvCandidate[ 0 ][ 0 ] = −2 * ( cbWidth << 4 )	(8-987)

defaultMvCandidate[ 0 ][ 1 ] = 0		(8-987)

defaultMvCandidate[ 1 ][ 0 ] = −2 * ( cbHeight << 4 )	(8-987)

defaultMvCandidate[ 1 ][ 1 ] = 0		(8-987)

defaultMvCandidate[ 2 ][ 0 ] = −1 * ( cbWidth << 4 )	(8-987)

defaultMvCandidate[ 2 ][ 1 ] = 0		(8-987)

defaultMvCandidate[ 3 ][ 0 ] = −1 * ( cbHeight  << 4 )	(8-987)

defaultMvCandidate[ 3 ][ 1 ] = 0		(8-987)

defaultMvCandidate[ 4 ][ 0 ] = −2 * ( cbWidth << 4 )	(8-987)

defaultMvCandidate[ 4 ][ 1 ] = −2 * ( cbHeight << 4 )	(8-987)

defaultMvCandidate[ 5 ][ 0 ] = −1 * ( cbWidth << 4 )	(8-987)

defaultMvCandidate[ 5 ][ 1 ] = −1 * ( cbHeight << 4 )	(8-987)

The following assignment is made repeatedly until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

mergeCandList[ numCurrMergeCand++ ] = defaultMvCandidate[ dMvpIdx++ ]	(8-987)

#### 8.6.2.3 Derivation process for spatial motion vector prediction merging candidates

……

For the derivation of availableFlagB0 and mvB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0 and mvB0 are derived as follows:

  - If one or more of the following conditions are true, availableFlagB0 is set equal to 0 and both components of   mvB0 are set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvB0 = MvL0[ xNbB0 ][ yNbB0 ]		(8-981)

For the derivation of availableFlagA0 and mvA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0 and mvA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0 and both components of  mvA0 are set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvA0 = MvL0[ xNbA0 ][ yNbA0 ]		(8-982)

For the derivation of availableFlagB2 and mvB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2 and mvB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0 and both components of  mvB2 are set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB0 is equal to TRUE and the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableA0 is equal to TRUE and the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvB2 = MvL0[ xNbB2 ][ yNbB2 ]		(8-983)

#### 8.6.2.4 Derivation process for pairwise average merging candidate

Inputs to this process are:

- a merging motion vector candidate list mergeMvCandList,

- the number of elements numCurrMergeCand within mergeMvCandList.

Outputs of this process are:

- the merging motion vector candidate list mergeMvCandList,

- the number of elements numCurrMergeCand within mergeCandList.

The candidate mvAvgCand is derived as follows and numCurrMergeCand is incremented by 1:

mvAvgCand[ 0 ] = ( ( mergeMvCandList[ 0 ][ 0 ] + mergeMvCandList[ 1 ][ 0 ] ) / 32 ) << 4	(8-984)

mvAvgCand[ 1 ] = ( ( mergeMvCandList[ 0 ][ 1 ] + mergeMvCandList[ 1 ][ 1 ] ) / 32 ) << 4	(8-985)

The candidate mvAvgCand is added at the end of mergeMvCandList and numCurrMergeCand is incremented by 1 as follows:

mergeMvCandList[ numCurrMergeCand++ ] = mvAvgCand	(8-986)

#### 8.6.2.6 Derivation process for luma motion vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

Output of this process is the prediction mvp in 1/16 fractional-sample accuracy of the motion vector mv.

The motion vector predictor mvp is derived in the following ordered steps:

- The derivation process for motion vector predictor candidate list as specified in clause 8.6.2.7 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the motion vector predictor candidate list mvpList as output.

- The motion vector predictor mvp is derived as follows:

mvp = mvpList[ mvp_l0_flag[ xCb ][ yCb ] ]		(8-988)

#### 8.6.2.7 Derivation process for motion vector predictor candidate list

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

Output of this process is motion vector predictor candidate list mvpList in 1/16 fractional-sample accuracy.

The motion vector predictor candidate list mvpList is derived in the following ordered steps:

- The derivation process for spatial motion vector predictor candidates from neighbouring coding unit partitions as specified in clause 8.6.2.8 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the availability flags availableFlagN and the motion vectors mvN, with N being replaced by A or B, as outputs.

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvN, with N being replaced by A or B, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvN, with N being replaced by A or B, as outputs.

- The motion vector predictor candidate list, mvpList, is constructed as follows:

numCurrMvpCand = 0
if( availableFlagA ) {
	mvpList[ numCurrMvpCand++ ] = mvA
	if( availableFlagB  &&  ( mvA  !=  mvB ) )
		mvpList[ numCurrMvpCand++ ] = mvB		(8-989)
} else if( availableFlagB )
	mvpList[ numCurrMvpCand++ ] = mvB

- When numCurrMvpCand is less than 2 and NumHmvpIbcCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpIbcCand ) until numCurrMvpCand is equal to 2:

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to the motion vector of the candidate HmvpIbcCandList[ i − 1 ], rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded motion vector of the candidate HmvpIbcCandList[ i − 1 ] as output is assigned to mvpList[ numCurrMvpCand++ ].

- When numCurrMvpCand is less than 2, the following applies for until numCurrMvpCand is equal to 2:

- mvpList[ numCurrMvpCand ][ 0 ] is set equal to 0.

- mvpList[ numCurrMvpCand ][ 1 ] is set equal to 0.

- numCurrMvpCand is increased by 1.

#### 8.6.2.8 Derivation process for spatial motion vector predictor candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

Outputs of this process are (with N being replaced by A or B):

- the motion vectors mvN in 1/16 fractional-sample accuracy of the neighbouring coding units,

- the availability flags availableFlagN of the neighbouring coding units.

Figure 8-2 provides an overview of spatial motion vector neighbours.

The variable currCb specifies the current luma coding block at luma location ( xCb, yCb ) and the variable currPic specifies the current picture.

The motion vector mvA and the availability flag availableFlagA are derived in the following ordered steps:

- The sample location ( xNbA0, yNbA0 ) is set equal to ( xCb − 1, yCb + cbHeight ) and the sample location ( xNbA1, yNbA1 ) is set equal to ( xNbA0, yNbA0 − 1 ).

- The availability flag availableFlagA is set equal to 0 and both components of mvA are set equal to 0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- The following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- When availableAk is equal to TRUE and availableFlagA is equal to 0, the following applies:

- If CuPredMode[ xNbAk ][ yNbAk ] is equal to MODE_IBC, availableFlagA is set equal to 1 and the following applies:

mvA = MvL0[ xNbAk ][ yNbAk ]	(8-990)

The motion vector mvB and the availability flag availableFlagB are derived in the following ordered steps:

- The sample locations ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) are set equal to ( xCb + cbWidth, yCb − 1 ), ( xCb + cbWidth − 1, yCb − 1 ) and ( xCb − 1, yCb − 1 ), respectively.

- The availability flag availableFlagB is set equal to 0 and the both components of mvB are set equal to 0.

- The following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagB is equal to 0, the following applies:

- If CuPredMode[ xNbBk ][ yNbBk ] is equal to MODE_IBC, availableFlagB is set equal to 1 and the following assignments are made:

mvB = MvL0[ xNbBk ][ yNbBk ]	(8-991)

# References

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).