| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0383-v1 |
| --- | --- |

| Title: | Non-CE8: IBC search range adjustment for implementation consideration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a modification is made on intra block copy search range when the current CU is the first CU of each 64x64 region (VPDU). The current specification allows the reference block comes from the collocated 64x64 region in the left CTU for IBC prediction, only for the first CU of each VPDU, but not for all other subsequent CUs in the region. It is asserted that allowing this corner case is not implementation friendly. The proposal in this document is to disallow such a case by adding a bitstream conformance restriction.

Simulation results report that by disallowing the first CU of each VPDU to refer to the collocated VPDU region in the left CTU:

- 0.07%/0.03%/-0.15% BD rate changes are observed for AI/RA/LB configurations for class F when compared to VTM-4.0 anchor.

- 0.22%/0.07%/0.14% BD rate changes are observed for AI/RA/LB configurations for class TGM when compared to VTM-4.0 anchor.

# Problem Statement

Currently, the current specification allows the reference block comes from the collocated 64x64 region in the left CTU for IBC prediction, only for the first CU of each VPDU, but not for all other subsequent CUs in the region. It is asserted that allowing this corner case is not implementation friendly.

In the example of Fig. 1, for all coding blocks in VPDU7, the reference block can be from areas of VPDU4, VPDU5, VPDU6 and recontructd part of VPDU7. In addition, if the block is the first coding block of VPDU7, reference area of VPDU3 can also be used.

Fig. 1: Current VPDU (7) and its collocated VPDU (3) in the left CTU

For the first coding block of each 64x64 coding region (VPDU7 in Fig. 1 as example), if its reference block comes from the colocated 64x64 region of the left CTU (VPDU3 in Fig. 1 as example), it is asserted that this could have implementation complications. The reason is that the memory used to store reference samples of the collocated 64x64 region will be at the same time the memory to store reconstucted samples of the current 64x64 coding region. Reading and writing on the same memory location is not preferable.

# Proposed method

In this proposal, one additional conformance check is added such that for all CUs in the current VPDU, the collocated VPDU in the left CTU cannot be used for IBC reference purpose.

Below is the specification change (highlighted in yellow):

  - When ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) – 1, the following conditions shall be true:

    - the derivation process for block availability as specified in clause 6.4.X [Neighbouring blocks availability checking process] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( ( xCb + ( mvL[ 0 ] >> 4 ) + CtbSizeY ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ), ( ( yCb + ( mvL[ 1 ] >> 4 ) ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ) ) as inputs, and the output shall be equal to FALSE.

    - The luma location ( ( ( xCb + ( mvL[ 0 ] >> 4 ) + CtbSizeY ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ), ( ( yCb + ( mvL[ 1 ] >> 4 ) ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ) ) shall not be equal to ( xCb, yCb ).

# Simulation results

The proposed method is implemented on top of VTM-4.0. Simulation is performed using the common test condition as defined in JVET-M1010.

Table I: Proposed restriction vs VTM-4.0 anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4-IBC-on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.07% | 0.08% | -0.01% | 100% | 101% |
| Class SCC 1080p | 0.22% | 0.22% | 0.20% | 103% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM4-IBC-on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.03% | 0.13% | -0.03% | 100% | 101% |
| Class SCC 1080p | 0.07% | 0.09% | 0.06% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4-IBC-on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.15% | 0.20% | -0.16% | 98% | 99% |
| Class SCC 1080p | 0.14% | 0.21% | 0.24% | 101% | 101% |

# Conclusion

In this proposal, a corner case is restricted in IBC’s reference area availability. It is asserted that by disallowing the first CU of each 64x64 region to access its collocated 64x64 region in the left CTU, reading from and writing to the same memory can be avoid for the implementation of IBC. It is proposed to include this modification to the next VVC specification and VTM release.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2] X. Xu, X. Li, S. Liu, E. Chai, “CE8: CPR reference memory reuse without increasing memory requirement (CE8.1.2a and CE8.1.2d)”, JVET-M0407, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).