| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0386-v1 |
| --- | --- |

| Title: | AOV sequence in 4:4:4 format for VVC standardization and suggestions for class F |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Jing Ye, Xiang Li and Shan Liu, Lori Wu, Chemin Xie, Keane Liu, Berton Wang, Ping Liu, Kane Dong, Ying Kuang, Weizhong Feng | Tel: Email: | {xiaozhongxu, jingye, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to include the AOV test sequence in YUV4:4:4 format, whose 4:2:0 version is currently in Class F test set, as part of the 4:4:4 test set in VVC.

# Sequence description

The following sequence is in 1080p resolution and in YUV 4:4:4 format. The length of the sequence is 10 seconds, and BT.709 color primaries.

| Sequence | Source | Frame rate | Number of frames |
| --- | --- | --- | --- |
| Tencent_AOV_1920x1080_60_8bit_444.yuv | Screen capture AOV | 60 fps | 600 |

The screen shot of this sequence is shown below:

| Tencent_AOV_1920x1080_60_8bit_444.yuv |  |
| --- | --- |

# Objective Evaluation

RD performance of AI, RA and LDB configurations for this test sequence is evaluated by HM-16.17+SCM-8.6 [1]. The default configurations provided in the cfg files [2] in the software are used. 4 QPs 22-27-32-37 are tested.

The RD curves are presented as follows:

TBA:

Fig. 1 Rate-Distortion curves of proposed sequence (RA and LDB)

# Replace outdated Class F sequences with the SCC TGM Sequences

The current Class F sequences have been used by the community for a number of years, however are now becoming outdated in that they neither have the resolutions, nor the frame rates current systems use.

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| Class F | 832x480 1024x768 1280x720 1280x720 | BasketballDrillText ChinaSpeed SlideEditing SlideShow | 50 30 30 20 |

We propose to replace those sequences with the SCC TGM 4:2:0 sequences, in particular:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| SCC TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 |

For these TGM sequences, both 4:2:0 and YUV 4:4:4 formats are available.

# Conclusions

In this contribution, the 4:4:4 version of the AOV sequence in class F is proposed to include into 4:4:4 test set for VVC. In addition, a suggestion is made to replace those old sequences in class F with SCC TGM sequences.

# References

- https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-16.17%2BSCM-8.6

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common test conditions for screen content coding,” JCTVC-U1015, 21st Meeting: Warsaw, PL, 19–26 June 2015.

- Copyright information

This sequence and all intellectual property rights therein remain the property of Shenzhen city Tencent computer system Co. Ltd. and Tencent America LLC. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen city Tencent computer system Co. Ltd. and Tencent America LLC. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.