| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0387r1 |
| --- | --- |

| Title: | CE3: report of CE3-1.1 on reduction of the number of reference samples in CCLM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports results of CE3-1.1 related to CCLM simplification. The proposal consists in limiting the number of reference samples used to derive the CCLM parameters to up to 6 (3 for the neighboring top line, 3 for the neighboring left column).

The reported PSNR-Y, -U and -V  BD-rate variations for AI are 0.02%, 0.14%, 0.17% and for RA 0.02%, -0.02%, 0.01%.

# Proposal

The proposed solution consists in reducing the number of reference samples in LM Chroma process, as initially proposed in JVET-M0108.

The luma values and corresponding chroma values in the neighboring of the chroma blocks are selected as follows:

- Upper line: 3 locations (i – 1, 0), (i – 1, nS – 1), (i – 1, nS/2)

- Left column: 3 locations (0, j – 1), (nS – 1, j – 1), (nS/2, j – 1)

where nS corresponds to the number of samples in the upper line / left column of the chroma and downsampled luma blocks used for deriving the linear model parameters, and (i, j) is a sample position related to the block’s origin in the picture.

In case of CCLM, up to 6 samples are used. In case of CCLM_L and CCLM_T only 3 samples, from the upper line / left column, are used.

# Experimental results

The results are produced using the VTM4.0 as anchor, for AI and RA configurations. It is observed that the average BD-rate impact of the CCLM simplification is very limited, while encoding and decoding times are slightly decreased.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.37% | 0.25% | 98% | 98% |
| Class A2 | 0.06% | 0.49% | 0.28% | 98% | 98% |
| Class B | 0.01% | 0.03% | 0.14% | 98% | 97% |
| Class C | 0.01% | -0.07% | 0.10% | 98% | 97% |
| Class E | 0.01% | 0.01% | 0.11% | 97% | 97% |
| Overall | 0.02% | 0.14% | 0.17% | 98% | 97% |
| Class D | 0.02% | 0.10% | 0.00% | 97% | 97% |
| Class F | 0.06% | 0.19% | 0.06% | 97% | 98% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.10% | 100% | 99% |
| Class A2 | 0.01% | 0.33% | 0.16% | 101% | 99% |
| Class B | 0.01% | -0.12% | -0.20% | 99% | 99% |
| Class C | 0.03% | -0.12% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | 0.01% | 100% | 99% |
| Class D | 0.02% | -0.15% | -0.34% | 100% | 99% |
| Class F | 0.00% | 0.11% | -0.05% | 99% | 99% |

# Conclusion

It is possible to reduce substantially the number of reference samples used in the derivation of the CCLM parameters without any noticeable coding penalty. It is therefore proposed to consider this option for adoption.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).