| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0388-v3 |
| --- | --- |

| Title: | Non-CE6: Shape Adaptive Transform Selection for ISP, SBT and MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Tangi Poirier Fabrice Le Léannec | Tel: Email: | karam.naser@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In the 13th JVET meeting, three coding tools associated with transform coding were adopted, namely: Intra Sub-partitioning (ISP, JVET-M0102), Sub-block Transform (SBT, JVET-M0140), and shape adaptive implicit transform selection (ImplicitMTS, JVET-M0303). The transform design corresponding to these tools is different from the default design, which requires additional specification for the up-coming standard. Additionally, the ImplicitMTS mode uses an additional transform pair with a flag coded in the bitstream, whereas it can be purely implemented as one pair of the MTS pairs.

This contribution proposes a homogeneous design of the transform coding in which the transform pair of ISP and SBT is the same as the one for ImplicitMTS. Additionally, the ImplicitMTS transform pair is embedded within the MTS design, so that it becomes an encoder option without signaling. In a first test (test1), ImplicitMTS is only applied to ISP, but not SBT. In a second test (test2), ImplicitMTS applies also to SBT transform selection.

Reported PSNR-Y, U, V BD-rates variations compared to VTM4.0 anchor are as follows. For test1, 0.00%, 0.02%, -0.01% in AI, 0.00%, -0,01%, -0.02% in RA and 0.00%, 0.04%, 0.22% in LDB configurations. For test2, 0.00%, 0.02%, -0.01% in AI, 0.13%, 0.01%, -0.08% in RA and 0.17%, 0.16%, 0.00% in LDB configurations. Encoding and decoding times are reportedly not impacted.

# Introduction

The newly adopted tools of ISP, SBT and ImplicitMTS have specific transform designs that are distinguished from the one of MTS. Specifically:

- ISP transform pairs are selected based on the intra prediction mode, where a combination of DST7 and DCT2 is used

- SBT transform pairs are selected based on the location of the transform subblock, where a combination of DST7 and DCT8 is used

- ImplicitMTS transform pair is selected based on the dimension of the transform block, where a combination of DCT2 and DST7 is used.

Additionally, an ImplicitMTS flag is signaled in the bit stream to indicate that this mode is enabled for the considered sequence, for which the encoder has less transform pairs to check in comparison with the default mode.

The main issue with the current design of these tools is the additional specification in the standardization. Additionally, both ISP and SBT require access to information that create additional dependency. Namely, ISP requires accessing the intra prediction mode in order to deduce the transform pairs, whereas SBT requires the coding of SBT position.

# Proposed Method

## ImplicitMTS as an MTS candidate

Instead of having a distinguished coding mode as “ImplicitMTS”, the latter can be embedded in the MTS pairs. Specifically, the last MTS pair of {DCT8,DCT8} is removed and ImplicitMTS is placed instead. The binarization is performed according to the table below:

| tu_mts_idx | transform type |  | binarization |  |  |
| --- | --- | --- | --- | --- | --- |
|  | horizontal | vertical | MTS & TS enabled | MTS enabled | TS enabled |
| 0 | SKIP | SKIP | 0 | - | 0 |
| 1 | DCT-II | DCT-II | 10 | 0 | 1 |
| 2 | DST-VII | DST-VII | 110 | 10 | - |
| 3 | DCT-VIII | DST-VII | 1110 | 110 | - |
| 4 | DST-VII | DCT-VIII | 11110 | 1110 | - |
| 5 | ImplicitMTS | ImplicitMTS | 11111 | 1111 | - |

The transform selection for ImplicitMTS is performed as follows:

trTypeHor = trTypeVer = DCT2;

if ( width < height)

trTypeHor = DST7;

else if ( height < width)

trTypeVer = DST7;

else if ( width == height)

trTypeHor = trTypeVer = DST7;

It should be noted that if width or height is greater than 32, the transform type of Horizontal and Vertical is DCT2.

To optimize the coding of MTS index, if the ImplicitMTS pair is available as one of the original pairs of the table, ImplicitMTS pair is removed. This happens in one of the two cases:

- width or height is greater than 32: ImplicitMTS pair is {DCT2,DCT2} which is the same as tu_mts_idx = 1

- width is equal to height: ImplicitMTS pair is {DST7,DST7}, which is the same as tu_mts_idx=2

For these two cases, the binarization table is modified to:

| tu_mts_idx | transform type |  | binarization |  |  |
| --- | --- | --- | --- | --- | --- |
|  | horizontal | vertical | MTS & TS enabled | MTS enabled | TS enabled |
| 0 | SKIP | SKIP | 0 | - | 0 |
| 1 | DCT-II | DCT-II | 10 | 0 | 1 |
| 2 | DST-VII | DST-VII | 110 | 10 | - |
| 3 | DCT-VIII | DST-VII | 1110 | 110 | - |
| 4 | DST-VII | DCT-VIII | 1111 | 111 | - |

Optionally, the encoder can choose a fast mode where only the pair corresponding to ImplicitMTS is tested, along with transform skip. This is to reduce the encoder search time and maintain a reasonably good coding gain with ImplicitMTS pair. To do so, the encoder tests only the last pair from the table, or the third pair if the table length is 5. The signaling of MTS index is expected to be negligible as its redundancy can be well encoded using the CABAC coder.

## ImplicitMTS for ISP and SBT

For both ISP and SBT, the subblock width and height are checked, and the transform pair is obtained using the same method as ImplicitMTS, namely:

trTypeHor = trTypeVer = DCT2;

if ( width < height)

trTypeHor = DST7;

else if ( height < width)

trTypeVer = DST7;

else if ( width == height)

trTypeHor = trTypeVer = DST7;

# Experimental Results

## Test 1 - Implicit MTS for ISP and inside MTS

In this test, the simplification related to SBT described in 2.2 was not considered. In other words, solely ISP simplification and embedding ImplicitMTS in MTS set described in 2.1 are considered. The proposed method was tested under the common testing conditions [4] for the three configurations of AI, RA and LDB, with VTM4.0 as anchor.

The experimental results are shown in the tables below.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 99% | 99% |
| Class A2 | -0.01% | 0.12% | -0.01% | 100% | 99% |
| Class B | 0.01% | -0.02% | 0.07% | 100% | 100% |
| Class C | 0.02% | -0.08% | -0.05% | 100% | 100% |
| Class E | -0.01% | 0.09% | -0.14% | 100% | 100% |
| Overall | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.02% | -0.18% | 100% | 100% |
| Class F | 0.00% | 0.06% | 0.00% | 98% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.09% | 100% | 99% |
| Class A2 | -0.03% | -0.22% | -0.08% | 101% | 99% |
| Class B | 0.00% | 0.15% | -0.17% | 101% | 101% |
| Class C | 0.02% | -0.07% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.02% | 101% | 100% |
| Class D | 0.00% | -0.29% | -0.14% | 100% | 99% |
| Class F | 0.00% | -0.24% | -0.11% | 117% | 109% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.27% | 0.07% | 101% | 101% |
| Class C | 0.01% | 0.25% | -0.18% | 101% | 101% |
| Class E | 0.00% | 0.30% | 0.99% | 101% | 101% |
| Overall | 0.00% | 0.04% | 0.22% | 101% | 101% |
| Class D | 0.01% | -0.20% | -0.37% | 101% | 102% |
| Class F | -0.07% | -0.28% | -0.25% | 101% | 100% |

## Test 2 - Implicit MTS for ISP and SBT and inside MTS

A second test was performed with combination of ImplicitMTS as an MTS candidate of section2.1 and ImplicitMTS for ISP and SBT of section 2.2.

The proposed method was tested under the common testing conditions [4] for the three configurations of AI, RA and LDB, with VTM4.0 as anchor. Below is the BD-rate performance.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 99% | 99% |
| Class A2 | -0.01% | 0.12% | -0.01% | 100% | 99% |
| Class B | 0.01% | -0.02% | 0.07% | 100% | 100% |
| Class C | 0.02% | -0.08% | -0.05% | 100% | 100% |
| Class E | -0.01% | 0.09% | -0.14% | 100% | 100% |
| Overall | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.02% | -0.18% | 100% | 100% |
| Class F | 0.00% | 0.06% | 0.00% | 98% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.07% | 0.03% | 99% | 98% |
| Class A2 | 0.14% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.12% | 0.13% | -0.27% | 99% | 99% |
| Class C | 0.16% | -0.06% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.01% | -0.08% | 99% | 99% |
| Class D | 0.15% | -0.16% | -0.15% | 100% | 99% |
| Class F | 0.07% | -0.24% | -0.13% | 116% | 108% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | -0.21% | -0.23% | 100% | 101% |
| Class C | 0.18% | 0.39% | -0.13% | 100% | 97% |
| Class E | 0.13% | 0.46% | 0.56% | 100% | 99% |
| Overall | 0.17% | 0.16% | 0.00% | 100% | 99% |
| Class D | 0.17% | 0.31% | -0.08% | 101% | 102% |
| Class F | 0.13% | -0.52% | 0.20% | 100% | 100% |

As can be seen, the performance in AI configuration is very close to the anchor. This indicates that the shape adaptive selection of the transform for ISP and the embedding of ImplicitMTS in MTS works very well. Some 0.1% loss in RA indicates that changing the SBT transform pair causes some loss. This can be considered as a small loss for the SBT gain (around 0.5%).

The results of AI are the same as the previous test, as no modifications is made on intra coding.

# Conclusion

The contribution proposes a method to unify the transform design of ISP, SBT and ImplicitMTS by embedding the latter in MTS and use it to infer the transform pairs of ISP and SBT. Experimental results show an equivalent performance under AI testing scenario and a little loss in RA and LDB. It is recommended to adopt the proposed method if the associated loss is acceptable, or partially adopt it without altering the transform design of SBT.

# References

[1] S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)” JVET-M0102, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[2] Y. Zhao, H. Gao, H. Yang, J. Chen, “CE6: Sub-block transform for inter blocks (Test 6.4.1)” JVET-M0140, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[3] J. Lainema, “CE6: Shape adaptive transform selection (Test 3.1)” JVET-M0303, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[4] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).