| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_max_dec_pic_buffering_minus1 | ue(v) |
| long_term_ref_pics_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < !rpl1_same_as_rpl0_flag ? 2 : 1; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_tile_group_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_tile_groups != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_tile_group | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_tile_group | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_tile_group_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma | ue(v) |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_pcm_enabled_flag | u(1) |
| if( sps_pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| If (sps_explicit_mts_intra_enabled_flag ¦¦ sps_explicit_mts_inter_enabled_flag) |  |
| sps_mts_enabled_flag=1 |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_gbi_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| sps_lmcs_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Transformation process for scaled transform coefficients

#### General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT-:

- Or if cu_sbt_flag is equal to 1-:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 32  &&  nTbW <= nTbH ) ? 1 : 0	(8-1030)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 32  &&  nTbH <= nTbW ) ? 1 : 0	(8-1031)

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-13 for nTbW not equal to nTbH, or Table 8-14 for equal nTbW and nTbH, depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-1032)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(8-1033)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-1034)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-13 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ], for a block width and height not that are not equal

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | Eqn((8-1030) |
| trTypeVer | 0 | 1 | 1 | 2 | Eqn((8-1) |

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 |
| trTypeVer | 0 | 1 | 1 | 2 |

Table 8-14 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ] for a block width and height not that are not equal

-

-