The following spec is based on version 9 of JVET-L1001.The added syntax is highlighted in yellow.

### 8.5.5.1	Picture reconstruction with mapping process

This clause specifies picture reconstruction with mapping process. The picture reconstructon with mapping process for luma sample value is specified in 8.5.5.1.1. The picture reconstructon with mapping process for chroma sample value is specified in 8.5.5.1.2.

### 8.5.5.1.1	Picture reconstructon with mapping process for luma sample value

Inputs to this process are:

–	an (nCurrSw)x(nCurrSh) array predSamples specifying the luma predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the luma residual samples of the current block.

–	The array InputPivot[ i ] and ReshapePivot[ i ], with i in the range of 0 to MaxBinIdx + 1, inclusive, derived in 7.4.4.1.

–	The array ScaleCoeff[ i ] and InvScaleCoeff[ i ] with i in the range of 0 to MaxBinIdx, inclusive, derived in 7.4.4.1.

The output for this process are:

–	an (nCurrSw)x(nCurrSh) mapped luma prediction sample array predMapSamples,

–	an (nCurrSw)x(nCurrSh) reconstructed luma sample array recSamples.

The predMapSamples is derived as follows:

–	If ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_INTRA ) || ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_IBC) || ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_INTER && mh_intra_flag[ xCurr ][ yCurr ] )

predMapSamples[  i ][  j ] =  predSamples[ i ][ j ]	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise ( ( CuPredMode[ xCurr ][ yCurr ] = = MODE_INTER && !mh_intra_flag[ xCurr ][ yCurr ] )), the following applies:

shiftY = 14
idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
predMapSamples[  i ][  j ] =  ReshapePivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] *(predSamples[ i ][ j ]  − InputPivot[ idxY ] )
	 + ( 1 << ( shiftY – 1 ) ) ) >> shiftY 	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

The recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1Y ( predMapSamples[  i ][   j ]+ resSamples[ i ][ j ] ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.5.5.1.2	Picture reconstructon with mapping process for chroma sample value

Inputs to this process are:

–	an (nCurrSwx2)x(nCurrShx2) array mapped predMapSamples specifying the mapped luma predicted samples of the current block,

- when tile_group_type is equal to 2 (I tile group) and qtbtt_dual_tree_intra_flag is equal to 1, an array recSamples specifying the reconstructed luma of the current picture,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

–	The array InputPivot[ i ] and ReshapePivot[ i ], ] with i in the range of 0 to MaxBinIdx + 1, inclusive, derived in 7.4.4.1.

–	The array InvScaleCoeff[ i ] and ChromaScaleCoef[i]  with i in the range of 0 to MaxBinIdx, inclusive, derived in 7.4.4.1.

The output for this process is reconstructed chroma sample array recSamples.

The recSamples is derived as follows:

–	If ( !tile_group_reshaper_chroma_residual_scale_flag || ( (nCurrSw)x(nCurrSh) <= 4) )

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C ( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) 	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tile_group_reshaper_chroma_residual_scale_flag && ( (nCurrSw)x(nCurrSh) > 4)), the following applies:

The variable varScale is derived as follows:

–	invAvgLuma is derived as follows:

  - If tile_group_type is equal to 2 (I tile group) and qtbtt_dual_tree_intra_flag is equal to 1, the following applies.

  - The chroma location (xCh, yCh) of the top-left sample of the chroma current block relative to the top-left chroma sample of the current picture is identified.

  - The luma location ( xTL, yTL ) is set equal to the top-left sample location of the collocated luma coding block ColLumaBlock covering the location given by (xCh << 1, yCh << 1) relative to the top-left luma sample of the current picture, and wColoc and hColoc are set equal to the width and height of ColLumaBlock.

  - If ( (32 + yCh) / 32 ) is equal to ( (64 + yTL – 1 ) / 64 ), invAvgLuma is set equal to recSamples[ yTL – 1 ][  xTL ]

  - Otherwise if ( (32 + xCh) / 32 ) is equal to ( (64 + xTL – 1 ) / 64 ), invAvgLuma is set equal to recSamples[ yTL ][  xTL – 1 ]

  - Otherwise invAvgLuma is set equal to –1.

  - Otherwise, the following applies:

invAvgLuma = Clip1Y( ( ij predMapSamples[ i ][  j ] + nCurrSw * nCurrSh *2) / ( nCurrSw * nCurrSh *4 ) )

–	If invAvgLuma is not equal to –1, the following applies:

  - the variable idxYInv is derived by involing the identification of piece-wise function index as specfied in clause 8.5.6.2 with the input of sample value invAvgLuma.

  - varScale is set equal to ChromaScaleCoef[ idxYInv ]

–	Otherwise varScale is set equal to ( 1 << shiftC )

The recSamples is derived as follows:

–	 If tu_cbf_cIdx [ xCurr ][ yCurr ] equal to 1, the following applies:

shiftC = 11
recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1 ( predSamples[ i ][ j ] + Sign( resSamples[ i ][ j ] )
 * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << ( shiftC – 1 ) ) ) >> shiftC ) )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tu_cbf_cIdx[ xCurr ][ yCurr ] equal to 0)

recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1(predSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1