| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0390r1 |
| --- | --- |

| Title: | AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Franck Galpin Fabrice Le Léannec, Tangi Poirier | Tel: Email: | edouard.francois@technicolor.com franck.galpin@technicolor.com fabrice.leleannec@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution relates to the usage of CCLM in tile groups supporting separate luma/chroma partitioning tree. In AhG16 discussions on Implementation studies, it was noticed that Separate luma/chroma partitioning tree may introduce a long latency for chroma intra prediction when the CCLM is on. The worst case in VTM4.0 corresponds to a latency of 64x64 luma reconstructed samples, required to predict chroma samples. This contribution aims at introducing constraints to CCLM activation when separate luma/chroma tree is used. In one option (named “restrict16x16”), CCLM is enabled only if the chroma block is inside a 16x16 non-overlapping area in a 16x16 sample grid, and if all its co-located luma samples belong to luma blocks included in the corresponding 32x32 area. In a second option(named “restrict16x32or32x16”), the chroma non-overlapping area is of size 16x32 or 32x16, and the co-located luma blocks are constrained to be inside the corresponding 32x64 or 64x32 non-overlapping area. These two options are evaluated in VTM4.0. To compensate for the loss, the option of enabling BT split for 64x64 luma blocks is also considered (while in VTM4.0 only QT split is enabled). Results for different combinations of these options are reported.

# Introduction

CCLM and separate luma-chroma tree (CST) are two performing tools of the VTM4.0. As reported from AhG13 activities on Tools evaluation, the average performance of CCLM and CST individual tools are evaluated as follows in VTM4.0 (tool off evaluation).

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-Y | BDR-U | BDR-V | EncTime | DecTime |
| CST | 0.29% | 11.28% | 12.41% | 121% | 101% |
| CCLM | 2.08% | 19.05% | 19.23% | 99% | 99% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-Y | BDR-U | BDR-V | EncTime | DecTime |
| CST | 0.16% | 4.90% | 6.35% | 102% | 99% |
| CCLM | 0.91% | 17.37% | 17.31% | 101% | 102% |

However, as mentioned in AhG16 report JVET-M0016, there is a potential latency issue for generating the chroma predicted samples when applying CCLM in separate luma/chroma tree. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed. The report was suggesting investigating solutions to address this issue.

This contribution reports results from different investigated options on CCLM restrictions in tile groups using separate luma/chroma partitioning tree.

# Considered options

The following options have been tested. They have been tested individually or combined.

Option1: restrict16x16

CCLM is enabled for a chroma block if the following conditions are true:

- The chroma block is inside a 16x16 non-overlapping area aligned with a 16x16 grid of chroma samples

- The co-located luma samples belong to luma blocks inside the corresponding 32x32 non-overlapping area aligned with a 32x32 grid of luma samples

This is illustrated in the figure below (where the chroma block relative dimensions are scaled by 2). CCLM is enabled for the chroma block since it is included in the 16x16 grid, and its co-located luma blocks are also inside the co-located 32x32 area.

In this option, the maximum structural delay to process the chroma prediction corresponds to the reconstruction of the 32x32 collocated luma samples.

Option2: restrict16x32or32x16

CCLM is enabled for a chroma block if the following conditions are true:

- The chroma block is inside a 16x32 or 32x16 non-overlapping area aligned with a 16x16 grid of chroma samples

- The co-located luma samples belong to luma blocks inside the corresponding 32x64 or 64x32 non-overlapping area aligned with a 32x32 grid of luma samples

This is illustrated in the figure below. CCLM is enabled for the chroma block since it is included in the 16x32 grid, and its co-located luma blocks are also inside the co-located 32x64 area.

In this option, the maximum structural delay to process the chroma prediction corresponds to the reconstruction of the 32x64/64x32 collocated luma samples.

Option3: enableBT

To compensate for the loss due to using one of the two restrictions above, the BT split is enabled for luma blocks of size 64x64, while in VTM4.0, only QT split is enabled.

Option4: forceQTorBT

When BT split is enabled for luma blocks of size 64x64 (option enableBT), QT or BT split is enforced for luma blocks of size 64x64.

# Results

The BD-rate performance for different combinations of options are reported in the tables below for the AI configuration. Results are presented in section 3.1compared to VTM4.0 with CCLM on, and in section 3.2 with CCLM off.

In the worst case, when using restriction restrict16x16, the CCLM restriction leads to an average BD-rate PSNR-Y, -U, -V loss of 0.28%, 2.57%, 2.56%. In the best case, an average BD-rate PSNR-Y, -U, -V loss of 0.04%, 2.18%, 2.18% is obtained by enabling BT for luma blocks 64x64 and by applying the restriction “restrict16x32or32x16” (configuration “enableBT_restrict16x32or32x16”). This penalty is significantly less, especially for chroma, than the loss due to disabling separate luma/chroma partitioning tree (0.29%, 11.28%, 12.41%).

It can be observed from results of section 3.2, that uses VTM4.0 without CCLM as anchor, that most of the CCLM gain is anyway preserved. CCLM BD-rate PSNR-Y, -U, -V gain is 1.98%, 14.46%, 15.05%. Applying restrict16x16 leads to 1.71%, 12.47%, 13.03%. Applying restrict16x16 while enabling BT for 64x64 luma blocks (enableBT_restrict16x16) leads to 1.88%, 12.39%, 12.85%, which is close to the gain of the complete (unrestricted) CCLM.

It can also be noted that for the sequence presenting the higher loss, Tango2, the loss of not using CCLM is 4.08%, 51.99%, 50.51%. When restrict16x16 is used, the loss is 1.43%, 14.49%, 12.22%.

Some visual checks have been performed on the 2 clips presenting the higher objective chroma loss in AI, Tango2 and FoodMarket4. Comparisons were made between VTM4.0, and restrict16x16 configuration, at highest QPs where the objective loss is higher. Despite the objective loss, it was extremely difficult to identify noticeable quality difference when inspecting the pictures in still picture mode.

## All Intra using VTM4.0 as anchor

|  | no CCLM |  |  |  | restrict16x16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |  |  |  |
| Class A1 | 5.98% | 49.10% | 41.80% |  | 1.02% | 7.75% | 7.40% |  |  |  |  |
| Class A2 | 2.17% | 16.41% | 9.22% |  | 0.28% | 2.54% | 1.38% |  |  |  |  |
| Class B | 0.98% | 12.89% | 20.10% |  | 0.15% | 2.17% | 3.18% |  |  |  |  |
| Class C | 1.78% | 16.19% | 18.19% |  | 0.06% | 0.31% | 0.35% |  |  |  |  |
| Class E | 0.31% | 5.71% | 6.57% |  | 0.05% | 1.13% | 0.84% |  |  |  |  |
| Overall | 2.08% | 19.05% | 19.23% |  | 0.28% | 2.57% | 2.56% |  |  |  |  |
| Class D | 1.18% | 12.88% | 12.96% |  | 0.03% | 0.22% | 0.09% |  |  |  |  |
| Class F | 3.57% | 18.00% | 20.77% |  | 0.13% | 0.65% | 0.94% |  |  |  |  |
| EncT | 99% |  |  |  | 99% |  |  |  |  |  |  |
| DecT | 99% |  |  |  | 98% |  |  |  |  |  |  |
|  | enableBT |  |  |  | enableBT_restrict16x16 |  |  |  | enableBT_restrict16x32or32x16 |  |  |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |
| Class A1 | -0.54% | -0.60% | -0.76% |  | 0.72% | 8.43% | 8.50% |  | 0.40% | 6.56% | 5.87% |
| Class A2 | -0.24% | -0.21% | -0.26% |  | 0.12% | 2.73% | 1.40% |  | 0.06% | 2.18% | 1.08% |
| Class B | -0.19% | -0.23% | -0.25% |  | -0.02% | 2.16% | 3.33% |  | -0.04% | 1.88% | 2.98% |
| Class C | -0.01% | -0.10% | -0.03% |  | 0.03% | 0.24% | 0.42% |  | 0.03% | 0.21% | 0.36% |
| Class E | -0.26% | -0.23% | -0.21% |  | -0.21% | 1.03% | 0.84% |  | -0.20% | 0.89% | 0.68% |
| Overall | -0.23% | -0.26% | -0.28% |  | 0.11% | 2.68% | 2.81% |  | 0.04% | 2.18% | 2.18% |
| Class D | -0.02% | -0.15% | -0.15% |  | 0.02% | 0.09% | 0.01% |  | 0.02% | 0.11% | 0.01% |
| Class F | -0.08% | -0.01% | -0.07% |  | 0.03% | 0.89% | 1.08% |  | 0.02% | 0.74% | 0.94% |
| EncT | 111% |  |  |  | 110% |  |  |  | 111% |  |  |
| DecT | 107% |  |  |  | 107% |  |  |  | 107% |  |  |
|  | forceQTorBT |  |  |  | forceQTorBT_restrict16x16 |  |  |  | forceQTorBT_restrict16x32or32x16 |  |  |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |
| Class A1 | -0.08% | 1.19% | 0.36% |  | 0.95% | 8.40% | 8.02% |  | 0.42% | 5.58% | 3.81% |
| Class A2 | -0.07% | 0.59% | 0.19% |  | 0.18% | 3.04% | 1.64% |  | 0.05% | 2.27% | 1.03% |
| Class B | -0.12% | -0.01% | 0.20% |  | 0.04% | 2.13% | 3.28% |  | 0.01% | 1.45% | 2.51% |
| Class C | -0.02% | -0.07% | -0.11% |  | 0.02% | 0.30% | 0.26% |  | 0.01% | 0.26% | 0.18% |
| Class E | -0.03% | 0.41% | 1.25% |  | 0.01% | 1.44% | 2.02% |  | 0.00% | 1.29% | 1.94% |
| Overall | -0.06% | 0.35% | 0.33% |  | 0.21% | 2.80% | 2.91% |  | 0.09% | 1.98% | 1.87% |
| Class D | -0.03% | -0.03% | -0.16% |  | 0.01% | 0.03% | 0.00% |  | 0.00% | 0.05% | 0.01% |
| Class F | 0.14% | 0.44% | 0.45% |  | 0.26% | 1.00% | 1.07% |  | 0.25% | 1.03% | 1.11% |
| EncT | 113% |  |  |  | 114% |  |  |  | 115% |  |  |
| DecT | 107% |  |  |  | 106% |  |  |  | 105% |  |  |

## All Intra using VTM4.0 without CCLM as anchor

|  | CCLM |  |  |  | restrict16x16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |  |  |  |
| Class A1 | -5.51% | -30.78% | -28.84% |  | -4.55% | -25.12% | -23.35% |  |  |  |  |
| Class A2 | -2.11% | -13.50% | -8.09% |  | -1.83% | -11.51% | -6.88% |  |  |  |  |
| Class B | -0.97% | -11.38% | -16.29% |  | -0.82% | -9.57% | -13.68% |  |  |  |  |
| Class C | -1.74% | -13.62% | -15.06% |  | -1.68% | -13.40% | -14.82% |  |  |  |  |
| Class E | -0.31% | -5.39% | -6.15% |  | -0.25% | -4.37% | -5.40% |  |  |  |  |
| Overall | -1.98% | -14.46% | -15.05% |  | -1.71% | -12.47% | -13.03% |  |  |  |  |
| Class D | -1.16% | -11.30% | -11.46% |  | -1.13% | -11.12% | -11.38% |  |  |  |  |
| Class F | -3.44% | -14.86% | -17.08% |  | -3.32% | -14.37% | -16.46% |  |  |  |  |
| EncT | 101% |  |  |  | 100% |  |  |  |  |  |  |
| DecT | 101% |  |  |  | 99% |  |  |  |  |  |  |
|  | enableBT |  |  |  | enableBT_restrict16x16 |  |  |  | enableBT_restrict16x32or32x16 |  |  |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |
| Class A1 | -6.03% | -31.19% | -29.39% |  | -4.84% | -24.63% | -22.57% |  | -5.14% | -26.06% | -24.51% |
| Class A2 | -2.34% | -13.65% | -8.32% |  | -1.99% | -11.35% | -6.87% |  | -2.05% | -11.80% | -7.15% |
| Class B | -1.16% | -11.56% | -16.50% |  | -0.99% | -9.58% | -13.57% |  | -1.01% | -9.82% | -13.85% |
| Class C | -1.74% | -13.70% | -15.09% |  | -1.70% | -13.47% | -14.75% |  | -1.71% | -13.48% | -14.80% |
| Class E | -0.57% | -5.59% | -6.35% |  | -0.52% | -4.46% | -5.39% |  | -0.51% | -4.58% | -5.53% |
| Overall | -2.20% | -14.66% | -15.28% |  | -1.88% | -12.39% | -12.85% |  | -1.94% | -12.80% | -13.34% |
| Class D | -1.18% | -11.42% | -11.58% |  | -1.14% | -11.24% | -11.46% |  | -1.14% | -11.22% | -11.46% |
| Class F | -3.52% | -14.84% | -17.07% |  | -3.42% | -14.18% | -16.36% |  | -3.43% | -14.30% | -16.43% |
| EncT | 112% |  |  |  | 111% |  |  |  | 112% |  |  |
| DecT | 108% |  |  |  | 108% |  |  |  | 108% |  |  |
|  | forceQTorBT |  |  |  | forceQTorBT_restrict16x16 |  |  |  | forceQTorBT_restrict16x32or32x16 |  |  |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |
| Class A1 | -5.59% | -29.97% | -28.57% |  | -4.62% | -24.49% | -22.84% |  | -5.12% | -26.70% | -25.98% |
| Class A2 | -2.17% | -13.05% | -7.92% |  | -1.93% | -11.10% | -6.65% |  | -2.06% | -11.73% | -7.19% |
| Class B | -1.08% | -11.40% | -16.13% |  | -0.93% | -9.61% | -13.57% |  | -0.96% | -10.19% | -14.21% |
| Class C | -1.76% | -13.67% | -15.16% |  | -1.72% | -13.40% | -14.89% |  | -1.73% | -13.42% | -14.95% |
| Class E | -0.33% | -5.00% | -4.99% |  | -0.30% | -4.07% | -4.26% |  | -0.31% | -4.21% | -4.35% |
| Overall | -2.04% | -14.21% | -14.76% |  | -1.78% | -12.26% | -12.70% |  | -1.90% | -12.92% | -13.52% |
| Class D | -1.19% | -11.33% | -11.59% |  | -1.15% | -11.29% | -11.46% |  | -1.16% | -11.27% | -11.45% |
| Class F | -3.31% | -14.51% | -16.81% |  | -3.19% | -14.10% | -16.36% |  | -3.21% | -14.06% | -16.35% |
| EncT | 114% |  |  |  | 115% |  |  |  | 116% |  |  |
| DecT | 108% |  |  |  | 108% |  |  |  | 106% |  |  |

## Random access using VTM4.0 as anchor

RA results are provided below for the cases restrict16x16, enableBT_restrict16x16, forceQTorBT_restrict16x32or32x16. Partial results for enableBT_restrict16x32or32x16 are also provided. These results show that the impact of the various considered restrictions/options remain limited in RA configuration. When considering 1-digit BD-rate values, average PSNR-Y, -U, -V BD-rate variations for restrict16x16 are 0.1%, 1.3%, 1.0%, enableBT_restrict16x16, 0.0%	, 1.3%, 1.1%.

The combination forceQTorBT_restrict16x62or32x16 leads to BD-rate variations of 0.0%, 0.8%, 0.6%.

restrict16x16

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.17% | 2.13% | 1.60% | 99% | 99% |
| Class A2 | 0.06% | 1.34% | 0.69% | 98% | 101% |
| Class B | 0.01% | 1.59% | 1.75% | 98% | 102% |
| Class C | 0.00% | 0.11% | 0.01% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 1.25% | 1.04% | 99% | 100% |
| Class D | -0.03% | -0.04% | -0.02% | 101% | 100% |
| Class F | 0.03% | 0.37% | 0.30% | 98% | 101% |

enableBT_restrict16x16

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.12% | 2.47% | 2.00% | 100% | 98% |
| Class A2 | -0.02% | 1.10% | 0.62% | 101% | 100% |
| Class B | 0.01% | 1.60% | 1.60% | 103% | 100% |
| Class C | 0.02% | 0.02% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 1.25% | 1.06% | 101% | 100% |
| Class D | 0.01% | -0.23% | -0.08% | 102% | 100% |
| Class F | 0.00% | 0.35% | 0.42% | 101% | 102% |

forceBTorBT_restrict16x32or32x16

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.06% | 1.46% | 0.82% | 101% | 100% |
| Class A2 | -0.05% | 1.17% | 0.46% | 99% | 97% |
| Class B | 0.00% | 0.76% | 0.92% | 99% | 100% |
| Class C | 0.01% | 0.15% | 0.32% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.82% | 0.65% | 101% | 100% |
| Class D | -0.01% | 0.09% | 0.20% | 99% | 98% |
| Class F | 0.12% | 0.58% | 0.59% | 103% | 99% |

enableBT_restrict16x32or32x16

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 1.53% | 1.36% | 98% | 99% |
| Class C | 0.01% | 0.10% | -0.03% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.21% | -0.09% | 101% | 101% |
| Class F | -0.02% | 0.21% | 0.38% | 100% | 101% |

# Conclusion

Several options for limiting the latency of processing chroma blocks coded in CCLM mode in tile groups supporting separate luma/chroma tree, while trying to limit the coding loss, have been evaluated.

The proposed restrictions reduce the latency for processing a chroma block when it is coded in CCLM mode in case of separate luma/chroma coding tree. While in current VTM, the maximum latency is from reconstructing a 64x64 luma block, when using restrict16x16, it is reduced to a 32x32 luma block, and when using restrict16x32or32x16, it is reduced to a 32x64 or 64x32 luma block.

In AI configuration, the worst case, when forbidding CCLM for chroma blocks larger than 16x16 (restrict16x16 option), the average BD-rate PSNR-Y, -U, -V loss is 0.3%, 2.6%, 2.6%. In the best case, when forbidding CCLM for chroma blocks larger than 16x32 or 32x16, and enabling BT for 64x64 luma blocks (enableBT_restrict16x32or32x16 configuration), the average BD-rate PSNR-Y,-U,-V loss is 0.0%, 2.2%, 2.2%. In both cases this is much less than disabling separate luma/chroma partitioning tree, which leads to an average BD-rate PSNR-Y, -U, -V loss of 0.3%, 11.3%, 12.4%.

In RA configuration, the impact is limited to 0.1%, 1.3%, 1.0% for the restrict16x16 option, to 0.0%, 1.3%, 1.1% for the combination enableBT_restrict16x16, and to 0.0%, 0.8%, 0.6% for the combination forceQTorBT_restrict16x62or32x16.

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).