| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0391 |
| --- | --- |

| Title: | AHG14: Updates on intra refresh proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson, David Gommelet VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com david.gommelet@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution reports an update of the software modifications for the study of progressive intra refresh (AHG 14). This code has been updated in the AHG Gitlab repository. Both encoder-only modifications and a normative solution have been updated to VTM 4.0.1 with handling of new tools adopted at Marrakech meeting. In particular, stronger deblocking filter significantly deteriorates the performance of encoder-only implementation. Affine prediction also implies new constraints at refresh borders within clean region. This contribution introduces the implementation of discussed testing conditions. In particular the adaptation of refresh region to handle whatever refresh cycle and the computation of bitrate and metrics after the first refreshing cycle. The corresponding performance draws similar conclusion as the results presented at Marrakech meeting with a higher loss of XX% in average for the encoder-only modifications against a low-delay B reference with a comparable Intra period of 1second. This significant loss demonstrates that a better handling of progressive intra refresh in VVC is relevant. The normative proposal brings an improvement of XX% against the encoder-only modification solution by adding a straightforward signalization to derive the position of the intra refresh area and then exploit this position for decoding process. This improvement has been split in two parts with first the mandatory modifications and secondly an enhanced version.

Considering the strong activity on the intra refresh topic, authors suggest to integrate the encoder-only modifications for intra-refresh support on VTM 5 and to consider the proposed normative solution for integration in the standard.

# Introduction

A proposal for normative handling of progressive intra refresh was made at Ljubljana [1] and lead to the creation of the Ad hoc Group 14 which aims at studying the opportunity of integrating such a normative solution. At Macao, [2] provides a great overview of the issues to tackle to have an efficient support of intra refresh within VVC. We respectively proposed in [3] an encoder-only implementation of intra refresh and in [4] an implementation of the proposed intra refresh support. This new contribution provides an update of these two contributions with integration within VTM 4.0.1. This code is available on the AHG14 gitlab: https://vcgit.hhi.fraunhofer.de/jvet-l-ahg-14/VVCSoftware_VTM/tree/AHG14-IntraRefresh_EncOnly

Since the Marrakech meeting, the following test conditions have been discussed on the JVET mailing-list, and agreed by several experts:

•	Basis = Low-Delay B Main10 configuration.

•	Without pyramidal B structure (=> GOPSize = 1) in order to have all the pictures with equivalent bits sizes.

•	QP offset of +1 between the first I and the P picture.

•	Intra refresh period of 1scd, meaning that the stream shall be refreshed after a cycle of 1scd (can be less).

•	PSNR and bitrates computed without the first I picture + the first cycle (meaning that the first 31 pictures are not considered in 30Hz, 61 in 60Hz…)

In order to support those conditions, the proposed software includes the associated modifications.

# Encoder-only intra refresh

Figure 1: Intra Refresh principle.

The software modifications to support intra refresh functionality at the encoder-level in VTM software include:

-	Intra prediction mode forced on coding unit on column basis. The granularity of refresh is aligned on 8x8 CU basis and an adaptive column size has been added to support whatever refresh cycle.

-	Constrained Intra Prediction enabled to ensure reconstruction of Intra CU (Constrained Intra Prediction as define in HEVC is used since it is available in VTM while it is not yet included in the VVC draft).

-	Motion vectors constrained to point within the clean area while taking into account an additional margin to avoid motion compensation filters error spreading. This constraint is now of 8 pixels due to strong deblocking filter.

-	Removing of former reference pictures when re-looping the Intra column.

-	Disabling of Adaptive Loop Filter at CTU level for CTU including Intra refresh pixels.

The additional parameters added to the configuration file are as follows:

IntraRefreshType			  : 1			# Intra Refresh type flag 0: disable, 1: enc-only, 2: normative

IntraRefreshStart			  : 0			# Position of the first column of Intra refresh (in pixels)

IntraRefreshDirection		  : 0			# Direction of Intra Refresh 0: left to right, 1: right to left

IntraRefreshPeriod			  : 30		# Period of the Intra Refresh

# Proposed normative intra refresh

## Description of the method

The proposed solution adds the minimum signalization needed for deriving the position of Intra Refreshed pixels and then exploits this position knowledge to improve the quality within this low-latency use-case. The modifications at the encoder side have been split in two as suggested by experts during Marrakech meeting. The first version includes the most necessary modifications which are related to the boundary handling between clean and dirty region. A second version is evaluated on top of this one to evaluate the improvement brought by additional modifications. The list of tools is described below:

Version 1:

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the intra refresh Area.

- Disabling of Deblocking Filter and Sample Adaptive Filter at the intra refresh boundary.

Version 2:

- Version 1 +

- Avoid the transmission of pred_mode_flag and cu_skip_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Specific handling of Adaptive Loop Filter to disable it within CTU on the intra refresh boundary.

- Usage of a specific delta QP for the intra refresh region.

## Signalization

The proposed additional signalization is:

- at SPS level:

  - sps_intra_refresh_enabled_flag specifies if the Intra Refresh tool is enabled.

- at PPS level:

  - pps_intra_refresh_mode specifies the mode of Intra Refresh (Column or Line for instance)

  - pps_intra_refresh_size  specifies the size of the refresh pattern (width of the Column, height of the Line, width x height for block-based refresh). This can be signaled as a multiple of the CU min size (0 for 8 pixels, 1 for 16 pixels, …).

  - pps_intra_refresh_delta_qp specifies a delta QP for the Intra Refresh blocks. This allows to optimize the encoder quality balancing between Intra Refresh blocks and others.

- at Tiles-group level:

  - tile_group_intra_refresh_direction specifies the direction of the refresh (right to left/left to right for Column, top to bottom/bottom to top for Line, …). This is used to determine motion vectors constraints during the encoding/decoding.

  - tile_group_intra_refresh_position specifies the position of the Intra Refresh blocks, this is given according to pps_intra_refresh_size.

The associated draft modifications will be added in annex.

# Results

## Results for encoder-only modifications against reference

## Results for proposed normative solution against encoder-only

### Version 1: necessary modifications

### Version 2: enhanced modifications

# Conclusion

It is recommended to integrate the encoder only modifications within the next VTM software version and to consider the proposed normative solution for integration in the standard.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2] K.Kazui, “AHG14: Study of methods for progressive intra refresh”, JVET-L0079, October 2018.

[3]  J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

[4] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Normative Intra Refresh Proposal”, JVET-L0161, October 2018.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).