| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 |  |  |  | Document: JVET-N0392-v2 |  |
| --- | --- | --- | --- | --- | --- |
| Title: | 4:4:4 and 4:2:2 chroma formats support for VVC |  |  |  |  |  |
| Status: | Input document to JVET |  |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Timofey Solovyev, Roman Chernyak, Jianle Chen | Email: | alexey.filippov@huawei.com, vasily.rufitskiy@huawei.com, solovyev.timofey@huawei.com, chernyak.roman@huawei.com, jianle.chen@huawei.com |  |  |  |
| Source: | Huawei Technologies Co. |  |  |  |  |  |

_____________________________

# Abstract

This document proposes modifications of the VTM software and the VVC spec draft to support YUV4:4:4, YUV4:2:2, and RGB4:4:4. For enabling that, straightforward changes of CCLM and affine sub-block motion vector derivation were made. In order to evaluate compression performance, we use the HEVC RExt test set and HM-16.18-SCM8.7-RExt as the anchor. For the YUV4:4:4 content, the proposed modifications reportedly show the following overall results: -21.8% (Y), -13.7% (U), -14.2% (V) with encoding time of 1475% and decoding time of 141% for AI configuration and -31.9% (Y), -23.0% (U), -19.4% (V) with encoding time of 576% and decoding time of 128% for RA configuration. For the RGB content, the proposed modifications reportedly demonstrate the following overall results: -21.7% (Y), - 14.5% (U), -15.3% (V) with encoding time 2000% and decoding time of 143% for AI configuration and -31.7% (Y), -19.3% (U), -21.7% (V) with encoding time 723% and decoding time of 134% for RA configuration. For the YUV4:2:2 content, the proposed modifications reportedly demonstrate the following overall results: -17.8% (Y), - 19.5% (U), -20.0% (V) with encoding time 2435% and decoding time of 157% for AI configuration and -22.1% (Y), -25.9% (U), -23.2% (V) with encoding time 665% and decoding time of 138% for RA configuration.

# Introduction

The current version of the VTM-4.0 software natively supports YUV 4:2:0 content as input format. Definitely, support of other chroma formats (primarily, 4:4:4) is highly desirable and even mandatory for a wide range of applications. However, consistency with the current design of VVC would be an attractive feature that can provide its wide deployment.

# Proposal

In this contribution, we propose the following modifications that are straightforward and provide the design consistency with the current version of VTM and the VVC spec draft:

- Changes for 4:4:4:

- Downsampling filtering is not performed for 4:4:4 chroma format;

- Affine sub-block motion vector derivation process for chroma is same as for luma.

- Changes for 4:2:2:

- Horizontal [1, 2, 1]/4 downsampling filter is used for 4:2:2 chroma format;

- Affine sub-block motion vector derivation process for chroma subblock uses averaging for motion vectors of two horizontally-adjacent luma sub-blocks.

In addition, some software modifications of DMVR and In-loop Reshaper were done to enable support for 4:4:4 and 4:2:2 color format. Besides, a minor editorial change is proposed for ISP as shown below:

| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) ) |  |
| --- | --- |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / SubWidthC2 |  |
| hC = CbHeight[ x0 ][ y0 ] / SubHeightC2 |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |

Compression performance for YUV 4:4:4 and RGB content was evaluated using the HEVC RExt test set described in [1] and listed in Table 1. The same cfg-files were used.

Table 1. Test sequences

| Class / Encoding format | Input bit-depth | Frame rate | Test sequence filename | Encoded frame count |
| --- | --- | --- | --- | --- |
| RGB 4:4:4 | 12 | 30 | Traffic_2560x1600_30_12bit_444_crop.rgb | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.rgb | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.rgb | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.rgb | 250 |
|  | 8 | 30 | VenueVu_1920x1080_30_8bit_444.rgb | 150 |
|  | 10 | 30 | DucksAndLegs_1920x1080_30_10bit_444.rgb | 150 |
|  | 10 | 50 | OldTownCross_1920x1080_50_10bit_444.rgb | 250 |
|  | 10 | 24 | ParkScene_1920x1080_24_10bit_444.rgb | 240 |
| YCbCr 4:4:4 | 10 | 30 | Traffic_2560x1600_30_10bit_444_crop.yuv | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.yuv | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.yuv | 250 |
|  | 10 | 30 | VenueVu_1920x1080_30_10bit_444.yuv | 150 |
|  | 10 | 60 | BirdsInCage_1920x1080_60_10bit_444.yuv | 300 |
|  | 10 | 24 | CrowdRun_1920x1080_50_10bit_444.yuv | 250 |
| YCbCr 4:2:2 | 10 | 30 | Traffic_2560x1600_30_10bit_422_crop.yuv | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_422.yuv | 240 |
|  | 10 | 50 | EBUHorse_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUGraphics_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | Seeking_1920x1080_50_10bit_422.yuv | 250 |

# Experiment Results

The results reportedly obtained for YUV 4:2:2, YUV 4:4:4 and RGB content using the common test conditions [2] are presented in Table 2-Table 4 for AI and RA configurations.

Table 3. Experimental results for YUV 4:2:2

| All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  |
| Traffic_2560x1600_30_10bit_422_crop.yuv |  | -23.5% |  | -16.6% |  | -21.2% |  |
| Kimono1_1920x1080_24_10bit_422.yuv |  | -22.4% |  | -16.4% |  | -13.8% |  |
| EBUHorse_1920x1080_50_10bit_422.yuv |  | -13.2% |  | -17.2% |  | -25.3% |  |
| EBUGraphics_1920x1080_50_10bit_422.yuv |  | -20.2% |  | -19.0% |  | -20.8% |  |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv |  | -14.8% |  | -10.1% |  | -10.4% |  |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv |  | -10.0% |  | -26.2% |  | -26.2% |  |
| Seeking_1920x1080_50_10bit_422.yuv |  | -20.4% |  | -30.7% |  | -21.9% |  |
| Overall |  | -17.8% |  | -19.5% |  | -20.0% |  |
| Enc Time[%] |  | 2435% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 157% |  |  |  |  |  |  |  |

| Random Access |  |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -28.2% | -20.5% | -22.4% |
| Kimono1_1920x1080_24_10bit_422.yuv | -26.1% | -23.9% | -15.8% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -19.3% | -27.4% | -27.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -28.0% | -25.3% | -25.9% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -14.6% | -12.6% | -10.5% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | -11.9% | -32.5% | -34.6% |
| Seeking_1920x1080_50_10bit_422.yuv | -26.8% | -38.9% | -25.8% |
| Overall | -22.1% | -25.9% | -23.2% |
| Enc Time[%] | 665% |  |  |
| Dec Time[%] | 138% |  |  |

Table 3. Experimental results for YUV 4:4:4

| All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -24.8% | -10.9% |  | -11.6% |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -24.3% | -8.9% |  | -7.6% |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -18.2% | -18.9% |  | -22.8% |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -17.5% | -15.6% |  | -16.2% |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -24.8% | -20.8% |  | -18.4% |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -27.9% | -8.8% |  | -9.7% |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -15.3% | -12.3% |  | -12.9% |  |
| Overall | -21.8% | -13.7% |  | -14.2% |  |
| Enc Time[%] | 1475% |  |  |  |  |
| Dec Time[%] | 141% |  |  |  |  |
| Random Access |  |  |  |  |  |
|  | Y | U |  | V |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -31.8% | -17.4% |  | -15.7% |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -30.0% | -18.0% |  | -15.0% |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -24.2% | -24.8% |  | -24.9% |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -24.3% | -14.5% |  | -13.5% |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -39.8% | -33.0% |  | -32.0% |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -48.3% | -32.8% |  | -13.2% |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -24.7% | -20.3% |  | -21.8% |  |
| Overall | -31.9% | -23.0% |  | -19.4% |  |
| Enc Time[%] | 576% |  |  |  |  |
| Dec Time[%] | 128% |  |  |  |  |

Table 4. Experimental results for RGB

| All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  |
| Traffic_2560x1600_30_12bit_444.rgb | -27.8% | -13.1% |  | -18.1% |  |
| Kimono1_1920x1080_24_10bit_444.rgb | -28.9% | -10.6% |  | -11.2% |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -25.7% | -21.3% |  | -23.5% |  |
| EBURainFruits_1920x1080_50_10bit_444.rgb | -24.5% | -19.1% |  | -24.2% |  |
| VenueVu_1920x1080_30_8bit_444.rgb | -25.7% | -21.0% |  | -19.7% |  |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -6.3% | -6.8% |  | -3.4% |  |
| OldTownCross_1920x1080_50_10bit_444.rgb | -13.6% | -14.2% |  | -12.3% |  |
| ParkScene_1920x1080_24_10bit_444.rgb | -21.5% | -10.1% |  | -10.3% |  |
| Overall | -21.7% | -14.5% |  | -15.3% |  |
| Enc Time[%] | 2000% |  |  |  |  |
| Dec Time[%] | 143% |  |  |  |  |
| Random Access |  |  |  |  |  |
|  | Y | U |  | V |  |
| Traffic_2560x1600_30_12bit_444.rgb | -34.8% | -17.3% |  | -23.5% |  |
| Kimono1_1920x1080_24_10bit_444.rgb | -35.5% | -17.5% |  | -23.3% |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -31.3% | -28.6% |  | -26.9% |  |
| EBURainFruits_1920x1080_50_10bit_444.rgb | -19.7% | -19.0% |  | -19.3% |  |
| VenueVu_1920x1080_30_8bit_444.rgb | -38.1% | -33.5% |  | -33.3% |  |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -26.9% | -8.9% |  | -12.5% |  |
| OldTownCross_1920x1080_50_10bit_444.rgb | -35.5% | -13.9% |  | -15.4% |  |
| ParkScene_1920x1080_24_10bit_444.rgb | -31.4% | -15.5% |  | -19.1% |  |
| Overall | -31.7% | -19.3% |  | -21.7% |  |
| Enc Time[%] | 723% |  |  |  |  |
| Dec Time[%] | 134% |  |  |  |  |

# Conclusion

We propose to adopt the proposed modifications into the VTM software and the VVC spec draft.

# References

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” , JCTVC-P1006, 16th JCT-VC Meeting, San Jose, CA, January 2014.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).