| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0393 |
| --- | --- |

| Title: | CE3-3.3: MPM list harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution presents the simulation results of CE3-3.3 tests on top of VTM-4.0 and CE3-3 codebase. In CE3-3.3, three tests on MPM list harmonization are carried out to find a better trade off between coding gain and encoding and decoding complexity. It is reported that test CE3-3.3.1, CE3-3.3.2, and CE3-3.3.3 obtains 0.02%, 0.04%, and 0.07% coding gain respectively for the AI configuration on top of VTM-4.0 with around 100% coding time.

Introduction

In VTM-4.0, three different MPM list generation processes are employed for zero reference line with ISP off, zero reference line with ISP on, and non-zero reference lines, respectively. For zero reference line with ISP off, Planar and DC modes are always included in the MPM list, and 4 angular MPM candidates are derived. For zero reference line with ISP on, Planar mode is always included from the MPM list, DC mode is always excluded from the MPM list, and 5 angular MPM candidates are derived. For non-zero reference lines, Planar and DC modes are always excluded from the MPM list, and 6 angular MPM candidates are derived.

Test description

For all the tests in this proposal, only one MPM list derivation process is employed and one superset of MPM list is generated, wherein Planar and DC modes are always included in this superset MPM list, all the MPM candidates in this superset MPM list are used for zero reference line with ISP off, MPM candidates except DC mode in this superset MPM list are used for zero reference line with ISP on, and only the angular MPM candidates in this superset MPM list are used for non-zero reference lines.

For CE3-3.3.1 ~ CE3-3.3.3, 4 angular MPM candidates are derived by using zero reference line MPM list derivation process, and the number of candidates in the superset MPM list is 6.

For CE3-3.3.1, the position of Planar and DC modes in the superset MPM list is not fixed, which is the same as that of the zero reference line MPM list with ISP off in VTM4.0 except when one neighbouring mode is Planar or DC mode, and the other neighbouring mode is angular mode, Planar mode is proposed to firstly insert into the MPM list.

For CE3-3.3.2 and CE3-3.3.3, Planar and DC modes are always the first two candidates in the superset MPM list. The first two bins of the MPM index is context coded and other bins of MPM index is bypass coded. The only difference between CE3-3.3.2 and CE3-3.3.3 is the number of the contexts used for the first bin of MPM index. To be specific, the number of contexts used for all the tests in this proposal is shown in Table 1.

Table 1: Number of contexts used for each test

| Test | Number of context for the 1st bin of MPM index | Number of context for the 2nd bin of MPM index | Other bins of MPM index |
| --- | --- | --- | --- |
| CE3-3.3.1 | 0 | 0 | 0 |
| CE3-3.3.2 | 1 | 1 | 0 |
| CE3-3.3.3 | 3 | 1 | 0 |

Experimental results

The tests are implemented on top of VTM4.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-M1010 [2].

Table 2: Test results of CE3-3.3.1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.04% | -0.09% | 97% | 101% | -0.04% | -0.04% | -0.03% | 98% | 99% |
| Class A2 | -0.02% | 0.05% | -0.05% | 102% | 103% | 0.01% | -0.14% | 0.08% | 100% | 100% |
| Class B | -0.02% | 0.04% | 0.05% | 98% | 98% | 0.00% | 0.02% | -0.18% | 100% | 100% |
| Class C | 0.10% | 0.02% | 0.13% | 99% | 99% | 0.07% | 0.10% | -0.17% | 99% | 99% |
| Class E | -0.03% | 0.13% | -0.01% | 99% | 100% |  |  |  |  |  |
| Overall | -0.02% | 0.04% | 0.02% | 99% | 100% | 0.01% | 0.00% | -0.09% | 99% | 100% |
| Class D | 0.01% | 0.02% | -0.18% | 99% | 100% | -0.01% | -0.04% | -0.37% | 99% | 99% |
| Class F | 0.21% | 0.11% | 0.08% | 98% | 96% | 0.09% | -0.01% | 0.05% | 97% | 93% |

Table 3: Test results of CE3-3.3.1 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.08% | 0.08% | 99% | 100% | -0.02% | 0.12% | -0.15% | 99% | 100% |
| Class A2 | 0.01% | -0.01% | 0.02% | 98% | 100% | 0.00% | -0.05% | -0.05% | 101% | 100% |
| Class B | 0.02% | 0.03% | 0.08% | 99% | 99% | 0.02% | 0.07% | 0.01% | 100% | 99% |
| Class C | 0.10% | 0.03% | 0.10% | 99% | 102% | 0.05% | -0.01% | -0.01% | 100% | 101% |
| Class E | 0.04% | 0.00% | -0.02% | 100% | 100% |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.06% | 99% | 100% | 0.02% | 0.03% | -0.04% | 100% | 100% |
| Class D | 0.02% | 0.11% | -0.02% | 99% | 101% | 0.06% | -0.23% | -0.46% | 101% | 100% |
| Class F | 0.20% | 0.15% | 0.02% | 100% | 99% | 0.07% | 0.19% | 0.17% | 101% | 99% |

Table 4: Test results of CE3-3.3.2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.22% | -0.22% | 96% | 99% | -0.11% | -0.07% | -0.16% | 100% | 101% |
| Class A2 | -0.02% | 0.10% | 0.04% | 100% | 102% | 0.01% | 0.08% | 0.07% | 101% | 101% |
| Class B | -0.03% | 0.04% | 0.04% | 101% | 101% | 0.00% | 0.09% | -0.07% | 100% | 100% |
| Class C | 0.09% | 0.04% | 0.13% | 101% | 99% | 0.04% | -0.06% | -0.19% | 99% | 99% |
| Class E | 0.03% | 0.28% | 0.15% | 98% | 99% |  |  |  |  |  |
| Overall | -0.04% | 0.05% | 0.03% | 99% | 100% | -0.01% | 0.02% | -0.09% | 100% | 100% |
| Class D | -0.01% | 0.08% | -0.02% | 99% | 97% | 0.05% | -0.33% | -0.11% | 99% | 99% |
| Class F | 0.25% | 0.30% | 0.20% | 101% | 101% | 0.10% | 0.07% | -0.07% | 100% | 101% |

Table 5: Test results of CE3-3.3.2 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.19% | -0.11% | 99% | 100% | -0.04% | 0.18% | -0.20% | 99% | 100% |
| Class A2 | 0.01% | 0.01% | 0.05% | 98% | 100% | 0.02% | 0.15% | 0.07% | 99% | 100% |
| Class B | 0.00% | 0.05% | 0.11% | 99% | 95% | 0.01% | -0.07% | 0.06% | 99% | 97% |
| Class C | 0.04% | 0.06% | 0.25% | 102% | 98% | 0.02% | 0.08% | -0.08% | 99% | 97% |
| Class E | 0.10% | 0.14% | 0.26% | 99% | 96% |  |  |  |  |  |
| Overall | -0.01% | 0.02% | 0.12% | 100% | 98% | 0.00% | 0.06% | -0.03% | 99% | 98% |
| Class D | -0.03% | 0.11% | -0.03% | 100% | 95% | 0.05% | -0.49% | -0.54% | 100% | 98% |
| Class F | 0.19% | 0.17% | 0.20% | 99% | 95% | 0.03% | 0.04% | 0.20% | 99% | 97% |

Table 6: Test results of CE3-3.3.3 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.37% | -0.31% | 95% | 97% | -0.09% | 0.01% | -0.08% | 100% | 101% |
| Class A2 | -0.04% | 0.05% | -0.03% | 101% | 100% | 0.02% | 0.04% | 0.07% | 101% | 101% |
| Class B | -0.06% | 0.00% | 0.05% | 102% | 104% | -0.02% | 0.00% | -0.01% | 100% | 101% |
| Class C | 0.08% | 0.08% | 0.03% | 100% | 99% | 0.03% | -0.04% | -0.09% | 99% | 99% |
| Class E | -0.02% | 0.11% | 0.05% | 98% | 99% |  |  |  |  |  |
| Overall | -0.07% | -0.02% | -0.03% | 99% | 100% | -0.01% | 0.00% | -0.03% | 100% | 100% |
| Class D | -0.03% | 0.09% | -0.22% | 99% | 99% | 0.03% | -0.06% | -0.10% | 99% | 98% |
| Class F | 0.20% | 0.19% | 0.09% | 100% | 100% | 0.06% | -0.06% | -0.10% | 100% | 100% |

Table 7: Test results of CE3-3.3.3 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.28% | -0.14% | 100% | 101% | -0.07% | 0.08% | -0.18% | 98% | 100% |
| Class A2 | 0.00% | -0.05% | 0.00% | 98% | 99% | 0.01% | 0.07% | 0.12% | 99% | 100% |
| Class B | -0.02% | 0.00% | 0.01% | 100% | 98% | -0.02% | 0.02% | -0.08% | 98% | 96% |
| Class C | 0.03% | 0.01% | 0.10% | 102% | 94% | 0.00% | -0.02% | 0.00% | 98% | 94% |
| Class E | 0.05% | 0.20% | 0.21% | 102% | 96% |  |  |  |  |  |
| Overall | -0.03% | -0.02% | 0.04% | 100% | 97% | -0.02% | 0.03% | -0.04% | 98% | 97% |
| Class D | -0.03% | 0.17% | 0.15% | 99% | 92% | 0.04% | -0.51% | -0.35% | 97% | 92% |
| Class F | 0.15% | 0.21% | 0.09% | 99% | 92% | -0.02% | 0.16% | 0.01% | 98% | 93% |

Conclusion

This contribution reports the simulation results of CE3-3.3 tests on MPM list harmonization. The proposed methods harmonize and simplify the MPM list generation process with no encoding and decoding time increase, and Test CE3-3.3.3 achieves 0.07% coding gain in AI configuration on top of VTM-4.0. It is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).