| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0393_r2 |
| --- | --- |

| Title: | CE3-3.3: MPM list harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution presents the simulation results of CE3-3.3 tests on top of VTM-4.0 and CE3-3 codebase. In CE3-3.3, three tests on MPM list harmonization are carried out to find a better trade off between coding gain and encoding and decoding complexity. It is reported that test CE3-3.3.1, CE3-3.3.2, and CE3-3.3.3 obtains 0.02%, 0.07%, and 0.11% coding gain respectively for the AI configuration on top of VTM-4.0 with around 100% coding time.

Introduction

In VTM-4.0, three different MPM list generation processes are employed for zero reference line with ISP off, zero reference line with ISP on, and non-zero reference lines, respectively. For zero reference line with ISP off, Planar and DC modes are always included in the MPM list, and 4 angular MPM candidates are derived. For zero reference line with ISP on, Planar mode is always included from the MPM list, DC mode is always excluded from the MPM list, and 5 angular MPM candidates are derived. For non-zero reference lines, Planar and DC modes are always excluded from the MPM list, and 6 angular MPM candidates are derived.

Test description

For all the tests in this proposal, only one MPM list derivation process is employed and one superset of MPM list is generated, wherein Planar and DC modes are always included in this superset MPM list, all the MPM candidates in this superset MPM list are used for zero reference line with ISP off, MPM candidates except DC mode in this superset MPM list are used for zero reference line with ISP on, and only the angular MPM candidates in this superset MPM list are used for non-zero reference lines.

For CE3-3.3.1 ~ CE3-3.3.3, 4 angular MPM candidates are derived by using zero reference line MPM list derivation process, and the number of candidates in the superset MPM list is 6.

For CE3-3.3.1, the position of Planar and DC modes in the superset MPM list is not fixed, which is the same as that of the zero reference line MPM list with ISP off in VTM4.0 except when one neighbouring mode is Planar or DC mode, and the other neighbouring mode is angular mode, Planar mode is proposed to firstly insert into the MPM list.

For CE3-3.3.2 and CE3-3.3.3, Planar and DC modes are always the first two candidates in the superset MPM list. The first two bins of the MPM index is context coded and other bins of MPM index is bypass coded. The only difference between CE3-3.3.2 and CE3-3.3.3 is the number of the contexts used for the first bin of MPM index. To be specific, the number of contexts used for all the tests in this proposal is shown in Table 1.

Table 1: Number of contexts used for each test

| Test | Number of context for the 1st bin of MPM index | Number of context for the 2nd bin of MPM index | Other bins of MPM index |
| --- | --- | --- | --- |
| CE3-3.3.1 | 0 | 0 | 0 |
| CE3-3.3.2 | 1 | 1 | 0 |
| CE3-3.3.3 | 3 | 1 | 0 |

Experimental results

The tests are implemented on top of VTM4.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-M1010 [2]. Table 2 ~ 7 illustrates the coding performance of CE3-3.3.1 ~ CE3-3.3.3 on top of VTM4.0 and CE3-3 code base.

One encoder bug related to CE3-3.3.2 and CE3-3.3.3 is found and the bugfix has been committed to CE3-3 git branch. The bug is caused by comparing an unsigned int with a negative int variable. Only line line change is needed, which is listed below:

#if PLANAR_DC_ALWAYS_FIRST_TWO_MPMS

int mpm_idx = numMPMs;

#else

unsigned mpm_idx = numMPMs;

#endif

After the bugfix, tests CE3-3.3.2 and CE3-3.3.3 have been re-run. Table 8 and Table 9 illustrates the coding performance of these two tests with bugfix on top of VTM4.0.

Table 2: Test results of CE3-3.3.1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.04% | -0.09% | 97% | 101% | -0.04% | -0.04% | -0.03% | 98% | 99% |
| Class A2 | -0.02% | 0.05% | -0.05% | 102% | 103% | 0.01% | -0.14% | 0.08% | 100% | 100% |
| Class B | -0.02% | 0.04% | 0.05% | 98% | 98% | 0.00% | 0.02% | -0.18% | 100% | 100% |
| Class C | 0.10% | 0.02% | 0.13% | 99% | 99% | 0.07% | 0.10% | -0.17% | 99% | 99% |
| Class E | -0.03% | 0.13% | -0.01% | 99% | 100% |  |  |  |  |  |
| Overall | -0.02% | 0.04% | 0.02% | 99% | 100% | 0.01% | 0.00% | -0.09% | 99% | 100% |
| Class D | 0.01% | 0.02% | -0.18% | 99% | 100% | -0.01% | -0.04% | -0.37% | 99% | 99% |
| Class F | 0.21% | 0.11% | 0.08% | 98% | 96% | 0.09% | -0.01% | 0.05% | 97% | 93% |

Table 3: Test results of CE3-3.3.1 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.08% | 0.08% | 99% | 100% | -0.02% | 0.12% | -0.15% | 99% | 100% |
| Class A2 | 0.01% | -0.01% | 0.02% | 98% | 100% | 0.00% | -0.05% | -0.05% | 101% | 100% |
| Class B | 0.02% | 0.03% | 0.08% | 99% | 99% | 0.02% | 0.07% | 0.01% | 100% | 99% |
| Class C | 0.10% | 0.03% | 0.10% | 99% | 102% | 0.05% | -0.01% | -0.01% | 100% | 101% |
| Class E | 0.04% | 0.00% | -0.02% | 100% | 100% |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.06% | 99% | 100% | 0.02% | 0.03% | -0.04% | 100% | 100% |
| Class D | 0.02% | 0.11% | -0.02% | 99% | 101% | 0.06% | -0.23% | -0.46% | 101% | 100% |
| Class F | 0.20% | 0.15% | 0.02% | 100% | 99% | 0.07% | 0.19% | 0.17% | 101% | 99% |

Table 4: Test results of CE3-3.3.2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.22% | -0.22% | 96% | 99% | -0.11% | -0.07% | -0.16% | 100% | 101% |
| Class A2 | -0.02% | 0.10% | 0.04% | 100% | 102% | 0.01% | 0.08% | 0.07% | 101% | 101% |
| Class B | -0.03% | 0.04% | 0.04% | 101% | 101% | 0.00% | 0.09% | -0.07% | 100% | 100% |
| Class C | 0.09% | 0.04% | 0.13% | 101% | 99% | 0.04% | -0.06% | -0.19% | 99% | 99% |
| Class E | 0.03% | 0.28% | 0.15% | 98% | 99% |  |  |  |  |  |
| Overall | -0.04% | 0.05% | 0.03% | 99% | 100% | -0.01% | 0.02% | -0.09% | 100% | 100% |
| Class D | -0.01% | 0.08% | -0.02% | 99% | 97% | 0.05% | -0.33% | -0.11% | 99% | 99% |
| Class F | 0.25% | 0.30% | 0.20% | 101% | 101% | 0.10% | 0.07% | -0.07% | 100% | 101% |

Table 5: Test results of CE3-3.3.2 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.19% | -0.11% | 99% | 100% | -0.04% | 0.18% | -0.20% | 99% | 100% |
| Class A2 | 0.01% | 0.01% | 0.05% | 98% | 100% | 0.02% | 0.15% | 0.07% | 99% | 100% |
| Class B | 0.00% | 0.05% | 0.11% | 99% | 95% | 0.01% | -0.07% | 0.06% | 99% | 97% |
| Class C | 0.04% | 0.06% | 0.25% | 102% | 98% | 0.02% | 0.08% | -0.08% | 99% | 97% |
| Class E | 0.10% | 0.14% | 0.26% | 99% | 96% |  |  |  |  |  |
| Overall | -0.01% | 0.02% | 0.12% | 100% | 98% | 0.00% | 0.06% | -0.03% | 99% | 98% |
| Class D | -0.03% | 0.11% | -0.03% | 100% | 95% | 0.05% | -0.49% | -0.54% | 100% | 98% |
| Class F | 0.19% | 0.17% | 0.20% | 99% | 95% | 0.03% | 0.04% | 0.20% | 99% | 97% |

Table 6: Test results of CE3-3.3.3 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.37% | -0.31% | 95% | 97% | -0.09% | 0.01% | -0.08% | 100% | 101% |
| Class A2 | -0.04% | 0.05% | -0.03% | 101% | 100% | 0.02% | 0.04% | 0.07% | 101% | 101% |
| Class B | -0.06% | 0.00% | 0.05% | 102% | 104% | -0.02% | 0.00% | -0.01% | 100% | 101% |
| Class C | 0.08% | 0.08% | 0.03% | 100% | 99% | 0.03% | -0.04% | -0.09% | 99% | 99% |
| Class E | -0.02% | 0.11% | 0.05% | 98% | 99% |  |  |  |  |  |
| Overall | -0.07% | -0.02% | -0.03% | 99% | 100% | -0.01% | 0.00% | -0.03% | 100% | 100% |
| Class D | -0.03% | 0.09% | -0.22% | 99% | 99% | 0.03% | -0.06% | -0.10% | 99% | 98% |
| Class F | 0.20% | 0.19% | 0.09% | 100% | 100% | 0.06% | -0.06% | -0.10% | 100% | 100% |

Table 7: Test results of CE3-3.3.3 over codebase

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.28% | -0.14% | 100% | 101% | -0.07% | 0.08% | -0.18% | 98% | 100% |
| Class A2 | 0.00% | -0.05% | 0.00% | 98% | 99% | 0.01% | 0.07% | 0.12% | 99% | 100% |
| Class B | -0.02% | 0.00% | 0.01% | 100% | 98% | -0.02% | 0.02% | -0.08% | 98% | 96% |
| Class C | 0.03% | 0.01% | 0.10% | 102% | 94% | 0.00% | -0.02% | 0.00% | 98% | 94% |
| Class E | 0.05% | 0.20% | 0.21% | 102% | 96% |  |  |  |  |  |
| Overall | -0.03% | -0.02% | 0.04% | 100% | 97% | -0.02% | 0.03% | -0.04% | 98% | 97% |
| Class D | -0.03% | 0.17% | 0.15% | 99% | 92% | 0.04% | -0.51% | -0.35% | 97% | 92% |
| Class F | 0.15% | 0.21% | 0.09% | 99% | 92% | -0.02% | 0.16% | 0.01% | 98% | 93% |

Table 8: Test results of CE3-3.3.2 with bugfix over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.26% | -0.20% | 96% | 101% | -0.11% | -0.06% | -0.21% | 99% | 100% |
| Class A2 | -0.05% | 0.04% | -0.04% | 100% | 102% |  |  |  |  |  |
| Class B | -0.07% | -0.03% | 0.01% | 100% | 102% | -0.01% | 0.01% | -0.08% | 99% | 98% |
| Class C | 0.09% | 0.02% | 0.10% | 100% | 99% | 0.03% | -0.19% | 0.02% | 100% | 101% |
| Class E | -0.01% | 0.20% | 0.11% | 100% | 99% |  |  |  |  |  |
| Overall | -0.07% | -0.01% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class D | -0.02% | -0.03% | -0.10% | 102% | 100% | 0.00% | -0.02% | -0.32% | 99% | 100% |
| Class F | 0.21% | 0.19% | 0.13% | 99% | 101% | 0.07% | -0.02% | -0.05% | 99% | 99% |

Table 9: Test results of CE3-3.3.3 with bugfix over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.44% | -0.33% | 96% | 100% | -0.10% | -0.08% | -0.12% | 99% | 100% |
| Class A2 | -0.07% | 0.01% | -0.11% | 100% | 102% | -0.01% | -0.02% | 0.14% | 100% | 100% |
| Class B | -0.10% | -0.05% | -0.04% | 100% | 102% | -0.03% | -0.10% | -0.07% | 99% | 98% |
| Class C | 0.04% | -0.01% | 0.08% | 101% | 98% | 0.03% | 0.02% | 0.06% | 99% | 100% |
| Class E | -0.09% | 0.04% | 0.09% | 100% | 99% |  |  |  |  |  |
| Overall | -0.11% | -0.08% | -0.05% | 100% | 100% | -0.02% | -0.05% | 0.00% | 99% | 100% |
| Class D | -0.04% | -0.05% | -0.10% | 100% | 99% | 0.00% | -0.30% | -0.34% | 100% | 101% |
| Class F | 0.17% | 0.17% | 0.16% | 100% | 102% | 0.07% | -0.01% | 0.07% | 99% | 99% |

Spec text changes

The following three sub-sections describes spec text modification of CE3-3.3.1, CE3-3.3.2, and CE3-3.3.3. It is based on VVC draft5 v6 [3]. The added texts are highlighted with in green, and the deleted texts are marked with strikethrough.

CE3-3.3.1 spec text changes

The following describes spec text modification of CE3-3.3.1.

8.4.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The variables ispDefaultMode1 and ispDefaultMode2 are defined as follows:

- If IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, ispDefaultMode1 is set equal to INTRA_ANGULAR18 and ispDefaultMode2 is set equal to INTRA_ANGULAR5.

- Otherwise, ispDefaultMode1 is set equal to INTRA_ANGULAR50 and ispDefaultMode2 is set equal to INTRA_ANGULAR63.

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-9)

candModeList[ 1 ] = INTRA_PLANAR	(8-10)

candModeList[ 2 ] = INTRA_DC		(8-11)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-12)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-13)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-14)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-15)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-16)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-17)

  - If one of the following conditions is true,

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and candIntraPredModeA  is less than INTRA_ANGULAR34,

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and candIntraPredModeA  is greater than or equal to INTRA_ANGULAR34,

  - IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0,

the following applies:

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-18)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-19)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-20)

  - Otherwise, the following applies:

candModeList[ 3 ] = ispDefaultMode1	(8-21)

candModeList[ 4 ] = ispDefaultMode2	(8-22)

candModeList[ 5 ] = INTRA_PLANAR	(8-23)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-24)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-25)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-26)

candModeList[ 1 ] = candIntraPredModeB	(8-27)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-28)

candModeList[ 3 ] = INTRA_DC		(8-29)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-30)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-31)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-33)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

  - When IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, and abs( candIntraPredModeB − ispDefaultMode1 ) is less than abs( candIntraPredModeA − ispDefaultMode1 ), the following applies:

candModeList[ 0 ] = candIntraPredModeB	(8-34)

candModeList[ 1 ] = candIntraPredModeA	(8-35)

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-36)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-37)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-38)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-39)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-40)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-41)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-42)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-43)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-44)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-46)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-48)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-49)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-50)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-51)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeAINTRA_PLANAR	(8-52)

candModeList[ 1 ] = candIntraPredModeBmaxAB	(8-53)

candModeList[ 2 ] = 1 – minABINTRA_DC	(8-54)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-55)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-56)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-57)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = maxAB		(8-58)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-59)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-60)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-61)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-62)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-63)

  - Otherwise (IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-64)

candModeList[ 1 ] = maxAB		(8-65)

candModeList[ 2 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-66)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-67)

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-68)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-69)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-70)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-71) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-72)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-73)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-74)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-75)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-76)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-77)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-78)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-79)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-80)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-81)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-82)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-83)

candModeList[ 2 ] = INTRA_ANGULAR25	(8-84)

candModeList[ 3 ] = INTRA_ANGULAR10	(8-85)

candModeList[ 4 ] = INTRA_ANGULAR65	(8-86)

candModeList[ 5 ] = INTRA_ANGULAR50	(8-87)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-88)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-89)

candModeList[ 2 ] = INTRA_ANGULAR43	(8-90)

candModeList[ 3 ] = INTRA_ANGULAR60	(8-91)

candModeList[ 4 ] = INTRA_ANGULAR3	(8-92)

candModeList[ 5 ] = INTRA_ANGULAR18	(8-93)

- If IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, when candModeList[ i ] is greater than INTRA_DC, for i = 0..5 and j = 0, the following applies:

candModeList[ j ++] = candModeList[i] 	(8-94)

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, when candModeList[ i ] is not equal to INTRA_DC, for i = 0..5 and j = 0, the following applies:

candModeList[ j ++] = candModeList[i] 	(8-95)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

CE3-3.3.2 spec text changes

The following describes spec text modification of CE3-3.3.2.

8.4.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The variables ispDefaultMode1 and ispDefaultMode2 are defined as follows:

- If IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, ispDefaultMode1 is set equal to INTRA_ANGULAR18 and ispDefaultMode2 is set equal to INTRA_ANGULAR5.

- Otherwise, ispDefaultMode1 is set equal to INTRA_ANGULAR50 and ispDefaultMode2 is set equal to INTRA_ANGULAR63.

- The candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-9)

candModeList[ 1 ] = INTRA_DC		(8-10)

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 02 ] = candIntraPredModeA	(8-911)

candModeList[ 1 ] = INTRA_PLANAR	(8-10)

candModeList[ 2 ] = INTRA_DC		(8-11)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-12)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-13)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-14)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-15)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-16)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-17)

  - If one of the following conditions is true,

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and candIntraPredModeA  is less than INTRA_ANGULAR34,

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and candIntraPredModeA  is greater than or equal to INTRA_ANGULAR34,

  - IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0,

the following applies:

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-18)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-19)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-20)

  - Otherwise, the following applies:

candModeList[ 3 ] = ispDefaultMode1	(8-21)

candModeList[ 4 ] = ispDefaultMode2	(8-22)

candModeList[ 5 ] = INTRA_PLANAR	(8-23)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-24)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-25)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 02 ] = candIntraPredModeA	(8-26)

candModeList[ 13 ] = candIntraPredModeB	(8-27)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-28)

candModeList[ 3 ] = INTRA_DC		(8-29)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-30)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-31)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-33)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

  - When IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, and abs( candIntraPredModeB − ispDefaultMode1 ) is less than abs( candIntraPredModeA − ispDefaultMode1 ), the following applies:

candModeList[ 0 ] = candIntraPredModeB	(8-34)

candModeList[ 1 ] = candIntraPredModeA	(8-35)

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-36)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-37)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-38)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-39)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-40)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-41)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-42)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-43)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-44)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-46)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-48)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-49)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-50)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-51)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-52)

candModeList[ 1 ] = candIntraPredModeB	(8-53)

candModeList[ 2 ] = 1 − minABmaxAB	(8-54)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-55)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-56)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-57)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = maxAB		(8-58)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-59)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-60)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-61)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-62)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-63)

  - Otherwise (IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-64)

candModeList[ 1 ] = maxAB		(8-65)

candModeList[ 2 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-66)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-67)

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-68)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-69)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-70)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-71) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-72)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-73)

candModeList[ 4 ] = INTRA_ANGULAR462	(8-74)

candModeList[ 5 ] = INTRA_ANGULAR5434	(8-75)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-76)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-77)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-78)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-79)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-80)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-81)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-82)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-83)

candModeList[ 2 ] = INTRA_ANGULAR25	(8-84)

candModeList[ 3 ] = INTRA_ANGULAR10	(8-85)

candModeList[ 4 ] = INTRA_ANGULAR65	(8-86)

candModeList[ 5 ] = INTRA_ANGULAR50	(8-87)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-88)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-89)

candModeList[ 2 ] = INTRA_ANGULAR43	(8-90)

candModeList[ 3 ] = INTRA_ANGULAR60	(8-91)

candModeList[ 4 ] = INTRA_ANGULAR3	(8-92)

candModeList[ 5 ] = INTRA_ANGULAR18	(8-93)

- If IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, for i = 0..3, the following applies:

candModeList[ i] = candModeList[i+2] 	(8-94)

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, for i = 1..4, the following applies:

candModeList[ i] = candModeList[i + 1] 	(8-95)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| pred_mode_flag | 0,1 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_ibc_flag | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | na | na | na | na |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | na |
| intra_subpartition_split_flag | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | Bypass0 | Bypass1 | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |

CE3-3.3.3 spec text changes

Spec text changes of CE3-3.3.3 are the same with CE3-3.3.2 as provided in Section 4.2 except the following changes.

| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| pred_mode_flag | 0,1 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_ibc_flag | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | na | na | na | na |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | na |
| intra_subpartition_split_flag | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | Bypass0,1,2 (clause a.b.c.d.e) | Bypass3 | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |

a.b.c.d.e Derivation process of ctxInc using left-bottom and above-right syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, and the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 − 1, y0 + cbHeight − 1 ) as inputs, and the output is assigned to availableLB.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to availableAR.

The same CTU checking process for a block as specified in clause 6.Y.X [Same CTU checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to sameCtuAR.

The assignment of ctxInc is specified as follows with condL, condLB, condA and condAR for the syntax elements intra_luma_angular_mode_flag[ x0 ][ y0 ] specified in Table 9-xx:

ctxInc = ( condLB  &&  availableLB ) + ( condAR  &&  availableAR && sameCtuAR )	(9-yy)

Table 9-xx – Specification of ctxInc using left-bottom and above-right syntax elements

| condLB | condAR |
| --- | --- |
| intra_luma_mpm_flag [ xNbLB ][ yNbLB ] is true && intra_luma_mpm_idx [ xNbLB ][ yNbLB ] is equal to 0 | intra_luma_mpm_flag [ xNbAR ][ yNbAR ] is equal to true && intra_luma_mpm_idx [ xNbAR ][ yNbAR ] is equal to 0 |

Conclusion

This contribution reports the simulation results of CE3-3.3 tests on MPM list harmonization. The proposed methods harmonize and simplify the MPM list generation process with no encoding and decoding time increase, and Test CE3-3.3.3 achieves 0.11% coding gain in AI configuration on top of VTM-4.0. It is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1001, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).